| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0098-v1 |
| --- | --- |

| Title: | CE3: Combined test of CE3.2.1 and CE3.2.2 (Test 3.2.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Vadim Seregin Marta Karczewicz Hong-Jheng Jhu Yao-Jen Chang | Email: | aramasub@qti.qualcomm.com geertv@qti.qualcomm.com HongJheng@fginnov.com |
| Source: | Qualcomm Inc., Foxconn |  |  |

_____________________________

# Abstract

This contribution provides the simulation results of sub-test CE3.2.2.1 on multi-model LM (MMLM), a combined test of CE3.2.1 and CE3.2.2. In this test, a combination of the signalling and the number of lines from sub-tests CE3.2.1 and CE3.2.2 is studied. It is reported that when compared to VTM-3.0, the proposed combination has a BD-rate performance of (−0.27%, −1.71%, −2.26%) for (Y, Cb, Cr) with 99% encoding time and 99% decoding time for the AI configuration, and BD-rate performance of (−0.13%, −1.30%, −1.73%) for (Y, Cb, Cr) with 99% encoding time and 98% decoding time for the RA configuration.

# Introduction

This contribution reports the coding efficiency of the combined test of CE3.2.1 and CE3.2.2 in Core Experiment (CE) 3 [1]. In this contribution, a combination of the signalling and number of lines from the two tests is studied. Three MMLM modes are used: two-model equivalents corresponding to each of CCLM-TL, CCLM-L and CCLM-T modes. For signalling, the design of CE3.2.2 is used to indicate MMLM mode with one flag to indicate whether an MMLM mode is signalled, and when an MMLM mode is indicated, an index is signalled to indicate the particular MMLM mode. The number of lines used for model derivation is the same as in CE3.2.1, i.e., 2 lines above and 3 lines left of the current block for luma, and 1 line each above and left of the current block for chroma. The number of lines is the same as CCLM/MDLM in VTM 3.0, and when the top boundary of the current block shares the CTU boundary, only one luma line is used from above the block. A tabular description of the combined test vis-à-vis CE3.2.1 and CE3.2.2 is given below:

|  | CE3.2.1 | CE3.2.2 | CE3.2.2.1 |
| --- | --- | --- | --- |
| Signalling | Adds MMLM modes in LM symbol list | Uses flag and index to indicate MMLM mode | Same as CE3.2.2 |
| Number of lines used for model derivation | Same as CCLM/MDLM (i.e., Y: 2 lines above, 3 lines left, C: 1 line above and left) | (Y: 4 lines above, 5 lines left, C: 2 lines above and left) | Same as CE3.2.1 |

# Simulation results

The following experiments were conducted using the VTM-3.0 software with the VTM configuration and the JVET common test conditions [2] to collect the BD-rate, encoding time and decoding time of the proposed techniques in the All Intra and Random Access configurations. The proposed method has a BD-rate performance of (−0.27%, −1.71%, −2.26%) for (Y, Cb, Cr) with 99% encoding time and 99% decoding time for the AI configuration, and BD-rate performance of (−0.13%, −1.30%, −1.73%) for (Y, Cb, Cr) with 99% encoding time and 98% decoding time for the RA configuration.

Table 2: Simulation results of the proposed combined test CE3.2.2.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -2.81% | -4.75% | 97% | 98% |
| Class A2 | -0.10% | -1.03% | -0.94% | 102% | 103% |
| Class B | -0.20% | -2.05% | -2.54% | 98% | 100% |
| Class C | -0.28% | -1.85% | -2.40% | 99% | 100% |
| Class E | -0.02% | -0.55% | -0.47% | 98% | 96% |
| Overall | -0.27% | -1.71% | -2.26% | 99% | 99% |
| Class D | -0.09% | -1.25% | -1.30% | 99% | 100% |
| Class F | -0.38% | -1.86% | -2.46% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -2.02% | -3.47% | 98% | 99% |
| Class A2 | -0.02% | -0.69% | -0.36% | 101% | 101% |
| Class B | -0.06% | -1.45% | -1.96% | 98% | 97% |
| Class C | -0.05% | -1.05% | -1.16% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -1.30% | -1.73% | 99% | 98% |
| Class D | -0.01% | -1.01% | -0.97% | 100% | 100% |
| Class F | -0.15% | -1.01% | -1.41% | 99% | 99% |

# Conclusions

The results show that the combined test provides significant improvement to coding efficiency in terms of luma and chroma BD-rate with negligible increase in EncT/DecT. It is suggested to adopt the proposed tool as part of VVC.

# Reference

- G. Van der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-L1023, the 12th JVET meeting, Oct. 2018, Macao, CN.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Acknowledgements

We would like to thank Samsung for cross-checking this CE test.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).