| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0101 |
| --- | --- |

| Title: | AHG17: On VVC HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin, Karsten Sühring, Yago Sanchez (HHI) Miska M. Hannuksela, Kashyap Kammachi-Sreedhar (Nokia) Ye-Kui Wang, Hendry (Huawei) Stephan Wenger, Byeongdoo Choi (Tencent) Sachin Deshpande (Sharp) Rickard Sjöberg (Ericsson) | Tel: Email: |  |
| Source: | Fraunhofer HHI, Nokia, Huawei Technologies Co., Ltd., Tencent, Sharp Labs of America, Inc, Ericsson. |  |  |

_____________________________

# Abstract

This contribution discusses which HEVC high-level syntax aspects would be justified in VVC and proposes related specification text. The following topics are covered:

- Extension syntax for parameter sets

- Profile-level syntax structure

- Parameter set for bitstream properties

- Sub-bitstream extraction process

- VCL NAL unit types

- Non-VCL NAL unit types

- General decoding process

- Order of NAL units and AU boundary determination

- For discussion: middle-box-friendly extensions beyond the NAL unit header

Revision 1 of this document adds Ericsson as a co-signer.

# Extension syntax for parameter sets

Like in HEVC it is proposed to include extension syntax at the end of all parameter set syntax structures. In HEVC this design choice turned out to be useful for adding extension-specific syntax elements in later versions of the standard. An example is given for SPS:

| … |  |
| --- | --- |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Profile-level syntax structure

## Proposal

| profile_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_idc | u(8) |
| general_constraint_info( ) |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| if( maxNumSubLayersMinus1 > 0 ) |  |
| for( i = maxNumSubLayersMinus1; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_idc[ i ] | u(8) |
| sub_layer_constraint_info( i ) |  |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

## Motivation for the proposal

The HEVC design is proposed with the following deviations:

- profilePresentFlag input argument is not proposed in this contribution. It was introduced in HEVC for multi-layer extensions when the syntax structure is included in the VPS extension. Multi-layer hooks for VVC is suggested to be a subject of other contribution(s) and another discussion in JVET.

- profile_space is not proposed at this point of time.

Motivation: To our knowledge only the "default" profile_space has been used in HEVC.

For discussion: Some of the proponents raised a question whether profile_space would be justified as a hook for future extensions. Other proponents thought that extensions are possible by taking new profile_idc values into use.

- The tier concept and syntax element are not proposed at this point of time.

Motivation: While it is understood that a high tier could be important for some devices and use cases, such as high-end cameras and contribution coding, we felt that we are not the best experts to determine the need and particularly the parameters for the potential high tier. Moreover, the design of the HRD should perhaps be started before discussing tiers.

- As an editorial action, the constraint flags are encapsulated into their own syntax structures, general_constraint_info( ) and sub_layer_constraint_info( i ).

Motivation: Improved readability for profile_level( ).

- The following design decisions on individual constraint flags are proposed:

  - The constraint flags for progressive source and interlaced source are proposed with semantics inherited from HEVC, except that picture-level indication with picture timing SEI message is not proposed yet.

  - The constraint flag for non-packed content is proposed to have semantics that are similar in spirit to those of HEVC but written in a more general language to allow related SEI messages to be specified in the future. For clarity the proposed semantics are copied below:

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying. [Ed. Check whether there is a better name for this flag per the updated semantics.]

NOTE 2 – Examples for content that requires additional processing before displaying are bitstreams containing one or more frame packing arrangement SEI messages, equirectangular projection SEI messages, or cubemap projection SEI messages. [Ed. This NOTE should be updated at a later phase of VVC standardization based on which SEI messages are adopted into the VVC design.]

For discussion: Some proponents asked if individual constraint flags for different packing purposes (e.g. frame packing, projected omnidirectional video, fisheye omnidirectional video) would be reasonable.

  - The frame-only content flag is proposed with semantics inherited from HEVC.

  - Independent non-base layer decoding flag (general_inbld_flag) should be discussed after introducing layer identifier into the NAL unit header, if that is to be introduced, and is therefore not proposed at this point of time.

  - general_one_picture_only_constraint_flag and general_lower_bit_rate_constraint_flag relate to HEVC Still Picture and High Throughput profiles, respectively, and are therefore not proposed at this time.

  - Other HEVC constraints seem to have been covered by JVET-L0696. The additional syntax proposed in JVET-L0696 could be located in general_constraint_info( ) and sub_layer_constraint_info( i ), but since the motivation of this contribution is to propose design choices originating from HEVC, we leave proposals on other constraints to other contributions.

- The profile compatibility flags are not proposed.

Motivation: general_constraint_info( ) and sub_layer_constraint_info( i ) are assumed to provide sufficiently fine-grained information on the applied constraints.

## For discussion: syntax optimization

When maxNumSubLayersMinus1 is equal to 4, the following piece in the profile_level( ) syntax structure causes insertion of one byte of reserved bits redundantly:

| if( maxNumSubLayersMinus1 > 0 ) |  |
| --- | --- |
| for( i = maxNumSubLayersMinus1; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |

The redundant one reserved byte could be avoided with the following syntax:

| if( maxNumSubLayersMinus1 != 0 && maxNumSubLayersMinus1 != 4 ) |  |
| --- | --- |
| for( i = maxNumSubLayersMinus1 % 4; i < 4; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |

# Parameter set for bitstream properties

## Background

In HEVC:

- For the base layer, a sequence parameter set (SPS) applies to a coded video sequence (CVS).

- Bitstream properties are not indicated.

In VVC draft 3:

- An SPS applies to a CVS.

- Bitstream properties (constraints) are indicated at the start of an SPS.

## Proposal

A single parameter set is proposed for bitstream-level property indications. It is suggested to use the name Decoding Parameter Set (DPS).

At this point of time, the syntax structure is proposed to contain the maximum number of sub-layers (as in HEVC) and a profile_level( ) syntax structure, which indicates a profile and a level of a decoder that is capable of decoding the bitstream and includes the constraints that apply to the bitstream.

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_reserved_three_2bits | u(2) |
| dps_max_sub_layers_minus1 | u(3) |
| dps_reserved_zero_7bits | u(7) |
| profile_level( dps_max_sub_layers_minus1 ) |  |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Motivation for the proposal

It is asserted that the HEVC design of not indicating bitstream properties anywhere has implied constraints in systems. Specifically, a system has to:

- use a single SPS for the entire bitstream (and hence effectively complicating splicing of CVSs from different encoders), or

- announce all SPSs at the session start-up (and hence reducing encoders flexibility in run-time adjustments).

The proposal is asserted to provide flexibility for splicing: The bitstream constraints indicated in the DPS can be selected to be sufficiently loose to allow flexibility for potential splicing of CVSs from different encoders. For example, the bitstream constraints may indicate a level that is known to be higher than or equal to any levels used in CVSs that could potentially be spliced into this bitstream. SPSs can contain a tighter set of constraints that apply to individual CVSs.

The first two bytes of the proposed syntax are similar to those that HEVC initially had for VPS when multi-layer support was not yet introduced. dps_max_sub_layer_minus1 appears in the same bit position as nuh_temporal_id_plus1 in the NAL unit header.

For discussion: It is proposed to be discussed whether decoding parameter set should rather be a "bitstream header" NAL unit type, in which case the syntax would not include the parameter set ID and the standard would not have the activation process of the decoding parameter set.

# Sub-bitstream extraction process

An HEVC-like sub-bitstream extraction process is proposed. The nuh_layer_id part and related aspects are removed for now, since the signalling of layer is left for further study.

Motivation:

- It is needed for the semantics of sub-layer specific parts of the profile_level( ) syntax structure.

- It is invoked in the proposed HEVC-like general decoding process (see section 7).

# VCL NAL unit types

## Background: VCL NAL unit types in HEVC

HEVC includes the following VCL NAL unit types:

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2 3 | TSA_N TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4 5 | STSA_N STSA_R | Coded slice segment of an STSA picture slice_segment_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RADL_R | Coded slice segment of a RADL picture slice_segment_layer_rbsp( ) | VCL |
| 8 9 | RASL_N RASL_R | Coded slice segment of a RASL picture slice_segment_layer_rbsp( ) | VCL |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP SLNR VCL NAL unit types | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19 20 | IDR_W_RADL IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types | VCL |

## Proposed VCL NAL unit types in VVC

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL | Coded tile group of a non-STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2..7 | RSV_VCL_2.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14 RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |

## Motivation for the proposal

### IRAP NAL unit types

BLA pictures were thought to be unnecessary, since the same functionality can be achieved by including an EOS NAL unit before a CRA picture.

IDR_N_LP and IDR_W_RADL are kept matching to Stream Access Point (SAP) types 1 and 2, respectively, which are used extensively in MPEG-DASH and other systems. It is asserted that ISOBMFF file encapsulators may get only a bitstream as input and need to generate the sync and SAP sample signaling for the encapsulated bitstream. This process should preferably not require decoding beyond the NAL unit header.

### Other VCL NAL unit types

Sub-layer non-reference pictures are not indicated with the nal_unit_type values. We think that too many NAL unit types are consumed in HEVC by this indication which does not need to be present at the NAL unit header. The authors identified no strong use-case justifying the signaling overhead where a network element would drop those NAL units independently instead of a whole temporal sub-layer for a while.

Only STSA nal_unit_type value is included in this proposal. A TSA picture indication can in many cases be expressed by consecutive STSA pictures. Although the STSA type cannot perfectly replace the TSA type, no strong use case for specifically requiring a TSA picture instead of an STSA picture was identified.

### For discussion: leading picture NAL unit types

#### Background: HEVC

The HEVC decoding process allows a bitstream starting with a CRA picture. Moreover, the HEVC decoding process treats a CRA picture as a BLA picture when HandleCraAsBlaFlag is set equal to1 by external means or when a CRA picture is preceded by an EOS NAL unit. In all the mentioned cases, the CRA picture may be followed by any leading pictures (both RADL and RASL), and the RASL pictures associated with the CRA picture are not output by the decoder, as specified in clause 8.1.3 of HEVC:

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

#### Background: HEVC file format encapsulation (ISO/IEC 14496-15)

A CRA picture is mapped to SAP type 1, 2, or 3 depending on whether and which type of leading pictures follow it in decoding order. Sync samples are of SAP type 1 or 2 according the ISO base media file format; thus, the leading pictures following a CRA picture have an impact on whether a CRA picture can be marked as a sync sample. See more details from the excerpt of Table 10 of ISO/IEC 14496-15 below:

| NAL Unit Type | ISOBMFF sync sample status | DASH SAP type |
| --- | --- | --- |
| CRA | false (*1) true true | 3 (if the IRAP has associated RASL pictures) 2 (if the IRAP has no associated RASL pictures but has associated RADL pictures) 1 (if the IRAP has no associated leading pictures) |

#### Background: ISO base media file format (ISO/IEC 14496-12)

The ISO base media file format (ISOBMFF) specifies the SampleDependencyTypeBox as well as sample flags used in movie fragments, which enable indicating the decodable (RADL) and non-decodable (RASL) leading pictures with the is_leading syntax element, whose semantics are the following:

is_leading takes one of the following four values:

0:	the leading nature of this sample is unknown;

1:	this sample is a leading sample that has a dependency before the referenced I-picture (and is therefore not decodable);

2:	this sample is not a leading sample;

3:	this sample is a leading sample that has no dependency before the referenced I-picture (and is therefore decodable);

#### Discussion

The following options have been considered for NAL unit types of leading pictures:

- Two NAL unit types, RADL and RASL, as in HEVC.

- One leading picture NAL unit type

  - With an additional flag in the tile group header indicating whether the picture is RADL or RASL.

The flag should be specified in such a fixed position in the tile group header that is not preceded by start code emulation prevention bytes under any circumstances.

  - Without explicit syntax element differentiating between RASL and RADL.

A RASL picture is a leading picture that uses a reference picture preceding the associated CRA picture in decoding order in inter prediction.

  - Without explicit syntax element differentiating between RASL and RADL. All leading pictures of a CRA picture are treated like RASL pictures in the decoding process.

- A single NAL unit type is used for TRAIL and both types of leading pictures.

A leading picture can be detected by decoding POC values. A leading picture has a POC value less than the POC value of the associated CRA picture.

A RASL picture is a leading picture that uses a reference picture preceding the associated CRA picture in decoding order in inter prediction.

Identification of a leading picture and its type have been identified to be necessary or useful in the following cases:

- A bitstream starts with a CRA picture.

- A CRA picture is random-accessed, i.e. decoding starts from a CRA picture.

- A sequence of pictures starting with CRA picture is spliced after another sequence of coded pictures, and an EOS NAL unit is inserted in between these two sequences.

- A file writer gets a bitstream including CRA pictures as input and authors an ISOBMFF file with sync sample indications, SAP sample grouping, and/or is_leading indications.

Cases A to C above can be handled differently in the specification text and in practical decoder implementations: The specification text can be written similarly to HEVC: RASL pictures are decoded (for checking HRD compliancy) but not output. Practical decoder implementations can detect RASL pictures and omit their decoding. Decoder implementations could alternatively detect all leading pictures and omit their decoding; however, this could lead to an observable discontinuity in picture rate in the splicing use case.

The following table attempts to summarize the impacts of the options on the use cases:

| Use case | 1. RADL and RASL NAL unit types | 2a. LP NAL unit type, RADL/RASL flag | 2b. LP NAL unit type without RADL/RASL flag | 2c. LP NAL unit, RASL/RADL depending on IRAP | 3. Joint TRAIL/LP NAL unit type |
| --- | --- | --- | --- | --- | --- |
| A to C decoding specification text | RASL pictures are detected from the NAL unit type within clause 8.1. | RASL pictures are detected from the flag within clause 8.1. | RASL pictures are detected from references used for inter prediction within clause 8.3. | Leading pictures are detected from the NAL unit type. All leading pictures of CRA pictures are treated as RASL pictures within clause 8.1. | Leading pictures are detected from POC. RASL pictures detected from references used for inter prediction within clause 8.3. |
| A to C practical decoder implementation | RASL pictures are detected from the NAL unit type and their decoding is omitted. | RASL pictures are detected from the flag and their decoding is omitted. | Each picture is decoded. If a non-existing reference picture is referenced, the picture is not output. | Leading pictures are detected from the NAL unit type. Leading pictures of a CRA picture are treated as RASL and hence their decoding is omitted. | Leading pictures may be detected from POC. In cases A and B, decoding of leading pictures can be omitted. Otherwise, each picture is decoded. If a non-existing reference picture is referenced, the picture is not output. |
| D file writing | RASL pictures are detected from the NAL unit type. Sync sample and SAP status of CRA pictures is determined based on the following RASL pictures, if any. is_leading is set according to RADL and RASL types. | RASL pictures are detected from the flag. Sync sample and SAP status of CRA pictures is determined based on the following RASL pictures, if any. is_leading is set according to RADL and RASL types. | It is assumed that the file writer will not decode the bitstream. CRA pictures are marked as SAP type 3 always and are not marked as sync samples. is_leading for LP pictures is set to unknown leading picture type. | CRA pictures are marked as SAP type 3 always and are not marked as sync samples. is_leading for LP pictures of CRA pictures is set to non-decodable leading picture type. | The file writer decodes the POC values from the bitstream to identify leading pictures. CRA pictures are marked as SAP type 3 always and are not marked as sync samples. is_leading for LP pictures is set to unknown leading picture type. |

# Non-VCL NAL unit types

## Background: Non-VCL NAL unit types in HEVC

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

## Proposed non-VCL NAL unit types in VVC

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24..27 | RSV_NVCL24.. RSV_NVCL27 | Reserved | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

## Motivation for the proposal

VPS_NUT is replaced with DPS_NUT. SPS_NUT and PPS_NUT are kept. See section 3 for discussion on parameter sets.

FD_NUT is removed for now as it has implications on HRD and it is left for further study.

AUD_NUT is kept since it has been used in external specification, such as DVB. Therefore, section 8 proposes adding Order of NAL units and AU boundary determination in a similar manner to HEVC.

PREFIX_SEI_NUT and SUFFIX_SEI_NUT are kept as in HEVC.

RSV_NVCLX and UNSPECX are kept as well but with fewer values.

# General decoding process

The accompanying specification text document proposes text for clause 8.1. The respective HEVC text is reused with the following shortcuts thanks to the design choices above:

- Since the concept of CVSG is proposed not to be inherited from HEVC, a CVS decoding process is specified rather than a CVSG decoding process as in HEVC.

- HRD-specific text is not proposed at this point of time but should be considered as soon as HRD is included in VVC.

- Text related to BLA pictures is not proposed.

# Order of NAL units and AU boundary determination

As AUD_NUT has been used by external specifications (such as DVB) and seems to be useful, it is proposed to add HEVC-like sections 7.4.2.3 to 7.4.2.4.5 on NAL unit order and AU boundary detection. The proposed text removes the text related to nuh_layer_id in HEVC.

# For discussion: middle-box-friendly extensions beyond the NAL unit header

For discussion: If a need for any other additional information arises, it should be considered whether exposing such additional information to middle-boxes, file writers, or other such entities in an easily accessible manner is essential for any use cases. If so and if the NAL unit header is not considered to provide sufficient extension space for such additional information (or a part thereof), one approach could be to include such additional information into the tile group header such that:

- The information is fixed-length coded.

- The information is in such an early, fixed bit position within the tile group header that is not preceded by any entropy-coded syntax element and that is not preceded by start code emulation prevention bytes under any circumstances.

- Correct decoding of the information does not require activation or decoding of any parameter sets.

# Patent rights declaration(s)

Fraunhofer HHI, Nokia, Huawei Technologies Co., Ltd., Tencent, Sharp Labs of America, Inc., and Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).