| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0102-v4 |
| --- | --- |

| Title: | CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Valeri George, Jackie Ma, Tung Nguyen Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the Intra Sub-Partitions (ISP) coding mode on top of VTM-3.0. Various modifications have been carried out on the algorithm to find a better trade-off between the gain and the encoder run-time with respect to the results presented in the Macao Meeting. Test 1.1.1 obtains a gain of 0,59% with encoding and decoding times of 112% and 103% respectively for the AI configuration and a gain of 0,29% with encoding and decoding times of 102% and 103% respectively for the RA configuration. Regarding Test 1.1.2, the results are a gain of 0,46% with encoding and decoding times of 112% and 104% respectively for the AI configuration and a gain of 0,24% with encoding and decoding times of 102% and 102% respectively for the RA configuration.

An additional test result for Test 1.1.2 shows a gain of 0,52% with encoding and decoding times of 112% and 104% respectively for the AI configuration and a gain of 0,25% with encoding and decoding times of 102% and 102% respectively for the RA configuration.

Two simplifications have been additionally simulated on top of Test 1.1.1. Their combined effect shows a gain of 0.57% and 0.2x% in AI and RA respectively.

# Introduction

The Intra Sub-Partitions (ISP) coding mode is an updated version of the Line-Based Intra (LIP) coding that corrects the hardware-related issues of the previous design of the algorithm, as explained in [1]. The ISP tool divides luma intra-predicted blocks vertically or horizontally into 2 or 4 sub-partitions depending on the block size dimensions, as shown in Table 1. Figure 1 and Figure 2 show examples of the two possibilities. All sub-partitions fulfill the condition of having at least 16 samples.

Table 1: Number of sub-partitions depending on the block size

| Block Size | Number of Sub-Partitions |
| --- | --- |
| 4×4 | Not divided |
| 4×8 and 8×4 | 2 |
| All other cases | 4 |

Figure 1: Example of division of 4×8 and 8×4 blocks

Figure 2: Example of division of all blocks except 4×8, 8×4 and 4×4

For each of these sub-partitions, a residual signal is generated by entropy decoding the coefficients sent by the encoder and then inverse quantizing and inverse transforming them. Then, the sub-partition is intra predicted and finally the corresponding reconstructed samples are obtained by adding the residual signal to the prediction signal. Therefore, the reconstructed values of each sub-partition will be available to generate the prediction of the next one, which will repeat the process and so on. All sub-partitions share the same intra mode.

Based on the intra mode and the split utilized, two different classes of processing orders are used, which are referred to as normal and reversed order. In the normal order, the first sub-partition to be processed is the one containing the top-left sample of the CU and then continuing downwards (horizontal split) or rightwards (vertical split). As a result, reference samples used to generate the sub-partitions prediction signals are only located at the left and above sides of the lines.  On the other hand, the reverse processing order either starts with the sub-partition containing the bottom-left sample of the CU and continues upwards or starts with sub-partition containing the top-right sample of the CU and continues leftwards.

A more detailed explanation of the algorithm can be found in [1].

Despite successfully reducing the hardware-related complexity of the tool, during the Macao meeting some concerns were raised regarding the encoding run-time of the tool (148% in the case of All Intra for one of the tests). For this reason, it was proposed in [2] to repeat the tests carried out in Macao with the aim of obtaining this time a better trade-off between the coding gain and the encoder run-time. Therefore, the tests are described as follows:

- Test 1.1.1 : ISP with a better trade-off between coding gain and encoder run-time.

- Test 1.1.2 : Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples. This implies that 4×N blocks can no longer be divided vertically (and therefore no split type parameter signaling is required) and 8×N blocks are divided in 2 sub-partitions if the split is vertical (instead of 4).

The modifications of the algorithm are presented in the next section.

# Modifications of the algorithm

## Interaction with Multiple Reference Line (MRL)

If a block has an MRL index other than 0, then the ISP coding mode will be inferred to be 0 and therefore it will not be sent to the decoder.

## Entropy coding coefficient group size

The sizes of the entropy coding sub-blocks used for the significance map have been modified so that they have 16 samples in all possible cases, as shown in Table 2. Note that the new sizes only affect blocks produced by ISP in which one of the dimensions is less than 4 samples. In all other cases coefficient groups keep the 4×4 dimensions.

Table 2: Entropy coding coefficient group sizes

| Block Size | Coefficient group Size |
| --- | --- |
| 1×N, N≥16 | 1×16 |
| N×1, N≥16 | 16×1 |
| 2×N, N≥8 | 2×8 |
| N×2, N≥8 | 8×2 |
| All other possible M×N cases | 4×4 |

## At least one non-zero CBF

Given a certain block using ISP, the decoder will assume that at least one of the sub-partitions has a non-zero CBF. For this reason, if n is the number of sub-partitions and the first n-1 sub-partitions have produced a zero CBF, then the CBF of the n-th sub-partition will be inferred to be 1. Therefore, it is not necessary to transmit and decode it.

## MPM usage

The ISP algorithm will only be tested with intra modes that are part of the MPM list. For this reason, if a block uses ISP, then the MPM flag will be inferred to be one. Besides, if ISP is used for a certain block, then the MPM list will be modified to exclude the DC mode and to prioritize horizontal intra modes for the ISP horizontal split and vertical intra modes for the vertical one.

## Encoder estimation process

To decide which modes will be fully RD tested, the encoder will apply the following process:

- Create the intra mode lists ISP-H and ISP-V for the horizontal and vertical ISP splits respectively.

- Initialize both lists with the intra modes of the fast candidate list created by normal (non-ISP) intra prediction.

- Remove modes that are not MPMs in both ISP lists according to their respective split type.

- Full RD test normal intra prediction (including MRL modes) using the first MTS pass.

  - After each intra mode is tested, divide the unquantized DCT-II coefficients in 3 regions: top-right triangle, bottom-left triangle and diagonal (it includes the DC coefficient).

  - Obtain the sum of absolute values of each region (topRight, belowLeft and diag).

  - Calculate the ratios r1=topRight/belowLeft and r2=diag/(topRight + belowLeft)

  - Repeat this process for all intra modes and save only the ratios of the intra mode that has the lowest cost.

- If r1 is greater than a certain threshold, discard all modes in ISP-H. Otherwise, if r1 is lower than a certain threshold, discard all modes in ISP-V.

- If r2 is greater than a certain threshold, discard all modes in ISP-H and ISP-V.

- Full RD test the remaining modes of ISP-H and ISP-V

- If the best cost of normal intra prediction is greater than the best cost of ISP times a threshold, discard the second MTS pass for normal intra prediction.

# Experimental results

According to the CE3 description [2] and the common test conditions [3], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-3.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler.

## Test 1.1.1

Table 3: Results for Test 1.1.1 with an AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.16% | -0.14% | -0.10% | 113% | 101% |
| Class A2 | -0.32% | -0.22% | -0.25% | 113% | 101% |
| Class B | -0.55% | -0.44% | -0.50% | 113% | 105% |
| Class C | -0.86% | -0.64% | -0.77% | 112% | 104% |
| Class E | -0.98% | -0.66% | -0.59% | 109% | 104% |
| Overall | -0.59% | -0.44% | -0.47% | 112% | 103% |
| Class D | -0.74% | -0.63% | -0.76% | 112% | 104% |
| Class F | -2.08% | -1.44% | -1.60% | 103% | 104% |

Table 4: Results for Test 1.1.1 with a RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.16% | -0.09% | 0.00% | 103% | 104% |
| Class A2 | -0.26% | -0.12% | 0.07% | 102% | 102% |
| Class B | -0.30% | -0.25% | -0.28% | 103% | 102% |
| Class C | -0.40% | -0.69% | -0.27% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.31% | -0.15% | 102% | 103% |
| Class D | -0.30% | -0.71% | -0.28% | 103% | 99% |
| Class F | -1.65% | -1.16% | -1.08% | 101% | 104% |

## Test 1.1.2

Table 5: Results for Test 1.1.2 with an AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.14% | -0.15% | -0.03% | 114% | 100% |
| Class A2 | -0.29% | -0.21% | -0.23% | 113% | 102% |
| Class B | -0.49% | -0.37% | -0.41% | 113% | 105% |
| Class C | -0.70% | -0.55% | -0.60% | 112% | 104% |
| Class E | -0.58% | -0.34% | -0.28% | 109% | 104% |
| Overall | -0.46% | -0.34% | -0.34% | 112% | 104% |
| Class D | -0.49% | -0.41% | -0.41% | 111% | 103% |
| Class F (optional) | -1.67% | -1.18% | -1.03% | 103% | 105% |

Table 6: Results for Test 1.1.2 with a RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.14% | -0.32% | -0.19% | 102% | 101% |
| Class A2 | -0.21% | -0.15% | 0.04% | 102% | 102% |
| Class B | -0.26% | -0.26% | -0.37% | 102% | 102% |
| Class C | -0.34% | -0.36% | -0.12% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.28% | -0.18% | 102% | 102% |
| Class D | -0.22% | -0.45% | -0.08% | 103% | 99% |
| Class F (optional) | -1.34% | -0.84% | -0.78% | 100% | 102% |

## Additional results

### Usage of a 1-D transform and entropy coding for vertically non-divisible blocks

As mentioned before, 4×N blocks cannot be vertically divided in Test 1.1.2 and therefore the split flag parameter is not transmitted, since only the horizontal split is possible. In this additional configuration, the split flag is transmitted to the decoder again:

- If the value is 0, a horizontal split will be performed in the same way as the normal configuration of Test 1.1.2

- If the value is 1, the 4×N (with N>4) block will be predicted using the exactly vertical intra mode (50) and then a 1-D vertical transform will be performed on the generated residual signal. The resulting coefficients will be quantized and entropy-coded as independent 1-D arrays. In this case, the CBF of each line will be independently coded from the CBFs of other coefficient lines. Note that in this case the prediction stage is exactly the same as in normal intra prediction. The only difference lies in the transform and the entropy coding. Since there is no intra dependency, all lines can be transformed and entropy coded at the same time. Besides, only the above reference samples are needed to create the prediction. No signaling will be used to transmit the intra mode, since it is inferred to be 50.

Table 7: Results for Test 1.1.2 with an AI configuration using a 1-D transform and entropy coding for 4×N (with N>4) blocks that cannot be divided vertically

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.13% | -0.12% | 114% | 103% |
| Class A2 | -0.28% | -0.19% | -0.19% | 113% | 102% |
| Class B | -0.50% | -0.41% | -0.42% | 113% | 104% |
| Class C | -0.76% | -0.64% | -0.79% | 112% | 105% |
| Class E | -0.82% | -0.52% | -0.44% | 109% | 104% |
| Overall | -0.52% | -0.40% | -0.42% | 112% | 104% |
| Class D | -0.63% | -0.53% | -0.62% | 112% | 104% |
| Class F (optional) | -2.07% | -1.44% | -1.42% | 103% | 104% |

Table 8: Results for Test 1.1.2 with a RA configuration using a 1-D transform and entropy coding for 4×N (with N>4) blocks that cannot be divided vertically

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.14% | -0.08% | 102% | 105% |
| Class A2 | -0.20% | -0.17% | 0.03% | 102% | 101% |
| Class B | -0.27% | -0.13% | -0.22% | 102% | 102% |
| Class C | -0.34% | -0.53% | -0.27% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.25% | -0.16% | 102% | 102% |
| Class D | -0.31% | -0.55% | -0.26% | 103% | 100% |
| Class F (optional) | -1.63% | -0.99% | -1.04% | 100% | 102% |

It can be observed that this configuration increases the overall gain (and class F more significantly) and it does not have an impact on the encoding run-time.

### Simplification: Non-DCT-II transform size restriction

All ISP transforms with a length larger than 16 points will use the DCT-2. This simplification is motivated by the JVET AHG report: Implementation studies (AHG16, document JVET-M0016), that identifies 32x32/32x16/16x32 DST7/DCT8 based transform units as the bottleneck for the implementation of primary transforms. Results show a gain of 0.02% in AI and no loss in RA.

Table 9: Results for Test 1.1.1 with an AI configuration using the non-DCT-II transform size restriction

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.13% | -0.11% | 113% | 101% |
| Class A2 | -0.33% | -0.24% | -0.23% | 112% | 99% |
| Class B | -0.57% | -0.48% | -0.47% | 112% | 101% |
| Class C | -0.88% | -0.61% | -0.82% | 112% | 104% |
| Class E | -1.02% | -0.71% | -0.67% | 108% | 101% |
| Overall | -0.61% | -0.45% | -0.48% | 111% | 101% |
| Class D | -0.75% | -0.65% | -0.68% | 110% | 103% |
| Class F (optional) | -2.09% | -1.52% | -1.50% | 102% | 103% |

Table 10: Results for Test 1.1.1 with a RA configuration using the non-DCT-II transform size restriction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.22% | -0.09% | 102% | 99% |
| Class A2 | -0.26% | -0.27% | 0.06% | 102% | 102% |
| Class B | -0.32% | -0.22% | -0.10% | 102% | 99% |
| Class C | -0.39% | -0.50% | -0.29% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.30% | -0.12% | 102% | 100% |
| Class D | -0.30% | -0.55% | -0.38% | 102% | 100% |
| Class F (optional) | -1.65% | -1.18% | -1.12% | 100% | 101% |

### Simplification: Removal of the reversed processing orders

This simplification replaces the reversed orders of the algorithm (from the right to the left and from the bottom to the top) and replaces it with the normal ones (from the top to the bottom and from the left to the right). It is implemented on top of the previous simplification (section 3.3.3). Results show a loss of 0.04% in AI and 0.0x% in RA, while eliminating the complexity related to the usage of the reversed processing orders.

Table 11: Results for Test 1.1.1 with an AI configuration using the non-DCT-II transform size restriction and disabling the reversed order

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.15% | -0.08% | 114% | 100% |
| Class A2 | -0.30% | -0.13% | -0.22% | 113% | 99% |
| Class B | -0.52% | -0.43% | -0.38% | 113% | 103% |
| Class C | -0.84% | -0.62% | -0.80% | 112% | 105% |
| Class E | -0.97% | -0.68% | -0.54% | 109% | 103% |
| Overall | -0.57% | -0.41% | -0.42% | 112% | 102% |
| Class D | -0.70% | -0.59% | -0.58% | 111% | 103% |
| Class F (optional) | -2.01% | -1.35% | -1.40% | 103% | 104% |

Table 12: Results for Test 1.1.1 with a RA configuration using the non-DCT-II transform size restriction and disabling the reversed order

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.26% | -0.15% | -0.29% | 102% | 100% |
| Class C | -0.41% | -0.43% | -0.39% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.30% | -0.69% | -0.22% | 102% | 99% |
| Class F (optional) | -1.61% | -0.81% | -0.79% | 100% | 101% |

# Cross-Check

Cross-checks for the tests have been performed by ITRI.

# Conclusion

The reported results show gains in all the tests and configurations. Class F yields clearly the highest BD-Rates. This is probably due to the fact screen content is typically richer in sharp edges, which can be better modelled with smaller partitions. On the other hand, for this type of sequences the difference in the gains reported in the AI and RA configurations is much smaller than typically.

Regarding the individual tests behaviors, Test 1.1.1 obtains the highest gains, as expected. Test 1.1.2 has lower gains than Test 1.1.1 in terms of BD-Rate, given that the ISP algorithm is applied on a smaller number of all the blocks available. However, it reduces the hardware implementation complexity with respect to Test 1.1.1.

On the other hand, the additional test presented in section 3.3.1 improves the gain of Test 1.1.2 while avoiding an increase in the encoding run-time.

Finally, the two simplifications presented as additional tests (sections 3.3.2 and 3.3.3) reduce the hardware complexity while a avoiding a significant BD-Rate loss.

# References

| [1] | S. De-Luxán-Hernández, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Line-based intra coding mode (Tests 2.1.1 and 2.1.2)," Document JVET-L0076, Macao, CN, 2018. |
| --- | --- |
| [2] | G. Van der Auwera, J. Heo and A. Filippov, "Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding," Document JVET-L1023, Macao, CN, 2018. |
| [3] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring (editors), "JVET common test conditions and software reference configurations for SDR video," Document JVET-L1010, Macao, CN, 2018. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).