| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0115-v1 |
| --- | --- |

| Title: | CE10-related: pipeline reduction for LIC and GBI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Tangi Poirier | Tel: Email: | +33.2.99.27.32.42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to reduce pipeline for LIC by disabling GBI when LIC is enabled at CU level.

The proposed process has been implemented in the JVET VTM3.0 on top of regular LIC (CE10.5.2). The reported simulation results show an average BD-rate gain of -0.54%,-0.26%,-0.33% in Y,Cb,Cr in RA configuration and of -0.48%,-0.54%,-0.46% in LDB configuration. The encoding and decoding times are reduced compared to the regular LIC.

# Introduction / Problem Statement / …

The Local Illumination Compensation (LIC) is an inter tool which allows compensating temporal illumination variation locally, between samples from reference frame (inter-prediction) and current frame. In the worst case, in inter, the two processes LIC and GBI with non-default weights can be applied successively (Figure 1).

Figure 1: overview of the VTM-3.0 pipeline in inter (top) and intra (bottom).

# Proposed pipeline reduction for LIC

When LIC and GBI with non-default weights are enabled together, two successive weighting and clipping stages are cascaded, what may be sub-optimal. It may be also counter-productive to apply additional GBI non-default weights on top of LIC weights that were specially computed to compensate illumination variation.

To overcome this inconsistency and to alleviate the inter pipeline when LIC is enabled, it is proposed:

- Mutual exclusion of LIC and GBI.

- LIC-flag is decoded first. If LIC-flag is true, then GBi-index is not decoded and inferred to defaults.

Figure 2: proposed reduced LIC pipeline in inter (top).

The Table 1 summarizes the results of the regular LIC (left) and the proposed reduced pipeline for LIC (right) implemented on VTM-3.0 with CTC, on random access (RA) and low delay B (LDB). The BD-rate gains of the proposed LIC pipeline are very close to regular LIC, while the encoder/decoder runtimes are reduced.

Besides alleviating the decoder pipeline worst case, the proposed modification allows reducing the encoder modes combinatory.

Table 1: results of the regular LIC (left) and the proposed reduced pipeline for LIC (right).

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular LIC VTM-3.0 (CE10.5.2) |  |  |  |  | Proposed LIC excludes GBI VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -0.60% | -0.91% | 148% | 100% | -0.93% | -0.47% | -0.83% | 144% | 98% |
| Class A2 | -0.38% | -0.14% | -0.22% | 125% | 100% | -0.36% | -0.17% | -0.23% | 122% | 100% |
| Class B | -0.60% | -0.22% | -0.24% | 131% | 101% | -0.57% | -0.15% | -0.20% | 129% | 99% |
| Class C | -0.35% | -0.34% | -0.23% | 138% | 103% | -0.33% | -0.32% | -0.22% | 136% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.56% | -0.31% | -0.37% | 135% | 101% | -0.54% | -0.26% | -0.33% | 132% | 99% |
| Class D | -0.04% | -0.04% | -0.15% | 134% | 102% | -0.02% | -0.01% | -0.10% | 133% | 101% |
| Class F | -0.88% | -0.57% | -0.51% | 131% | 103% | -0.85% | -0.61% | -0.55% | 129% | 101% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Regular LIC VTM-3.0 (CE10.5.2) |  |  |  |  | Proposed LIC excludes GBI VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.77% | -0.85% | -0.80% | 147% | 101% | -0.75% | -0.87% | -0.59% | 144% | 99% |
| Class C | -0.49% | -0.22% | -0.54% | 159% | 102% | -0.47% | -0.39% | -0.55% | 157% | 101% |
| Class E | -0.08% | -0.04% | -0.27% | 105% | 102% | -0.03% | -0.21% | -0.11% | 104% | 98% |
| Overall | -0.51% | -0.44% | -0.58% | 139% | 101% | -0.48% | -0.54% | -0.46% | 137% | 99% |
| Class D | -0.11% | -0.44% | -0.39% | 149% | 101% | -0.11% | -0.14% | -0.26% | 147% | 99% |
| Class F | -2.64% | -2.10% | -1.93% | 149% | 100% | -2.65% | -1.84% | -1.82% | 147% | 99% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).