| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0126 |
| --- | --- |

| Title: | Suggested specification changes for JVET-M0126 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Chun-Chi Chen, Han Huang, Chao-Hsiung Hung, Yan Zhang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document describes the suggested specification changes on top of version 7 of JVET-L1001. The newly added parts are highlighted in yellow and the text to be removed is marked as red with strikethrough

# Decoding process

#### 8.3.2.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the bi-prediction weight index gbiIdx.

The bi-prediction weight index gbiIdx is set equal to 0.

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.3.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, and the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, gbiIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index gbiIdxCol for the temporal merging candidate Col are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-196)

predFlagL0Col = availableFlagL0Col		(8-197)

predFlagL1Col = 0		(8-198)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-199)

predFlagL1Col = availableFlagL1Col		(8-200)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1   numLeftAboveMergeCand = i
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-201)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numSpatialTemporalMergeCand, numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and NumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.3.2.6 is invoked with mergeCandList, and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- The derivation process for pairwise average merging candidates specified in clause 8.3.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every spatial and temporal candidate N in mergeCandList, numSpatialTemporalMergeCand, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCandk and refIdxL1avgCandk, the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk and the motion vectors mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate avgCandk being added into mergeCandList is set equal to 0. The number of candidates being added, numAvgMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numAvgMergeCand is greater than 0, k ranges from 0 to numAvgMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in clause 8.3.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The bi-prediction weight index gbiIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numAvgMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(8-202)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-203)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-204)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-205)

gbiIdx = gbiIdxN		(8-206)

- When mmvd_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.3.2.7 is invoked with the luma location ( xCb, yCb ), the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(8-207)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(8-208)

#### 8.3.2.4 Derivation process for pairwise average merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every spatial and temporal candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numSpatialTemporalMergeCand, numCurrMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList before pairwise average merging candidate derivation process.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCandk and refIdxL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process.

The variables numCutoff, avgIdx, avgStop, numRefLists are derived as follows:

numCutoff = Min( numCurrMergeCand, 4 ) numCutoff = Min( numSpatialTemporalMergeCand, 4 )	(8-229)

avgIdx = 0	(8-230)

avgStop = (avgIdx >= ( numCutoff * ( numCutoff − 1 ) / 2 )  ||  	(8-231)
	numCurrMergeCand  = =  MaxNumMergeCand )

numRefLists  =  ( slice_type = = B )  ?  2 : 1	(8-232)

The following ordered steps are repeated until avgStop is equal to TRUE:

- The variables p0CandIdx and p1CandIdx are derived using avgIdx as specified in Table 8-7.

- The following assignments are made, with p0Cand being the candidate at position p0CandIdx and p1Cand being the candidate at position p1CandIdx in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ p0CandIdx ]	(8-233)

p1Cand = mergeCandList[ p1CandIdx ]	(8-234)

- The candidate avgCandk with k equal to ( numCurrMergeCand − numOrigMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCandk, and the reference indices, the prediction list utilization flags and the motion vectors of avgCandk are derived as follows and numCurrMergeCand is incremented by 1:

- For each reference picture list LX with X ranging from 0 to ( numRefLists − 1 ), the following applies:

  - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], and mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-235)

predFlagLXavgCandk = 1	(8-236)

mvLXavgCandk[ 0 ] = ( mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ] ) / 2	(8-237)

mvLXavgCandk[ 1 ] = ( mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ] ) / 2	(8-238)

  - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp0Cand	(8-239)

predFlagLXavgCandk = 1	(8-240)

mvLXavgCandk[ 0 ] = mvLXp0Cand[ 0 ] 	(8-241)

mvLXavgCandk[ 1 ] = mvLXp0Cand[ 1 ] 	(8-242)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = refIdxLXp1Cand	(8-243)

predFlagLXavgCandk = 1	(8-244)

mvLXavgCandk[ 0 ] = mvLXp1Cand[ 0 ] 	(8-245)

mvLXavgCandk[ 1 ] = mvLXp1Cand[ 1 ] 	(8-246)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCandk, predFlagLXavgCandk, mvLXavgCandk[ 0 ], mvLXavgCandk[ 1 ] are derived as follows:

refIdxLXavgCandk = −1	(8-247)

predFlagLXavgCandk = 0	(8-248)

mvLXavgCandk[ 0 ] = 0	(8-249)

mvLXavgCandk[ 1 ] = 0	(8-250)

- When numRefLists is equal to 1, the following applies:

refIdxL1avgCandk = −1	(8-251)

predFlagL1avgCandk = 0	(8-252)

- The variable avgIdx is incremented by 1.

- When avgIdx is equal to ( numCutoff * ( numCutoff − 1 ) / 2 ) or numCurrMergeCand is equal to MaxNumMergeCand, avgStop is set equal to TRUE.

Table 8-7 – Specification of p0CandIdx and p1CandIdx

| avgIdx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| p0CandIdx | 0 | 0 | 1 | 0 | 1 | 2 |
| p1CandIdx | 1 | 2 | 2 | 3 | 3 | 3 |

#### 8.3.2.6 Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

Each element of the array isPruned[ i ] with i = 0..numCurrMergeCand − 1 is set equal to FALSE and the variable numOrigMergeCand is set equal to numCurrMergeCand.

The variable sampleIdx is set equal to 1 + j * 3, with j = 0, 1, 2, 3, sampleIdx ranges from 1 to NumHmvpCand. For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx =(addressRead + 10 – sampleIdx + 1) % 10, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any i = 0..numOrigMergeCand numLeftAboveMergeCand − 1, sameMotion and isPruned[ i ] are both set equal to TRUE:

  - sampleIdx <= 4.

  - The candidate HmvpCandList[NumHmvpCand − hMvpIdx] HmvpCandList[hMvpIdx] is equal to the merging candidate mergeCandList[ i ].

  - isPruned[ i ] is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate HmvpCandList[NumHmvpCand − hMvpIdx] HmvpCandList[hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = HmvpCandList[ NumHmvpCand − hMvpIdx ] HmvpCandList[hMvpIdx]  	(8-271)

#### 8.3.2.9 Derivation process for motion vector predictor candidate list

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the reference index of the current coding unit partition refIdxLX, with X being 0 or 1.

Output of this process is motion vector predictor candidate list mvpListLX in 1/16 fractional-sample accuracy with X being 0 or 1.

The motion vector predictor candidate list mvpListLX with X being 0 or 1 is derived in the following ordered steps:

- The derivation process for spatial motion vector predictor candidates from neighbouring coding unit partitions as specified in clause 8.3.2.10 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and refIdxLX, with X being 0 or 1 as inputs, and the availability flags availableFlagLXN and the motion vectors mvLXN, with N being replaced by A or B, as output.

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to mvLXN, with N being replaced by A or B, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvLXN, with N being replaced by A or B, as output.

- If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0.

- Otherwise, the following applies:

- The derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and refIdxLX, with X being 0 or 1 as inputs, and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to mvLXCol, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvLXCol as output.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows:

numCurrMvpCand = 0
if( availableFlagLXA ) {
	mvpListLX[ numCurrMvpCand++ ] = mvLXA
	if( availableFlagLXB  &&  ( mvLXA  !=  mvLXB ) )
		mvpListLX[ numCurrMvpCand++ ] = mvLXB	(8-297)
} else if( availableFlagLXB )
	mvpListLX[ numCurrMvpCand++ ] = mvLXB
if( numCurrMvpCand < 2  &&  availableFlagLXCol )
	mvpListLX[ numCurrMvpCand++ ] = mvLXCol

- When numCurrMvpCand is less than 2 and NumHmvpCand is greater than 0, the following applies for i= 1..Min( 4, NumHmvpCand ) i=1 + j * 3, with j = 0, 1, 2, 3, i ranges from 1 to NumHmvpCand, hMvpIdx =(addressRead + 10 – i + 1) % 10, until numCurrMvpCand is equal to 2:

- For each reference picture list LY with Y = 0..1, the following applies until numCurrMvpCand is equal to 2:

- When all of the following conditions are true, mvpListLX[ numCurrMvpCand++ ] is set to equal to the LY motion vector of the candidate HmvpCandList[ NumHmvpCand − i ] HmvpCandList[ hMvpIdx ]:

- The LY reference index of the history-based motion vector predictor candidate HmvpCandList[i] HmvpCandList[ hMvpIdx ] is equal to refIdxLX

- The motion vector predictor candidate list is empty (numCurrMvpCan is equal to 0) or the LY motion vector of the candidate HmvpCandList[ NumHmvpCand − i ] HmvpCandList[ hMvpIdx ] is not equal to mvpListLX[j] for j = 0.. numCurrMvpCand − 1.

- When numCurrMvpCand is less than 2, the following applies for until numCurrMvpCand is equal to 2:

mvpListLX[ numCurrMvpCand ][ 0 ] = 0		(8-298)

mvpListLX[ numCurrMvpCand ][ 1 ] = 0		(8-299)

numCurrMvpCand++		(8-300)

#### 8.3.2.15 Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

The MVP candidate hMvpCand consists of the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, and the bi-prediction weight index gbiIdx set equal to 0.

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The addressRead = addressWrite.

- HmvpCandList[ addressWrite ] is set equal to mvCand.

- The addressWrite is set equal to (addressWrite + 1) % 10.

- If NumHmvpCand < 6

  - NumHmvpCand++.

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpCand is equal to 6, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpCand − 1 ), HmvpCandList[ i − 1] is set equal to HmvpCandList[ i ].

  - HmvpCandList[ NumHmvpCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpCand is less than 6), the following applies:

  - HmvpCandList[ NumHmvpCand++ ] is set equal to mvCand.