| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0136-v1 |
| --- | --- |

| Title: | AHG12: Treating tile and tile group boundaries as picture boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen 2330 Central Expy Santa Clara, CA 95050, USA Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Maxim Sychev Building 1/7, Altufievskoe Shosse, Moscow 127106, Russia | Tel: Email: | +1-858-999-4243 jianle.chen@huawei.com +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +7-925-008-1124 sychev.maxim@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC, motion-constrained tile sets (MCTSs) boundaries are not treated as picture boundaries unless they collocate with picture boundaries. It is asserted, however, that treating MCTS boundaries would make coding of MCTSs more efficient thanks to the padding to the boundaries and allowing motion vectors to point to samples outside the boundaries in the reference pictures. This contribution therefore proposes to enable treating all MCTS boundaries as picture boundaries by enabling treating each tile or tile group boundary as picture boundary.

# Proposal

The proposal is summarized as follows:

- A flag, tile_boundary_treated_as_pic_boundary_flag, is added to the PPS, to specify whether each tile boundary is treated the same as picture boundary in the decoding process.

- A flag, tile_group_boundary_treated_as_pic_boundary_flag, is added to the tile group header, to specify whether each tile group boundary of the tile group is treated the same as picture boundary in the decoding process.

- When the boundaries of tiles or a tile group are indicated to be treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the tile or slice, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile or slice, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile or slice, in units of chroma samples, respectively.

The specification text, relative to the basis text in JVET-L0686-v2, is provided below, where the texts in the basis text that are not mentioned below apply as they are, and modified texts relative to the basis text are highlighted in yellow, and some removals are shown strikethroughs in red fonts.

### Picture parameter set RBSP syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| tile_boundary_treated_as_pic_boundary_flag | u(1) |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

...

tile_boundary_treated_as_picture_boundary_flag equal to 1 specifies that each tile boundary is treated the same as picture boundary in the decoding process described in clause 8 for pictures referring to the PPS. tile_boundary_treated_as_picture_boundary_flag equal to 0 specifies that each tile boundry may or may not be treated the same as picture boundary in the decoding process described in clause 8 for pictures referring to the PPS. When not present, the value of tile_boundary_treated_as_picture_boundary_flag is inferred to be equal to 0.

...

### Tile group header RBSP syntax and semantics

| tile_group_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !tile_boundary_treated_as_pic_boundary_flag ) |  |
| tile_group_boundary_treated_as_pic_boundary_flag |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

...

tile_group_boundary_treated_as_pic_boundary_flag equal to 1 specifies that each tile group boundary of the tile group is treated the same as picture boundary in the decoding process described in clause 8. tile_group_boundary_treated_as_pic_boundary_flag equal to 0 specifies that each tile group boundary may or may not be treated the same as picture boundary in the decoding process described in clause 8. When not present, the value of tile_group_boundary_treated_as_pic_boundary_flag is inferred to be equal to tile_boundary_treated_as_pic_boundary_flag.

...

### General tile group data semantics

end_of_tile_one_bit shall be equal to 1.

For each tile, the variables LeftBoundaryPos, TopBoundaryPos, RightBoundaryPos and BotBoundaryPos are derived as follows:

–	If tile_boundary_treated_as_pic_boundary_flag is equal to TRUE, the following applies:

tileColIdx = CurrTileIdx % ( num_tile_columns_minus1 + 1 )	(7-39)

tileRowIdx = CurrTileIdx / ( num_tile_columns_minus1 + 1 )	(7-40)

LeftBoundaryPos = ColBd[ tileColIdx ]  <<  CtbLog2SizeY	(7-41)

RightBoundaryPos = ( ( ColBd[ tileColIdx ] + ColWidth[ tileColIdx ] )  <<  CtbLog2SizeY ) − 1	(7-42)

TopBoundaryPos = RowBd[ tileRowIdx ]  <<  CtbLog2SizeY	(7-43)

BotBoundaryPos = ( ( RowBd[ tileRowIdx ] + RowHeight[ tileRowIdx ] )  <<  CtbLog2SizeY ) − 1	(7-44)

–	Otherwise if tile_group_boundary_treated_as_pic_boundary_flag is equal to TRUE, the following applies:

sliceFirstTileColIdx = tile_group_address % ( num_tile_columns_minus1 + 1 )	(7-45)

sliceFirstTileRowIdx = tile_group_address / ( num_tile_columns_minus1 + 1 )	(7-46)

sliceLastTileColIdx = ( tile_group_address + num_tiles_in_tile_group_minus1 ) %
						( num_tile_columns_minus1 + 1 )	(7-47)

sliceLastTileRowIdx = ( tile_group_address + num_tiles_in_tile_group_minus1 ) /
						( num_tile_columns_minus1 + 1 )	(7-48)

LeftBoundaryPos = ColBd[ sliceFirstTileColIdx ]  <<  CtbLog2SizeY	(7-49)

RightBoundaryPos = ( ( ColBd[ sliceLastTileColIdx ] + ColWidth[ sliceLastTileColIdx ] )  <<
							CtbLog2SizeY ) − 1		(7-50)

TopBoundaryPos = RowBd[ sliceFirstTileRowIdx ]  <<  CtbLog2SizeY	(7-51)

BotBoundaryPos = ( ( RowBd[ sliceLastTileRowIdx ] + RowHeight[ sliceLastTileRowIdx ] )  <<
							CtbLog2SizeY ) −1		(7-52)

–	Otherwise (tile_group_boundary_treated_as_pic_boundary_flag is equal to FALSE), the following applies:

LeftBoundaryPos = 0		(7-53)

RightBoundaryPos = pic_width_in_luma_samples − 1	(7-54)

TopBoundaryPos = 0		(7-55)

BotBoundaryPos = pic_height_in_luma_samples − 1	(7-56)

### 8.3.2.11 Derivation process for temporal luma motion vector prediction

...

- If yCb  >>  CtbLog2SizeY is equal to yColBr  >>  CtbLog2SizeY, yColBr is less than pic_height_in_luma_samples or equal to BotBoundaryPos and xColBr is less than pic_width_in_luma_samples or equal to RightBoundaryPos, the following applies:

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColBr  >>  3 )  <<  3, ( yColBr  >>  3 )  <<  3 ) inside the collocated picture specified by ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.9 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxLX , and control parameter controlParaFlag set equal to 0 as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

...

8.3.6.3.2	Luma sample 8-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If tile_boundary_treated_as_pic_boundary_flag is equal to 1 or tile_group_boundary_treated_as_pic_boundary_flag is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos, RightBoundaryPos, xIntL + i − 3 )	(8-673)

yInti = Clip3( TopBoundaryPos, BotBoundaryPos, yIntL + i − 3 )	(8-674)

–	Otherwise (both tile_boundary_treated_as_pic_boundary_flag and tile_group_boundary_treated_as_pic_boundary_flag are equal to 0), the following applies:

xInti = sps_ref_wraparound_enabled_flag  ?	(8-673)
	ClipH( sps_ref_wraparound_offset, picW, ( xIntL + i − 3 ) ):  Clip3( 0, picW − 1, xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-674)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(8-675)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(8-676)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ yFracL ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(8-677)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-678)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ yFracL ][ i ] * temp[ i ]   >>  shift2	(8-679)

...

### 8.3.6.3.3	Luma sample bilinear interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-10.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If tile_boundary_treated_as_pic_boundary_flag is equal to 1 or tile_group_boundary_treated_as_pic_boundary_flag is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos, RightBoundaryPos, xIntL + i )	(8-673)

yInti = Clip3( TopBoundaryPos, BotBoundaryPos, yIntL + i )	(8-674)

–	Otherwise (both tile_boundary_treated_as_pic_boundary_flag and tile_group_boundary_treated_as_pic_boundary_flag are equal to 0), the following applies:

xInti = sps_ref_wraparound_enabled_flag  ? 		(8-680)
	ClipH( sps_ref_wraparound_offset, picW, ( xIntL + i − 3 ) ) : Clip3( 0, picW − 1, xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(8-681)

...

### 8.3.6.3.4	Chroma sample interpolation process

##### Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The luma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-11.

The variable xOffset is set equal to sps_ref_wraparound_offset / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If tile_boundary_treated_as_pic_boundary_flag is equal to 1 or tile_group_boundary_treated_as_pic_boundary_flag is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos/SubWidthC, RightBoundaryPos/SubWidthC, xIntL + i )	(8-687)

yInti = Clip3( TopBoundaryPos/SubHeightC, BotBoundaryPos/SubHeightC, yIntL + i )	(8-687)

–	Otherwise (both tile_boundary_treated_as_pic_boundary_flag and tile_group_boundary_treated_as_pic_boundary_flag are equal to 0), the following applies:

xInti = sps_ref_wraparound_enabled_flag  ?  ClipH( xOffset, picWC, ( xIntC + i − 1 ) )	(8-687)
	:  Clip3( 0, picWC − 1, xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(8-688)

...

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).