| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0146-v1 |
| --- | --- |

| Title: | Non-CE3: MDLM template downsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Multi-direction linear model (MDLM) prediction was adopted in VTM-3.0 to further improve the coding efficiency of cross-component linear model prediction. MDLM is an alternative to cross-component linear model (CCLM), where either the left template or the above template is used to calculate the linear model parameters. MDLM uses extended templates, which includes non-adjacent reconstructed samples. However, if the chroma CU is not square, the number of samples used in linear model parameters estimation may be increased in MDLM compared to CCLM. This may impact the throughput on hardware implementations. In this contribution, it is proposed to downsample the MDLM templates to use the same number of samples to calculate the linear model parameters in MDLM as in CCLM when the chroma CU is not square. In this way, the number of operations to calculate the linear model parameters can be identical for CCLM and MDLM, independently of the CU aspect ratio. Overall results for all intra configuration show a slight gain of -0.04% and -0.06% for the U and V components, respectively. For RA and LB configurations, it has similar results as anchors.

# Problem statement

In CCLM [1], when the current chroma CU is not square, the template on the longer edge is dowsampled such that the number of samples used in CCLM is always 2×min⁡(W,H), where W and H are width and height of the chroma CU, respectively. For example, if the chroma CU is of size 2N×N, 2N samples are used for the determination of the linear model parameters: N samples for the left template and N samples from the downsampled above template. In this example, the above template is downsampled in CCLM by using every other samples in the above template.

MDLM [1] uses either the left template or the above template to calculate the linear model parameters, called LM_L and LM_A modes, respectively. In LM_A mode, only the above template is used to calculate the linear model parameters, but the above template is extended to twice the CU width (2W), as depicted in Figure 1. Similarly, in LM_L mode, only the left template is used to calculate the linear model parameters, but the left template is extended to twice the CU height (2H). Thus, for non-square chroma CUs, the number of samples used in MDLM may be larger than the number of samples used in CCLM. For example, if the current chroma CU is of size 2N×N, 2N samples are used for the left template, but 4N samples are used for the above template, which is twice the number of samples that is used in CCLM.

Figure 1. Locations of the samples used for derivation of linear model parameters in LM_A mode.

# MDLM template downsampling

In this contribution, it is proposed to downsample the MDLM templates to use the same number of samples to calculate the linear model parameters in MDLM as in CCLM when the chroma CU is not square, i.e., 2×min⁡(W,H). For example, if the current chroma CU is of size 2N×N, the MDLM above template is downsampled by a factor two such that only 2N samples are used in MDLM if LM_A mode is selected, as illustrated in Figure 2. Downsampling of the MDLM template is achieved by resampling the template as in CCLM. This may be beneficial for hardware implementation, as the latency can be reduced and the design can be harmonized between MDLM and CCLM.

Figure 2. Locations of the samples used in downsampled templates. The hatched samples are skipped.

# Simulation results

The software is implemented based on VTM-3.0. The encoding was done under SDR video CTC [2].

Table 1. BD-rate for all intra configuration.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.04% | 0.03% | 100% | 99% |
| Class B | 0.01% | 0.00% | -0.09% | 100% | 101% |
| Class C | -0.01% | -0.14% | -0.14% | 100% | 100% |
| Class E | 0.02% | -0.06% | -0.01% | 100% | 100% |
| Overall | 0.00% | -0.04% | -0.06% | 100% | 100% |
| Class D | -0.02% | 0.06% | 0.01% | 100% | 99% |
| Class F | 0.01% | 0.05% | -0.06% | 100% | 100% |

Table 2. BD-rate for random access configuration.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.03% | 100% | 99% |
| Class A2 | -0.01% | 0.04% | 0.08% | 100% | 100% |
| Class B | -0.01% | 0.16% | -0.06% | 100% | 100% |
| Class C | 0.03% | -0.04% | 0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Class D | -0.01% | -0.18% | 0.03% | 100% | 100% |
| Class F | -0.02% | -0.09% | -0.02% | 100% | 100% |

Table 3. BD-rate for low delay B configuration.

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.27% | -0.22% | 101% | 99% |
| Class C | 0.06% | 0.23% | 0.07% | 100% | 99% |
| Class E | -0.04% | 0.43% | -0.73% | 100% | 100% |
| Overall | 0.01% | 0.07% | -0.25% | 100% | 100% |
| Class D | -0.05% | 0.15% | 0.01% | 100% | 100% |
| Class F | 0.11% | 0.13% | 0.01% | 100% | 100% |

Overall results for all intra configuration show a slight gain of -0.04% and -0.06% for the U and V components, respectively. Decoding time is similar to that of VM-3.0.

# Conclusion

In the current MDLM design, if the chroma CU is not square, the number of samples used in linear model parameters estimation may be increased in MDLM compared to CCLM. In this contribution, it is proposed to downsample the MDLM templates to use the same number of samples to calculate the linear model parameters in MDLM as in CCLM when the chroma CU is not square. This may be beneficial for hardware implementation, as the latency can be reduced and the design can be harmonized between MDLM and CCLM. Overall results for all intra configuration show a slight gain of -0.04% and -0.06% for the U and V components, respectively.

# References

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, JVET-L1002, Oct. 2018, Macao, CN.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).