| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0152 |
| --- | --- |

| Title: | AHG17: On random access point for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Tel: Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following NAL unit types, syntax elements and restrictions on intra random access point (IRAP) picture:

Proposal 1: Three NAL unit types are assigned to indicate three IRAP picture types (IDR_W_LP, IDR_N_LP, CRA_W_LP).

- BLA_W_LP, BLA_W_RADL, BLA_N_LP are not defined.

- CRA_W_LP shall have at least one leading picture.

Proposal 2: A flag (cvs_start_flag) is present in NAL unit header.

- The flag equal to 1 indicates the start of a coded video sequence.

Proposal 3: CRA is handled as BLA, when one of the following conditions is satisfied.

- HandleCraAsBlaFlag is set equal to1 by external means.

- A CRA picture is preceded by an EOS NAL unit.

- (if proposal 2 is agreed) The value of the flag (cvs_start_flag) is equal to 1.

Proposal 4: CRA_NUT shall have at least one leading picture, when the CRA picture is not handled as a BLA picture.

Proposal 5: CRA handled as BLA shall not have any leading picture in a bitstream.

# Introduction

In JVET-L0064 discussion, it was agreed that a functionality to support an open GOP structure and that some basic picture type information indicating open/closed prediction structuring should be in the NAL unit header, not in an SEI message. Through several meetings, experts inclined to agree to have the reduced number of NAL unit type for VVC. In JVET-M0101, multiple companies propose some discussion points on random access point and leading picture indication.

In this contribution, agreeing the suggestions in JVET-M0101 in principle, technical details on random access point are proposed. The proposals regarding leading picture types are described in JVET-M0153.

# Discussion on Proposal 1

Proposal 1: Three NAL unit types are assigned to indicate three IRAP picture types (IDR_W_LP, IDR_N_LP, CRA_NUT).

- BLA_W_LP, BLA_W_RADL, BLA_N_LP are not defined.

Discussion: It is a good design to have three NAL unit types to independently indicate three IRAP picture types (IDR_W_LP, IDR_N_LP and CRA). The indication is matched to SAP types specified in ISOBMFF. A BLA pictures can be identified by the options suggested by Proposal 3.

# Discussion on Proposal 2

Proposal 2: A flag (cvs_start_flag) is present in a NAL unit header. The flag can explicitly indicate whether a new coded video sequence start from the current NAL unit and the associated CRA picture has a broken link.

Discussion: Signaling the flag in a very high level syntax structure (e.g. NUH) would be helpful to expose where the current coded video sequence (CVS) starts. If the current CVS starts with a CRA picture, then the first NAL unit of the current CVS is not explicitly identified by parsing a NAL unit type only. Sometimes, a preceding NAL unit with EOS NAL unit type can be lost. Note that JVET-M0520 proposes a flexible extension mechanism of NUH.

If the flag is equal to 1 with a CRA picture, the CRA picture shall have a broken link. Then, the CRA picture shall be handled as a BLA picture.

# Discussion on Proposal 3

Proposal 3: CRA is handled as BLA, when one of the following conditions is satisfied.

- HandleCraAsBlaFlag is set equal to1 by external means.

- A CRA picture is preceded by an EOS NAL unit.

- (if proposal 2 is agreed) The value of the flag (cvs_start_flag) is equal to 1.

Discussion: The proposed conditions are the same as in HEVC, except for the condition 3. Instead of burning an expensive NAL unit type for BLA, the presence of a broken link or the indication of a CVS start can be identified by other ways.

# Discussion on Proposal 4

Proposal 4: CRA_NUT shall have at least one leading picture, when the CRA picture is not handled as a BLA picture. (Hence,

Discussion: A picture with NAL unit type equal to CRA_NUT shall have at least one leading picture, when the CRA picture is not handled as a BLA picture. This is because a CRA picture is needed to support an open GOP structure, which allows a prediction across a GOP boundary. Without any leading picture, the GOP structure is always closed and there is no need to have a CRA.

# Discussion on Proposal 5

Proposal 5: CRA handled as BLA do not have any leading picture in a bitstream.

Discussion: A CRA picture is handled as a BLA picture, if a system spliced the bitstream and one of the suggested conditions in Proposal 3 was satisfied. Then, a splicer should discard unnecessary (non-decodable) leading pictures, so that the spliced bitstream does have any leading picture. At decoder side, if a CRA picture is handled as a BLA picture, the associated RASL pictures are needed to be discarded in system level, before the bitstream is fed into a decoder. This restriction simplifies the decoder’s behavior by removing unavailable picture generation process.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).