| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0154 |
| --- | --- |

| Title: | AHG17: On decoded picture buffer management for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Tel: Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an explicit signaling of “unused reference pictures” for management of a decoded picture buffer. The proposed syntax elements in a tile group header indicate which reference pictures shall be removed from a decoded picture buffer before decoding the current picture (or tile group). This proposal is intended to separate the syntax elements and the decoding process for management of a decoded picture buffer from the syntax elements and the decoding process for constructing reference picture lists. Based on the proposed method, the overall syntax and the decoding process for reference picture list construction and decoded picture buffer management can be simplified. Especially, when the proposed method is jointly used with a direct reference picture list indication, a problem on the picture removal timing can be resolved.

# Problem statement

In HEVC, the reference picture set (RPS) deﬁnes which reference pictures are stored in a decoded picture buffer (DPB) and which reference pictures are used for decoding the current picture or the subsequent pictures. By parsing and interpreting the RPS syntax elements, the reference picture lists to be used for decoding the current picture can be constructed. In this case, the reference pictures stored in DPB are explicitly determined from the RPS syntax elements. However, it is complicated to decode the RPS syntax elements to achieve the current reference picture lists and the decoded picture lists to be removed for a DPB.

Alternatively, reference picture lists, list 0 and list 1, can be directly signalled and derived, as proposed in [1]. The decoding process of the syntax elements for signaling direct reference picture lists may be relatively simple. However, the decoded pictures to be stored in DPB cannot be directly determined from those syntax elements. The direct RPL signaling cannot identify decoded pictures to be removed from DPB, because some decoded picture may not be used for decoding the current picture, but used for decoding the subsequent pictures. In [1], in order to distinguish the reference pictures to be used for a future reference from the currently used reference pictures, the concept of active and inactive entries is employed. However, this contribution argues that there is still an ambiguity to identify which reference pictures shall be removed from a DPB, or not.

# Proposal

Reference pictures, which are not used for decoding the current picture or the subsequent pictures in decoding order, are explicitly indicated in a tile group header for management of a decoded picture buffer. A syntax element (unused_ref_pic_signaling_enabled_flag) is signaled in SPS, which identifies whether the method for the explicit indication of removal pictures is used. The proposed syntax elements (num_unused_ref_pic, delta_poc_unused_ref_pic [ i ], unused_ref_pic_sign_flag [ i ]) in a tile group header indicate which decoded pictures are removed from a decoded picture buffer before decoding the current picture (or tile group). The decoded pictures indicated by the above syntax elements are marked as “unused for reference”. After a decoded picture in a DPB is bumped out, the decoded picture is marked as “not needed for output”. If a decoded picture in a DPB is marked as “unused for reference” and also marked as “not needed for output”, the decoded picture is removed from a DPB, after parsing the current tile group header and before decoding the current picture (or tile group).

It is noticeable that an entry (unused for reference) may be indicated per picture, on average, because the picture decoding rate could be identical to the picture removal rate. This is more bit-efficient than indicating all reference pictures to be used for reference, per picture.

# Proposed syntax elements

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| unused_ref_pic_signaling_enabled_flag | u(1) |
| … |  |
| } |  |

unused_ref_pic_signaling_enabled_flag equal to 0 specifies that num_unused_ref_pic, delta_poc_unused_ref_pic[ i ] and sign_unused_ref_pic[ i ] are not present in slice header (or tile/tile group header), and the removal timing of a decoded picture from the DPB is implicitly determined. unused_ref_pic_signaling_enabled_flag equal to 1 specifies that num_unused_ref_pic, delta_poc_unused_ref_pic[ i ] and sign_unused_ref_pic[ i ] are present in slice header (or tile/tile group header), and the removal timing of a decoded picture from the DPB is explicitly determined by parsing delta_poc_unused_ref_pic[ i ] and sign_unused_ref_pic[ i ].

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(unused_ref_pic_signaling_enabled_flag) { |  |
| num_unused_ref_pic | ue(v) |
| for( i = 0; i < num_unused_ref_pic; i++) { |  |
| delta_poc_unused_ref_pic [ i ] | ue(v) |
| unused_ref_pic_sign_flag [ i ] | u(1) |
| } |  |
| … |  |
| } |  |

num_unused_ref_pic specifies the number of unused reference picture entries.

delta_poc_unused_ref_pic [ i ] specifies the absolute difference between the picture order count values of the current picture and the unused decoded picture referred to by the i-th entry. The value of delta_poc_unused_ref_pic [ i ] shall be in the range of 0 to 215 − 1, inclusive.

unused_ref_pic_sign_flag [ i ] equal to 1 specifies that the picture order count value of the i-th entry is greater than the value of the picture order count of the current picture. unused_ref_pic_sign_flag [ i ] equal to 0 specifies that the picture order count value of the i-th entry is smaller than the value of the picture order count of the current picture.

# Discussion on the case that the proposed syntax elements are used with a direct RPL indication

The direct RPL signaling indicates which decoded pictures in a DPB shall be included in reference picture lists to decode the current picture. From the direct RPL signaling, the decoded pictures to be stored in a DPB or to be removed form a DPB cannot be explicitly/directly identified. This is because some decoded pictures may not be used for decoding the current picture, but may be used for decoding the subsequent pictures in decoding order. Those pictures shall be stored in a DPB. However, if a decoded picture is not used for decoding a picture and already an output, the decoded picture needs to be removed from a DPB, immediately. In [1], the direct RPL indication distinguishes inactive reference pictures from active reference pictures. However, the number of active or inactive reference pictures can be changed depending on the number of actual available reference pictures. Still, an additional identification is needed to sort out the pictures to be stored in a DPB from the pictures to be removed from a DPB. By explicitly managing the status of a decoded picture buffer with a separate set of syntax elements from the RPL indication, the proposed method can eliminate the ambiguity of the picture removal timing.

# Discussion on the case that the proposed syntax elements are used with an RPS indication

If the status of a DPB is managed by the syntax elements for marking unused reference pictures, the RPS needs not to indicate whether a reference picture is used for decoding the current picture or any following picture in decoding order. The RPS as in HEVC explicitly indicate the reference picture sets used for decoding the current picture and the reference picture sets used for decoding the subsequent pictures in decoding order. Then, the decoded pictures, which are not used for reference, are derived from a decoding processing of the RPS syntax elements. Since the RPS contained multiple information simultaneously and some information were derived from its decoding process, the RPS needed huge specification text and complicated process. If the DPB status is explicitly identified from the proposed syntax elements, the RPS needs to signal only the reference picture set to be used for decoding the current picture. Then, the entire RPS decoding process is simplified and also the syntax elements can be digested, as in the specification text attached in this contribution. In the RPS syntax set, for example, used_by_cur_pic_s0_flag[ i ] and used_by_cur_pic_s1_flag[ i ] can be removed, because delta_poc_s0_minus1[ i ] and delta_poc_s1_minus1[ i ] indicate the reference picture set used for decoding the current picture, only. It is  noticeable that the proposed syntax elements indicate an entry (unused for reference) per picture on average, while the RPS as in HEVC needs to signal multiple entries (used for reference) per picture (worst case: 16 entries).

| st_ref_pic_set(stRpsIdx ) { | Descriptor |
| --- | --- |
| for( i = 0; i < num_l0_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_l1_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |

delta_poc_s0_minus1[ i ] plus 1, when i is equal to 0, specifies the difference between the picture order count values of the current picture and i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value less than that of the current picture, or, when i is greater than 0, specifies the difference between the picture order count values of the ( i − 1 )-th entry and the i-th entry in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture. The value of delta_poc_s0_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_s0_flag[ i ] equal to 0 specifies that the i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value less than that of the current picture is not used for reference by the current picture.

delta_poc_s1_minus1[ i ] plus 1, when i is equal to 0, specifies the difference between the picture order count values of the current picture and the i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value greater than that of the current picture, or, when i is greater than 0, specifies the difference between the picture order count values of the i-th entry and the ( i − 1 )-th entry in the current candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture. The value of delta_poc_s1_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_s1_flag[ i ] equal to 0 specifies that the i-th entry in the current candidate short-term RPS that has picture order count value greater than that of the current picture is not used for reference by the current picture.

# References

- Y.-K. Wang, et al, “On reference picture management for VVC,” JVET-M0128, Marrakech, Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).