| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0168-v1 |
| --- | --- |

| Title: | CE2-related: Simplifications for inherited affine candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes simplifications for inherited affine candidates at coding tree unit (CTU) row boundaries and inside the current CTU row. Compared with the inherited affine candidates in VTM3.0, it is asserted that the CU width information line buffer and the CPMV buffer can be removed. At CTU row boundaries, it is proposed to use two or three subblock motion vectors (MVs) with a fixed distance for affine candidate inheritance, where an accurate distance is used to derive the affine motion and coding unit (CU) width information is not needed anymore for saving the CU width information line buffer. Inside the current CTU row (i.e., not at CTU row boundaries), it is proposed to use two or three subblock MVs with half CU height or half CU width for affine candidate inheritance, where control point motion vectors (CPMVs) are not needed anymore for saving the CPMV buffer. Results are summarized as follows.

Proposed inherited affine candidate derivation at CTU row boundaries by using two subblock MVs with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=101%}
VTM3.0-LB: {Y-BD-rate=0.04%, EncT=100%, DecT=100%}

Proposed inherited affine candidate derivation at CTU row boundaries by using two subblock MVs when the MVs are bi-prediction and using three subblock MVs when the MVs are uni-prediction with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=-0.02%, EncT=100%, DecT=101%}
VTM3.0-LB: {Y-BD-rate=0.01%, EncT=100%, DecT=100%}

Proposed inherited affine candidate derivation inside the current CTU row by using subblock MVs with half CU width or half CU height and inherited affine candidate derivation at CTU row boundaries by using two subblock MVs with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM3.0-LB: {Y-BD-rate=0.02%, EncT=101%, DecT=100%}

Proposed Inherited affine candidate derivation inside the current CTU row by using subblock MVs with half CU width or half CU height and inherited affine candidate derivation at CTU row boundaries by using two subblock MVs when the MVs are bi-prediction and using three subblock MVs when the MVs are uni-prediction with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=-0.02%, EncT=100%, DecT=100%}
VTM3.0-LB: {Y-BD-rate=0.02%, EncT=101%, DecT=101%}

# Introduction

During JVET-L meeting, the line buffer reduction methods for inherited affine candidates at CTU row boundaries presented in JVET-L0045 [1] and JVET-L0694 [2] were adopted. In JVET-L0045, it is proposed to use the 4-parameter affine motion model with the bottom-left and bottom-right CPMVs for affine motion inheritance from a neighbouring CU located at a CTU row boundary, regardless whether the neighbouring CU uses the 4- or 6-parameter affine motion model. In JVET-L0694, the bottom-left and bottom-right subblock MVs are used for the affine motion inheritance if the candidate block is from the upper CTU row. No line buffer is required for CPMVs. For 4K video for example, both JVET-L0045 and JVET-L0694 reduce the line buffer requirement for the affine mode from 9,216 bytes to 256 bytes.

The separation of storages of subblock MVs and CPMVs presented in JVET-L0047 [3] was adopted. In JVET-L0047, it is proposed to use the subblock MV field for the motion compensation, merging candidate list derivation, and storage of the temporal motion vector predictors (TMVPs), and to use the CPMVs only for the affine motion data inheritance.

There are still three shortcomings. First, additional 256-byte line buffer is needed for storing CU width information. Second, the distance between bottom-left and bottom-right subblock MVs for affine inheritance is not accurate, and the accurate distance should be the width of the CU minus 4. Third, additional CPMVs buffer is needed for storing CPMVs information. This contribution proposes to use two or three subblock MVs with a fixed distance, or half height and half width for affine inheritance to deal with these three problems.

# Proposed methods

## Two subblock MVs with a fixed distance for affine inheritance at CTU row boundaries

Two subblock MVs with a fixed distance are used for the affine motion inheritance if the candidate block is from the upper CTU row. As shown in Figure 1, B0 and B0’, B1 and B1’, and B2 and B2’ are used for deriving 4-parameter inherited affine candidates, and the distance between these two blocks is N. The CU width line buffer is not needed because the distance between two subblocks is fixed. To check the availability, in addition to checking whether B0 (or B1, B2) is affine coded, it is necessary to check whether B0’ (or B1’, B2’) is affine coded and has same reference index as B0 (or B1, B2) in both reference picture lists.

Figure 1. Inherited affine candidate derivation at CTU row boundaries

## Three subblock MVs with a fixed distance for affine inheritance at CTU row boundaries

Three subblock MVs with a fixed distance are used for the affine motion inheritance if the candidate block is from the upper CTU row and the subblock MVs are uni-prediction. As shown in Figure 2, B0, B0’, and B0’’, B1, B1’, and B1’’, and B2, B2’, and B2’’ are used for deriving 6-parameter inherited affine candidates, and the distance between blocks is N as denoted in Figure 2. The CU width or CU height line buffer are not needed because the distance between blocks is fixed. The availability check is the same as the method of two subblock MVs. The MV at B0’’ (or B1’’, B2’’) is directly used if the B0’’ (or B1’’, B2’’) and B0 (or B1’, B2’) are in the same CU, otherwise, MV at B0 (or B1’, B2’) is used instead.

Figure 2. Inherited affine candidate derivation at CTU row boundaries

## Subblock MVs with half CU height or half CU width for affine inheritance inside the current CTU row

Two or three subblock MVs with half CU height or half CU width are used for the affine motion inheritance if the candidate block is inside the current CTU row. As shown in Figure 3, “LT, RT, and LB” to the left of the current CU and “LT and RT” to the top of the current CU are used for deriving 6-parameter or 4-parameter inherited affine candidates, respectively, and the distance between LT and RT is half candidate CU width and the distance between LT and LB is half candidate CU height. The CPMV buffer is not needed because the affine inheritance derivation is from subblock MVs. Since these two or three subblock MVs belong to the same CU, additional availability check is not needed.

Figure 3. Inherited affine candidate derivation inside the current CTU row

# Experiment results

The proposed inherited affine candidate derivation have been implemented on top of VTM3.0. The common test conditions [4] are used for evaluation. Table 1 summarizes the results of the proposed method in Section 2.1. Table 2 summarizes the results of the proposed methods in Section 2.1 and Section 2.2. Table 3 summarizes the results of the proposed methods in Section 2.1 and Section 2.3. Table 4 summarizes the results of the proposed methods in Section 2.1, Section 2.2, and Section 2.3.

Table 1. Inherited affine candidate derivation at CTU row boundaries by using two subblock MVs with a fixed distance equal to 16

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.01% | 101% | 101% |
| Class A2 | -0.01% | 0.05% | -0.01% | 100% | 99% |
| Class B | 0.00% | 0.02% | -0.02% | 100% | 101% |
| Class C | 0.01% | 0.04% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.03% | 100% | 101% |
| Class D | -0.05% | -0.05% | 0.03% | 100% | 104% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.14% | 0.13% | 100% | 100% |
| Class C | 0.03% | 0.21% | 0.16% | 101% | 100% |
| Class E | 0.04% | 0.00% | -0.13% | 100% | 100% |
| Overall | 0.04% | 0.01% | 0.07% | 100% | 100% |
| Class D | 0.02% | -0.04% | 0.58% | 101% | 102% |
| Class F (optional) | -0.01% | 0.23% | 0.35% | 100% | 100% |

Table 2. Inherited affine candidate derivation at CTU row boundaries by using two subblock MVs when the MVs are bi-prediction and using three subblock MVs when the MVs are uni-prediction with a fixed distance equal to 16

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.04% | 100% | 98% |
| Class A2 | -0.06% | -0.05% | -0.03% | 100% | 99% |
| Class B | -0.03% | -0.02% | -0.10% | 100% | 105% |
| Class C | 0.01% | 0.02% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.06% | 100% | 101% |
| Class D | -0.05% | -0.05% | 0.03% | 100% | 99% |
| Class F (optional) | -0.02% | -0.02% | -0.03% | 100% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | 0.08% | 101% | 101% |
| Class C | 0.03% | 0.34% | -0.17% | 100% | 100% |
| Class E | -0.02% | 0.20% | -0.20% | 101% | 100% |
| Overall | 0.01% | 0.12% | -0.07% | 100% | 100% |
| Class D | 0.01% | 0.27% | 0.34% | 100% | 101% |
| Class F (optional) | -0.07% | -0.03% | 0.32% | 101% | 100% |

Table 3. Inherited affine candidate derivation at CTU row boundaries by using two subblock MVs with a fixed distance equal to 16; inherited affine candidate derivation inside the current CTU row by using subblock MVs with half CU width or half CU height

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Class A2 | -0.01% | 0.07% | 0.04% | 100% | 98% |
| Class B | -0.02% | 0.06% | -0.06% | 100% | 100% |
| Class C | 0.04% | 0.10% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.01% | 100% | 100% |
| Class D | 0.04% | 0.11% | -0.08% | 101% | 107% |
| Class F (optional) | -0.01% | 0.00% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.29% | -0.07% | 100% | 101% |
| Class C | 0.07% | 0.23% | 0.10% | 101% | 100% |
| Class E | -0.07% | 0.09% | -0.51% | 101% | 100% |
| Overall | 0.02% | -0.02% | -0.12% | 101% | 100% |
| Class D | 0.16% | 0.20% | -0.17% | 100% | 100% |
| Class F (optional) | -0.07% | -0.03% | 0.55% | 100% | 103% |

Table 4. Inherited affine candidate derivation at CTU row boundaries by using two subblock MVs when the MVs are bi-prediction and using three subblock MVs when the MVs are uni-prediction with a fixed distance equal to 16; inherited affine candidate derivation inside the current CTU row by using subblock MVs with half CU width or half CU height

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.16% | 0.02% | 101% | 102% |
| Class A2 | -0.06% | -0.08% | 0.09% | 100% | 99% |
| Class B | -0.04% | 0.16% | -0.06% | 100% | 99% |
| Class C | 0.04% | 0.09% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.03% | 0.03% | 100% | 100% |
| Class D | 0.03% | 0.09% | -0.05% | 100% | 101% |
| Class F (optional) | -0.03% | -0.03% | -0.02% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.22% | 0.09% | 100% | 101% |
| Class C | 0.03% | 0.30% | 0.08% | 100% | 100% |
| Class E | 0.02% | 0.45% | -0.02% | 101% | 101% |
| Overall | 0.02% | 0.12% | 0.06% | 101% | 101% |
| Class D | 0.15% | 0.20% | 0.28% | 100% | 100% |
| Class F (optional) | -0.08% | 0.22% | -0.31% | 101% | 100% |

# Conclusion

In this contribution, methods are proposed to simplify inherited affine candidates effectively. The coding efficiency impact is minor. Compared with the inherited affine candidates in VTM3.0, the CU width information line buffer and the CPMV buffer can be removed.

# References

[1] M. Zhou, “CE4: Test results of CE4.1.11 on line buffer reduction for affine mode,” Document of Joint Video Experts Team, JVET-L0045.

[2] H. Chen, H. Yang, J. Chen, H. Huang, W.-J. Chien, “CE4-related: Combination of affine mode clean up and line buffer reduction,” Document of Joint Video Experts Team, JVET-L0694.

[3] M. Zhou, B. Heng “CE4-related: A clean up for affine mode,” Document of Joint Video Experts Team, JVET-L0047.

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).