| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0172-v1 |
| --- | --- |

| Title: | CE5.1.9: CABAC engine with simplified range sub-interval derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In JEM CABAC engine, the rangeLPS is derived by using a 512×64×9-bit look-up table, where the size of the rangeLPS look-up table is 144 times of that in HEVC. In CE5.1.9, two alternative rangeLPS derivation methods are proposed. Each of the two methods can be implemented with a look-up table or a multiplier. For both methods, the size of the table for deriving the rangeLPS value is the same as that in HEVC, i.e., 32×8×8 bits, but the tables of the two methods are different. In Test-A, the table look-up values are calculated by using a 6-bit by 5-bit multiplier. In Test-B, the table look-up values are calculated by using a 6-bit by 4-bit multiplier. Both Test-A and Test-B are implemented on top of a double-window probability estimation model with linear state representation. Simulation results reportedly show that the average luma BD-rates are -0.79%/-0.57%/-0.50% in AI/RA/LB cases in Test-A and -0.76%/-0.55%/-0.49% in AI/RA/LB cases in Test-B.

# Introduction

In JEM CABAC engine, the range is divided into rangeLPS and rangeMPS for coding each bin. The range sub-interval derivation is realized by using a 512×64×9-bit look-up table (LUT). The table covers probability range of (0, 1). Compared with the 64×4×8-bit rangeLPS table in HEVC, the JEM CABAC LUT size is 144 times. To reduce the table size, two methods are proposed in this contribution. In both CE5.1.9 Test-A and CE5.1.9 Test-B, a 32×8×8-bit table is used for deriving the rangeLPS value. The values in the look-up table can be computed by a 6-bit by 5-bit multiplier in CE5.1.9 Test-A or by a 6-bit by 4-bit multiplier in CE5.1.9 Test-B.

# Proposed methods

## Double-window probability estimation model with linear state representation

The proposed simplified range sub-interval derivation is implemented on a double-window probability estimation model with linear state representation. For each context model, two probability states (P0 and P1) are used, which stored in 10 and 14 bits buffers, respectively. The window shift parameters of P0 and P1 are two fixed numbers, which are 4 and 7, respectively. The probability state update is as follows.

P0 –= (P0 >> 4)

P1 –= (P1>> 7)

if(bin == 1){

P0 += 1023

P1 += 16383

}

P = (P0 <<4) + P1

## Test-A: RangeLPS derivation by using a 6-bit by 5-bit multiplier

The interval subdivision is realized using a 6-bit by 5-bit multiplier or equivalently a 32x8x8 table. The rangeLPS derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = ((2 × probIdx +1) × (2 × ( range >> 5 ) + 1)) >> 3

or

rangeLPS  = LUT_32x8[ probIdx ][ ( range >> 5 ) & 7 ]

where the P is the probability of a context variable and the range is the current range in binary arithmetic coder (BAC). The supplementary results of using a 5-bit by 5-bit multiplier or equivalently a 16x8x8 table is also provided. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 31 – (P>>10) : (P>>10)

rangeLPS  = ((2 × probIdx +1) × (2 × ( range >> 5 ) + 1)) >> 2

or

rangeLPS  = LUT_16x8[ probIdx ][ ( range >> 5 ) & 7 ]

## Test-B: RangeLPS derivation by using a 6-bit by 4-bit multiplier

The interval subdivision is realized using a 6-bit by 4-bit multiplier or equivalently a 32x8x8 table. The rangeLPS derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = ((2 × probIdx +1) × ( range >> 5 )) >> 3

or

rangeLPS  = LUT_32x8[ probIdx ][ ( range >> 5 ) & 7 ]

where the P is the probability of a context variable and the range is the current range in BAC. Compared with the CE5.2.2.a, in CE5.2.2.b, the ( range >> 5 ) is used instead of (2 × ( range >> 5 ) + 1)). The supplementary results of using a 5-bit by 4-bit multiplier or equivalently a 16x8x8 table is also provided. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 31 – (P>>10) : (P>>10)

rangeLPS  = ((2 × probIdx +1) × ( range >> 5 )) >> 2

or

rangeLPS  = LUT_16x8[ probIdx ][ ( range >> 5 ) & 7 ]

In Test-B, when the range is 256, the maximum rangeLPS is 124, which guarantees that during renormalization after processing an MPS, the range register is left shifted by at most one bit position.

# Simulation results

The proposed methods are implemented on VTM3.0. The common test conditions [1] are used for evaluation. The Test-A results (double-window probability estimation model with linear state representation and 6-bit by 5-bit multiplier for rangeLPS derivation) are shown in Table 1. The Test-B results (double-window probability estimation model with linear state representation and 6-bit by 4-bit multiplier for rangeLPS derivation) are shown in Table 2. The supplementary results of using a 5-bit by 5-bit multiplier in Test-A are shown in Table 3. The supplementary results of using a 5-bit by 4-bit multiplier in Test-B are shown in Table 4.

Table 1.  Results of Test-A: Double-window probability estimation model with linear state representation + rangeLPS derivation by using a 6-bit by 5-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -1.28% | -1.13% | 107% | 101% |
| Class A2 | -0.85% | -1.29% | -1.30% | 106% | 103% |
| Class B | -0.77% | -0.91% | -1.11% | 105% | 101% |
| Class C | -0.79% | -0.87% | -1.19% | 103% | 101% |
| Class E | -0.76% | -1.02% | -1.19% | 105% | 101% |
| Overall | -0.79% | -1.05% | -1.18% | 105% | 101% |
| Class D | -0.72% | -0.84% | -0.89% | 103% | 100% |
| Class F | -0.69% | -0.69% | -0.87% | 105% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -1.22% | -1.26% | 103% | 98% |
| Class A2 | -0.65% | -0.94% | -0.89% | 102% | 100% |
| Class B | -0.60% | -1.02% | -1.20% | 102% | 101% |
| Class C | -0.51% | -1.06% | -0.64% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -1.05% | -1.00% | 102% | 100% |
| Class D | -0.45% | -1.01% | -0.68% | 101% | 100% |
| Class F | -0.35% | -0.58% | -0.58% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.95% | -0.54% | 102% | 103% |
| Class C | -0.44% | -0.70% | -0.56% | 103% | 99% |
| Class E | -0.54% | -0.77% | -1.00% | 102% | 101% |
| Overall | -0.50% | -0.82% | -0.66% | 103% | 101% |
| Class D | -0.38% | -0.55% | -0.20% | 101% | 100% |
| Class F | -0.16% | -0.19% | 0.19% | 103% | 100% |

Table 2.  Results of Test-B: Double-window probability estimation model with linear state representation + rangeLPS derivation by using a 6-bit by 4-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -1.25% | -1.10% | 107% | 101% |
| Class A2 | -0.81% | -1.26% | -1.27% | 106% | 101% |
| Class B | -0.75% | -0.89% | -1.09% | 105% | 102% |
| Class C | -0.75% | -0.83% | -1.15% | 103% | 102% |
| Class E | -0.73% | -0.99% | -1.16% | 105% | 101% |
| Overall | -0.76% | -1.02% | -1.15% | 105% | 102% |
| Class D | -0.70% | -0.82% | -0.87% | 104% | 101% |
| Class F | -0.64% | -0.65% | -0.83% | 105% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -1.20% | -1.24% | 104% | 99% |
| Class A2 | -0.62% | -0.91% | -0.86% | 102% | 100% |
| Class B | -0.59% | -1.00% | -1.18% | 102% | 101% |
| Class C | -0.48% | -1.02% | -0.60% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.55% | -1.03% | -0.98% | 102% | 100% |
| Class D | -0.42% | -0.99% | -0.65% | 102% | 100% |
| Class F | -0.32% | -0.54% | -0.55% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -0.94% | -0.53% | 102% | 100% |
| Class C | -0.41% | -0.67% | -0.53% | 102% | 100% |
| Class E | -0.55% | -0.77% | -1.01% | 102% | 101% |
| Overall | -0.49% | -0.81% | -0.65% | 102% | 101% |
| Class D | -0.37% | -0.54% | -0.19% | 101% | 101% |
| Class F | -0.14% | -0.17% | 0.21% | 102% | 100% |

Table 3.  Supplementary results of Test-A: Double-window probability estimation model with linear state representation + rangeLPS derivation by using a 5-bit by 5-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.73% | -1.20% | -1.05% | 107% | 100% |
| Class A2 | -0.76% | -1.22% | -1.22% | 106% | 104% |
| Class B | -0.71% | -0.85% | -1.05% | 105% | 101% |
| Class C | -0.73% | -0.81% | -1.13% | 104% | 101% |
| Class E | -0.70% | -0.96% | -1.13% | 105% | 101% |
| Overall | -0.72% | -0.98% | -1.11% | 105% | 101% |
| Class D | -0.68% | -0.79% | -0.84% | 103% | 101% |
| Class F | -0.64% | -0.64% | -0.82% | 106% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -1.15% | -1.19% | 103% | 97% |
| Class A2 | -0.57% | -0.87% | -0.82% | 103% | 97% |
| Class B | -0.54% | -0.96% | -1.14% | 102% | 100% |
| Class C | -0.47% | -1.01% | -0.59% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.51% | -0.99% | -0.94% | 102% | 99% |
| Class D | -0.41% | -0.97% | -0.64% | 102% | 100% |
| Class F | -0.31% | -0.53% | -0.54% | 103% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.45% | -0.88% | -0.47% | 102% | 101% |
| Class C | -0.40% | -0.65% | -0.51% | 102% | 101% |
| Class E | -0.51% | -0.74% | -0.97% | 102% | 102% |
| Overall | -0.45% | -0.77% | -0.61% | 102% | 101% |
| Class D | -0.35% | -0.52% | -0.17% | 101% | 101% |
| Class F | -0.11% | -0.14% | 0.24% | 103% | 100% |

Table 4.  Supplementary results of Test-B: Double-window probability estimation model with linear state representation + rangeLPS derivation by using a 5-bit by 4-bit multiplier

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -1.18% | -1.03% | 107% | 100% |
| Class A2 | -0.73% | -1.19% | -1.19% | 106% | 101% |
| Class B | -0.68% | -0.82% | -1.02% | 105% | 102% |
| Class C | -0.70% | -0.78% | -1.10% | 104% | 101% |
| Class E | -0.68% | -0.94% | -1.11% | 105% | 101% |
| Overall | -0.70% | -0.95% | -1.08% | 105% | 101% |
| Class D | -0.66% | -0.78% | -0.83% | 104% | 101% |
| Class F | -0.60% | -0.61% | -0.79% | 104% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -1.13% | -1.17% | 103% | 98% |
| Class A2 | -0.55% | -0.84% | -0.79% | 103% | 97% |
| Class B | -0.53% | -0.94% | -1.12% | 102% | 100% |
| Class C | -0.43% | -0.98% | -0.56% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.49% | -0.97% | -0.92% | 102% | 99% |
| Class D | -0.39% | -0.95% | -0.62% | 101% | 100% |
| Class F | -0.28% | -0.50% | -0.50% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | -0.86% | -0.46% | 103% | 101% |
| Class C | -0.36% | -0.63% | -0.48% | 102% | 101% |
| Class E | -0.51% | -0.74% | -0.97% | 102% | 101% |
| Overall | -0.43% | -0.75% | -0.59% | 102% | 101% |
| Class D | -0.34% | -0.51% | -0.16% | 101% | 102% |
| Class F | -0.09% | -0.12% | 0.26% | 103% | 105% |

# Conclusions

In this contribution, two alternative range subinterval derivation methods are proposed and implemented on a double-window probability estimation model with linear state representation. For each of the two methods, a 32×8×8-bit table is used for deriving the rangeLPS value. The two methods use different tables, but the table sizes of the two methods are the same as that in HEVC. In Test-A, the table look-up values are calculated by using a 6-bit by 5-bit multiplier. In Test-B, the table look-up values are calculated by using a 6-bit by 4-bit multiplier. Implementation-wise a look-up table or a multiplier can be chosen. Simulation results reportedly show that, the average luma BD-rate are -0.79%/-0.57%/-0.50% in AI/RA/LB cases in Test-A, and the average luma BD-rate are -0.76%/-0.55%/-0.49% in AI/RA/LB cases in Test-B.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).