| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0173-v1 |
| --- | --- |

| Title: | CE7 (Tests 7.1, 7.2, 7.3, and 7.4): Constraints on context-coded bins for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Shih-Ta Hsiang, Zhi-Yi Lin, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, shih-ta.hsiang, zhi-yi.lin, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the VTM3.0 coefficient coding, the averaged maximum number of context-coded bins per 16 samples is increased by 28% compared with that in HEVC. In this contribution, four tests are evaluated. In CE7.1, it is proposed to constrain the maximum number of context-coded bins to 30 for a 4x4 luma subblock, 16 for a 4x4 chroma subblock, and 4 for a 2x2 chroma subblock. The averaged maximum number of context-coded bins per 16 samples is only increased by 1.3% compared with that in HEVC. In CE7.2, on top of CE7.1, the constrained maximum number of context-coded bins per coefficient subblock is increased according to the last significant subblock position without increasing the worst-case averaged maximum number of context-coded bins per 16 samples. In CE7.3, on top of CE7.2, to improve the parsing throughput and reduce the number of coding passes, greater than 2 flags are moved to the first coding pass after the parity bit. In CE7.4, moving the greater than 2 flags to the first coding pass after the parity bit is implemented on top of VTM3.0. Simulation results are as follows.

CE7.1:
VTM3.0-AI: {BD-rate = 0.08% (Y) / 0.64% (Cb) / 0.44% (Cr), EncT=102%, DecT=101%}
VTM3.0-RA: {BD-rate = 0.07% (Y) / 0.87% (Cb) / 0.69% (Cr), EncT=101%, DecT=100%}
VTM3.0-LB: {BD-rate = 0.09% (Y) / 1.58% (Cb) / 0.89% (Cr), EncT=101%, DecT=102%}

CE7.2:
VTM3.0-AI: {BD-rate = 0.06% (Y) / 0.19% (Cb) / 0.10% (Cr), EncT=104%, DecT=102%}
VTM3.0-RA: {BD-rate = 0.03% (Y) / 0.35% (Cb) / 0.30% (Cr), EncT=101%, DecT=101%}
VTM3.0-LB: {BD-rate = 0.00% (Y) / 0.57% (Cb) / 0.14% (Cr), EncT=102%, DecT=101%}

CE7.3:
VTM3.0-AI: {BD-rate = 0.00% (Y) / 0.09% (Cb) / 0.05% (Cr), EncT=103%, DecT=101%}
VTM3.0-RA: {BD-rate = 0.00% (Y) / 0.30% (Cb) / 0.23% (Cr), EncT=101%, DecT=102%}
VTM3.0-LB: {BD-rate = 0.02% (Y) / 0.58% (Cb) / -0.12% (Cr), EncT=101%, DecT=100%}

CE7.4:
VTM3.0-AI: {BD-rate = -0.04% (Y) / -0.09% (Cb) / -0.10% (Cr), EncT=100%, DecT=101%}
VTM3.0-RA: {BD-rate = -0.06% (Y) / 0.03% (Cb) / -0.06% (Cr), EncT=100%, DecT=100%}
VTM3.0-LB: {BD-rate = -0.04% (Y) / 0.00% (Cb) / 0.12% (Cr), EncT=100%, DecT=100%}

# Introduction

In HEVC, for a 4x4 coefficient subblock, the maximum number of context-coded bins per 16 samples is 24 or 25, including 15 or 16 significant flags depending on whether the last significant coefficient is in the subblock, eight greater than 1 (gt1) flags, and one greater than 2 (gt2) flag. In VTM3.0, for a 4x4 coefficient coding subblock, at most 32 context-coded bins can be used. For a 2x2 chroma coefficient coding subblock, at most 8 context-coded bins can be used. The averaged maximum number of context-coded bins per 16 samples in VTM3.0 is 32 bins. Compared with that in HEVC, the worst-case number of context-coded bins is increased by 28%.

In this contribution, it is proposed to use different constraints according to colour components and subblock sizes. To improve coding efficiency, the constraints can be loosened if not all subblocks in a transform block (TB) need to be coded. According to the parsed last significant coefficient position, the constraint can be loosen by increasing the allowed maximum number of context-coded bins. Besides, the gt2 flag is moved to the first coding pass after the parity bit coding. Including the gt2 flag in the first coding pass can improve the parsing throughput and reduce the number of coding passes.

# Proposed methods

## Different constraints according to colour components and subblock sizes

To reduce the averaged maximum number of context-coded bins, it is proposed to reduce the constraint values on 4x4 luma subblocks, 4x4 chroma subblocks, and 2x2 chroma subblocks to 30, 16, and 4, respectively. From the analysis, the averaged maximum number of context-coded bins per 16 samples is 25.33 bins and is only 1.3% larger than that in HEVC.

Table 1. The averaged maximum number of context-coded bins per 16 samples in HEVC, VTM3.0, and the proposed method

| Number of maximum context-coded bins | 4x4 luma subblock | 4x4 chroma subblock | 2x2 chroma subblock | Maximum number of context-coded bins in one 64-sample luma block and two 16-sample chroma blocks | Averaged maximum number of context-coded bins per 16 samples |
| --- | --- | --- | --- | --- | --- |
| HEVC | 24 or 25 | 24 or 25 | N/A | 25*4 + 25*2 = 150 | 25.00 (100%) |
| VTM3.0 | 32 | 32 | 8 | 32*4 + 8*8 = 192 | 32.00 (128%) |
| Proposed | 30 | 16 | 4 | 30*4 + 4*8 = 152 | 25.33 (101.3%) |

## Last significant subblock position dependent constraint

The constraint on the number of context-coded bins is to limit the worst-case number of context-coded bins. When not all the subblocks in a TB need to be coded, the constraint value can be loosened. According to the last significant subblock position, the number of to-be-coded subblocks in a TB can be derived. The constraint value is determined according to the colour component and the coefficient subblock size. With the number of to-be-coded subblocks (NumToBeCodedSb) and the number of total coefficient subblocks in a TB (NumTotalSb), the constraint value (ConstraintValue) is modified as follows.

if (NumToBeCodedSb * 2 <= NumTotalSb)

ConstraintValue = ConstraintValue * 2;

else if (NumToBeCodedSb * 5 <= NumTotalSb * 4)

ConstraintValue = (ConstraintValue * 5) >> 2;

## Include gt2 flag into the first coding pass

In VTM2.0.1, in the first coding pass, the significant flag is first parsed. If the significant flag is 1, a parity bit and a gt1 flag are parsed in order. In the second coding pass, the gt2 flags are parsed. The third coding pass is remaining level coding, and the forth coding pass is sign coding. The context set selection of significant flag coding depends on the parity bit of the previous parsed coefficient. After parsing the parity bit, the time of parsing gt1 flag can be used to calculate the state and prepare the context variable set of the significant flag of the next coefficient. In VTM3.0, the parity bit is moved after the gt1 flag to reduce the code word length of coefficient level equal to 1. However, there is no “hidden” time to calculate the state and prepare the context variable set of the significant flag of the next coefficient.

In this contribution, the gt2 flag is proposed to be moved to the first coding pass after the parity bit. By doing this, the time of parsing gt2 flag can be used to calculate the state and prepare the context set of the significant flag of the next coefficient. Also, the number of coding passes can be reduced from 4 to 3. The parsed gt2 flag is used to calculate the locSumAbsPass1in the context modeling of sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag. Compared with VTM3.0, the reserved number of context-coded bins for gt2 flag in the second coding pass is also merged into the number of context-coded bins for the first coding pass.

# Simulation results

The proposed methods are implemented on top of VTM3.0. Four tests are implemented as follows.

Test-7.1: Set constraint values to 30 for 4x4 luma subblock, 16 for 4x4 chroma subblock, and 4 for 2x2 chroma subblock

Test-7.2: Test-7.1+ last significant subblock position dependent constraint

Test-7.3: Test-7.2+ include gt2 flag into the first coding pass

Test-7.4: Only include gt2 flag into the first coding pass

The common test conditions [1] are used for evaluation. The results of Test-7.1, Test-7.2, Test-7.3, and Test-7.4 are shown in Table 2, 3, 4, and 5, respectively.

The common test conditions with low-complexity setting are also used. In the low-complexity setting, the dependent quantization and rate-distortion optimized quantization (RDOQ) are disabled, and the sign-bit hiding is enabled. The results of Test-7.1, Test-7.2, Test-7.3, and Test-7.4 under low-complexity setting are shown in Table 6, 7, 8, and 9, respectively.

Table 2.  Results of Test-7.1: Set constraint values to 32 for 4x4 luma subblock, 16 for 4x4 chroma subblock, and 4 for 2x2 chroma subblock

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.44% | 0.57% | 103% | 99% |
| Class A2 | 0.20% | 1.22% | 1.15% | 102% | 103% |
| Class B | 0.01% | 0.78% | 0.29% | 103% | 101% |
| Class C | 0.03% | 0.50% | 0.19% | 102% | 101% |
| Class E | 0.03% | 0.20% | 0.22% | 103% | 101% |
| Overall | 0.08% | 0.64% | 0.44% | 102% | 101% |
| Class D | 0.05% | 0.26% | 0.15% | 102% | 100% |
| Class F (optional) | 0.15% | 0.64% | 0.63% | 102% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.30% | 0.71% | 0.71% | 101% | 98% |
| Class A2 | 0.00% | 1.36% | 1.46% | 100% | 98% |
| Class B | -0.01% | 0.97% | 0.36% | 101% | 101% |
| Class C | 0.02% | 0.49% | 0.50% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.87% | 0.69% | 101% | 100% |
| Class D | 0.03% | -0.03% | 0.08% | 101% | 101% |
| Class F (optional) | 0.16% | 0.68% | 0.76% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 1.78% | 1.03% | 101% | 103% |
| Class C | 0.12% | 1.44% | 1.20% | 101% | 100% |
| Class E | 0.14% | 1.43% | 0.26% | 101% | 102% |
| Overall | 0.09% | 1.58% | 0.89% | 101% | 102% |
| Class D | 0.06% | 0.38% | 0.92% | 101% | 101% |
| Class F (optional) | 0.18% | 1.23% | 0.98% | 101% | 99% |

Table 3.  Results of Test-7.2: Test-7.1 + last significant subblock position dependent constraint

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.35% | 0.35% | 104% | 103% |
| Class A2 | 0.18% | 0.37% | 0.33% | 104% | 103% |
| Class B | 0.01% | 0.15% | -0.04% | 104% | 101% |
| Class C | 0.00% | 0.16% | 0.01% | 104% | 101% |
| Class E | 0.00% | -0.06% | -0.05% | 104% | 102% |
| Overall | 0.06% | 0.19% | 0.10% | 104% | 102% |
| Class D | 0.00% | 0.14% | 0.15% | 104% | 100% |
| Class F | 0.14% | 0.66% | 0.45% | 104% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.46% | 0.24% | 102% | 100% |
| Class A2 | 0.00% | 0.63% | 0.72% | 101% | 100% |
| Class B | 0.00% | 0.34% | 0.09% | 101% | 102% |
| Class C | 0.02% | 0.08% | 0.30% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.35% | 0.30% | 101% | 101% |
| Class D | 0.01% | -0.13% | 0.17% | 102% | 100% |
| Class F | 0.11% | 0.60% | 0.59% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.17% | -0.06% | 102% | 101% |
| Class C | 0.07% | 0.64% | 0.43% | 102% | 100% |
| Class E | -0.01% | 1.16% | 0.09% | 102% | 102% |
| Overall | 0.00% | 0.57% | 0.14% | 102% | 101% |
| Class D | 0.04% | 0.64% | 0.32% | 101% | 100% |
| Class F | 0.09% | 0.59% | 0.34% | 102% | 99% |

Table 4.  Results of Test-7.3: Test-7.2 + include gt2 flag into the first coding pass

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.20% | 0.15% | 103% | 103% |
| Class A2 | 0.11% | 0.32% | 0.25% | 103% | 103% |
| Class B | -0.05% | 0.07% | -0.04% | 103% | 101% |
| Class C | -0.08% | 0.03% | -0.09% | 103% | 100% |
| Class E | -0.04% | -0.11% | 0.08% | 102% | 100% |
| Overall | 0.00% | 0.09% | 0.05% | 103% | 101% |
| Class D | -0.06% | 0.04% | 0.09% | 103% | 99% |
| Class F | 0.17% | 0.56% | 0.52% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.38% | 0.30% | 101% | 100% |
| Class A2 | 0.02% | 0.63% | 0.59% | 101% | 102% |
| Class B | -0.04% | 0.21% | 0.08% | 101% | 102% |
| Class C | -0.01% | 0.09% | 0.09% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.30% | 0.23% | 101% | 102% |
| Class D | -0.07% | -0.43% | -0.03% | 101% | 99% |
| Class F | 0.18% | 0.67% | 0.60% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.25% | -0.09% | 101% | 99% |
| Class C | -0.02% | 0.54% | 0.23% | 101% | 99% |
| Class E | 0.20% | 1.17% | -0.66% | 101% | 101% |
| Overall | 0.02% | 0.58% | -0.12% | 101% | 100% |
| Class D | 0.03% | 0.97% | 0.52% | 101% | 100% |
| Class F | 0.01% | 0.87% | 0.01% | 101% | 99% |

Table 5.  Results of Test-7.4: Only include gt2 flag into the first coding pass

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.11% | -0.05% | 100% | 101% |
| Class A2 | -0.09% | -0.09% | -0.11% | 100% | 102% |
| Class B | -0.08% | -0.13% | -0.08% | 100% | 101% |
| Class C | -0.11% | -0.04% | -0.23% | 99% | 100% |
| Class E | -0.06% | -0.08% | 0.02% | 99% | 100% |
| Overall | -0.08% | -0.09% | -0.10% | 100% | 101% |
| Class D | -0.07% | -0.10% | -0.13% | 103% | 103% |
| Class F | 0.02% | -0.01% | 0.06% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.17% | 0.00% | 100% | 97% |
| Class A2 | -0.06% | -0.08% | -0.06% | 100% | 101% |
| Class B | -0.07% | 0.06% | -0.13% | 100% | 101% |
| Class C | -0.05% | -0.02% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.03% | -0.06% | 100% | 100% |
| Class D | -0.10% | -0.35% | 0.14% | 102% | 100% |
| Class F | 0.01% | 0.09% | -0.17% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.59% | 0.00% | 100% | 101% |
| Class C | -0.04% | 0.20% | 0.31% | 100% | 100% |
| Class E | -0.03% | 0.72% | 0.07% | 100% | 100% |
| Overall | -0.04% | 0.00% | 0.12% | 100% | 100% |
| Class D | -0.05% | -0.26% | 0.12% | 100% | 99% |
| Class F | 0.02% | -0.12% | -0.22% | 100% | 99% |

Table 6.  Results of Test-7.1 in low complexity setting: Set constraint values to 32 for 4x4 kuma subblock, 16 for 4x4 chroma subblock, and 4 for 2x2 chroma subblock

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.27% | 0.67% | 100% | 103% |
| Class A2 | 0.34% | 0.81% | 0.88% | 100% | 101% |
| Class B | 0.04% | 0.51% | 0.45% | 100% | 100% |
| Class C | 0.04% | 0.34% | 0.27% | 102% | 101% |
| Class E | 0.03% | 0.14% | 0.14% | 100% | 101% |
| Overall | 0.12% | 0.42% | 0.47% | 100% | 101% |
| Class D | 0.03% | 0.29% | 0.40% | 99% | 97% |
| Class F | 0.19% | 0.76% | 0.75% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.31% | 0.19% | 0.63% | 100% | 98% |
| Class A2 | 0.05% | 0.80% | 1.00% | 100% | 103% |
| Class B | 0.04% | 0.55% | 0.56% | 100% | 105% |
| Class C | 0.01% | 0.25% | 0.26% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.45% | 0.58% | 100% | 102% |
| Class D | 0.04% | 0.42% | 0.55% | 100% | 100% |
| Class F | 0.21% | 0.67% | 0.66% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 1.58% | 0.72% | 100% | 102% |
| Class C | -0.06% | 1.03% | 0.38% | 100% | 98% |
| Class E | -0.03% | 0.13% | 0.92% | 100% | 100% |
| Overall | -0.02% | 1.03% | 0.66% | 100% | 100% |
| Class D | 0.00% | 1.07% | 0.94% | 100% | 103% |
| Class F | 0.05% | 0.54% | 1.11% | 100% | 99% |

Table 7.  Results of Test-7.2 in low complexity setting: Test-7.1 + last significant subblock position dependent constraint

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.17% | 0.32% | 100% | 102% |
| Class A2 | 0.23% | 0.29% | 0.32% | 100% | 104% |
| Class B | 0.01% | 0.12% | 0.08% | 100% | 108% |
| Class C | 0.01% | 0.11% | 0.12% | 100% | 97% |
| Class E | 0.00% | -0.07% | -0.10% | 100% | 99% |
| Overall | 0.06% | 0.12% | 0.14% | 100% | 102% |
| Class D | -0.01% | 0.09% | 0.10% | 100% | 96% |
| Class F | 0.14% | 0.61% | 0.56% | 99% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.03% | 0.29% | 100% | 99% |
| Class A2 | 0.02% | 0.39% | 0.43% | 100% | 102% |
| Class B | 0.01% | 0.11% | 0.15% | 100% | 105% |
| Class C | 0.00% | 0.09% | 0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.15% | 0.22% | 100% | 102% |
| Class D | -0.01% | 0.19% | 0.69% | 100% | 100% |
| Class F | 0.13% | 0.40% | 0.52% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.52% | 0.16% | 100% | 99% |
| Class C | -0.05% | 0.54% | -0.07% | 100% | 98% |
| Class E | -0.02% | 0.03% | 0.37% | 101% | 99% |
| Overall | -0.01% | 0.40% | 0.13% | 100% | 99% |
| Class D | -0.01% | 0.36% | 1.07% | 100% | 103% |
| Class F | 0.09% | 0.35% | 0.69% | 100% | 100% |

Table 8.  Results of Test-7.3 in low complexity setting: Test-7.2 + include gt2 flag into the first coding pass

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.09% | 0.26% | 99% | 102% |
| Class A2 | 0.14% | 0.20% | 0.23% | 98% | 101% |
| Class B | -0.04% | 0.02% | 0.06% | 99% | 105% |
| Class C | -0.05% | -0.03% | 0.07% | 98% | 97% |
| Class E | -0.06% | -0.12% | -0.03% | 99% | 99% |
| Overall | 0.00% | 0.03% | 0.11% | 99% | 101% |
| Class D | -0.04% | 0.10% | -0.15% | 98% | 98% |
| Class F | 0.17% | 0.60% | 0.58% | 97% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.10% | 0.24% | 100% | 99% |
| Class A2 | 0.04% | 0.32% | 0.39% | 100% | 102% |
| Class B | 0.01% | 0.14% | 0.05% | 100% | 106% |
| Class C | -0.02% | 0.18% | 0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.14% | 0.17% | 100% | 102% |
| Class D | 0.01% | 0.23% | 0.29% | 100% | 100% |
| Class F | 0.16% | 0.47% | 0.65% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.64% | -0.07% | 99% | 100% |
| Class C | -0.05% | 0.42% | -0.18% | 100% | 98% |
| Class E | 0.02% | 0.36% | 0.69% | 100% | 100% |
| Overall | -0.01% | 0.50% | 0.09% | 100% | 99% |
| Class D | -0.01% | 0.53% | 0.50% | 100% | 99% |
| Class F | -0.07% | -0.17% | 0.20% | 101% | 100% |

Table 9.  Results of Test-7.4 in low complexity setting: Only include gt2 flag into the first coding pass

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.02% | 100% | 101% |
| Class A2 | -0.04% | -0.04% | 0.00% | 99% | 102% |
| Class B | -0.05% | -0.07% | -0.06% | 100% | 104% |
| Class C | -0.07% | -0.11% | -0.14% | 99% | 97% |
| Class E | -0.06% | -0.10% | -0.16% | 99% | 100% |
| Overall | -0.05% | -0.08% | -0.08% | 99% | 101% |
| Class D | -0.06% | -0.05% | 0.04% | 98% | 96% |
| Class F | 0.05% | 0.22% | 0.16% | 98% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.19% | 0.08% | 100% | 98% |
| Class A2 | -0.03% | -0.32% | -0.07% | 100% | 102% |
| Class B | -0.02% | 0.03% | -0.01% | 100% | 106% |
| Class C | -0.02% | -0.10% | -0.07% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.12% | -0.02% | 100% | 102% |
| Class D | -0.04% | -0.09% | 0.08% | 100% | 100% |
| Class F | 0.02% | 0.06% | 0.21% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | -0.16% | 99% | 99% |
| Class C | -0.02% | -0.30% | -0.32% | 100% | 97% |
| Class E | 0.08% | -0.22% | 0.32% | 100% | 99% |
| Overall | 0.02% | -0.15% | -0.09% | 100% | 99% |
| Class D | -0.01% | 0.11% | 0.51% | 100% | 100% |
| Class F | -0.07% | -0.50% | -0.21% | 100% | 100% |

# Conclusions

In this contribution, the results of CE7.1, CE7.2, CE7.3, and CE7.4 are reported. It is proposed to constrain the maximum number of context-coded bins to 30 for 4x4 luma subblock, 16 for 4x4 chroma subblock, and 4 for 2x2 chroma subblock. The constraint values can be increased based on the last significant sub-block position. To improve parsing throughput and reduce the number of coding passes, the gt2 flags are moved to the first coding pass after the parity bit. The averaged maximum number of context-coded bins per 16 samples is reduced from 28% to 1.3% compared with HEVC. The loss is not significant.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).