| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0176-v1 |
| --- | --- |

| Title: | CE10.1.1: Multi-hypothesis prediction for improving non-skip & non-merge inter mode and merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, multi-hypothesis (MH) prediction is proposed to improve the existing prediction modes in inter pictures, including uni-prediction of non-skip & non-merge inter (NSNM-inter) mode and merge mode. The general concept is to combine an existing prediction mode with an extra merge indexed prediction. The merge indexed prediction is performed as in merge mode, where a merge index is signalled to acquire motion information for the motion compensated prediction. The final prediction is the weighted average of the merge indexed prediction and the prediction generated by the existing prediction mode. It is reported that, compared to VTM3.0, the proposed MH prediction for uni-prediction of NSNM-inter achieves -0.17% and 0.00% luma BD-rates with 7% and 7% encoding time increases and 0% and 1% decoding time increases for RA and LB, respectively. In CE10.1.1.b, when MH prediction is applied to merge mode, -0.17% and -0.12% luma BD-rates with 3% and 3% encoding time increases and 2% and 2% decoding time increases for RA and LB, respectively, are reported.

# Introduction

Two inter combined prediction modes are tested to combine merge indexed prediction with uni-prediction of non-skip & non-merge inter (NSNM-inter) mode and merge mode, respectively. When combined with uni-prediction of NSNM-inter mode, one motion is acquired using the original syntax elements in NSNM-inter mode while the other is acquired using the merge index, where totally two hypotheses are used. When combined with merge mode, implicit selection of succeeding merge candidate without signalling any additional merge index is used to acquire the additional motion information, where at most two more hypotheses are used.

# Proposed methods

## CE10.1.1.a: Multi-hypothesis prediction for uni-prediction of NSNM-inter mode

When the multi-hypothesis prediction is applied to improve uni-prediction of NSNM-inter mode, one flag is signalled to enable or disable multi-hypothesis prediction for inter_dir equal to 1 or 2, where 1, 2, and 3 represent list 0, list 1, and bi-prediction, respectively. Moreover, one more merge index is signalled when the flag is true. In this way, multi-hypothesis prediction turns uni-prediction into bi-prediction, where one motion is acquired using the original syntax elements in NSNM-inter mode while the other is acquired using the merge scheme. The final prediction uses 1:1 weights to combine these two predictions as in bi-prediction. The merge candidate list is first derived from merge mode with both mmvd_flag and merge_subblock_flag set to 0. Next, it is separated into two individual lists, one for list 0 (L0) containing all L0 motions from the candidates, and the other for list 1 (L1) containing all L1 motions. After removing redundancy and filling vacancy, two merge lists are generated for L0 and L1 respectively. There are two constraints when applying multi-hypothesis prediction for improving NSNM-inter mode. First, it is enabled for those CUs with the luma coding block (CB) area larger than or equal to 64. Second, it is only applied to L1 when in low delay B pictures.

## CE10.1.1.b: Multi-hypothesis prediction for merge mode

When the multi-hypothesis prediction is applied to merge mode, whether to enable multi-hypothesis prediction is explicitly signalled. An extra merge indexed prediction is selected in addition to the original one. Therefore, each candidate of multi-hypothesis prediction implies a pair of merge candidates, containing one for the 1st merge indexed prediction and the other for the 2nd merge indexed prediction. However, in each pair, the merge candidate for the 2nd merge indexed prediction is implicitly derived as the succeeding merge candidate (i.e., the already signalled merge index plus one) without signalling any additional merge index. After removing redundancy by excluding those pairs, containing similar merge candidates, the candidate list for multi-hypothesis prediction is formed. Then, motions from a pair of two merge candidates are acquired to generate the final prediction, where 5:3 weights are applied to the 1st and 2nd merge indexed predictions, respectively. Moreover, a merge or skip CU with multi-hypothesis prediction enabled can save the motion information of the additional hypotheses for reference of the following neighbouring CUs in addition to the motion information of the existing hypotheses. Note that the candidate list is derived from merge mode with both mmvd_flag and merge_subblock_flag set to 0. Moreover, the motion vectors of multiple hypotheses are signalled at luma full-pel accuracy.

# Experimental results

The proposed multi-hypothesis prediction methods have been implemented on VTM3.0 [1]. Table 1 and Table 2 show the results of CE10.1.1.a and CE10.1.1.b, respectively. The simulations were performed following the JVET common test conditions (CTC) [2].

For investigating the impact of multi-hypothesis inter prediction on cache-related aspects, the cache model, compiled into the reference decoder software by setting
#define JVET_J0090_MEMORY_BANDWITH_MEASURE 1
is used in conjunction with the cache configuration file provided in JVET-K0451 [3]. The statistic information of cache hit ratio for CE10.1.1.b is shown in Table 3.

Table 1.  Results of CE10.1.1.a: Multi-hypothesis prediction for uni-prediction of NSNM-inter mode

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.11% | -0.07% | 107% | 100% |
| Class A2 | -0.17% | -0.17% | -0.03% | 107% | 98% |
| Class B | -0.22% | -0.18% | -0.16% | 107% | 102% |
| Class C | -0.14% | -0.06% | -0.08% | 108% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.13% | -0.10% | 107% | 100% |
| Class D | -0.20% | -0.10% | -0.22% | 108% | 101% |
| Class F (optional) | -0.13% | -0.15% | -0.09% | 107% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.21% | 0.29% | 107% | 101% |
| Class C | 0.00% | 0.24% | 0.20% | 107% | 100% |
| Class E | 0.01% | 0.41% | 0.24% | 108% | 101% |
| Overall | 0.00% | 0.10% | 0.25% | 107% | 101% |
| Class D | -0.03% | 0.18% | -0.06% | 106% | 102% |
| Class F (optional) | 0.08% | 0.38% | 0.44% | 107% | 100% |

Table 2.  Results of CE10.1.1.b: Multi-hypothesis prediction for merge mode

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.49% | -0.31% | 102% | 100% |
| Class A2 | -0.05% | -0.08% | 0.07% | 103% | 98% |
| Class B | -0.30% | -0.34% | -0.43% | 103% | 102% |
| Class C | -0.03% | -0.10% | -0.09% | 103% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.25% | -0.22% | 103% | 102% |
| Class D | -0.03% | -0.12% | -0.15% | 102% | 102% |
| Class F (optional) | -0.22% | -0.26% | -0.27% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.66% | -0.23% | 103% | 102% |
| Class C | -0.01% | 0.21% | 0.02% | 102% | 104% |
| Class E | -0.07% | 0.06% | -0.31% | 105% | 101% |
| Overall | -0.12% | -0.19% | -0.17% | 103% | 102% |
| Class D | 0.06% | 0.54% | -0.08% | 103% | 104% |
| Class F (optional) | -0.24% | -0.37% | -0.92% | 102% | 102% |

Table 3.  Cache hit rate of CE10.1.1.b: Multi-hypothesis prediction for merge mode

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VTM-3.0 (%) |  |  | CE10.1.1.b (%) |  |  |
| Class A1 | 99.4735 |  |  | 99.4663 |  |  |
| Class A2 | 99.5509 |  |  | 99.5461 |  |  |
| Class B | 99.5308 |  |  | 99.5249 |  |  |
| Class C | 99.3467 |  |  | 99.3435 |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 99.4755 |  |  | 99.4702 |  |  |
| Class D | 99.3402 |  |  | 99.3401 |  |  |
| Class F (optional) | 98.8734 |  |  | 98.8607 |  |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | VTM-3.0 (%) |  |  | CE10.1.1.b (%) |  |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 99.6490 |  |  | 99.6414 |  |  |
| Class C | 99.5511 |  |  | 99.5415 |  |  |
| Class E | 99.5562 |  |  | 99.5517 |  |  |
| Overall | 99.5854 |  |  | 99.5782 |  |  |
| Class D | 99.5090 |  |  | 99.496 |  |  |
| Class F (optional) | 99.0382 |  |  | 99.0268 |  |  |

# Conclusions

In this contribution, it is proposed to improve the existing prediction modes in inter pictures, including uni-prediction of NSNM-inter mode and merge mode through combining an existing prediction mode with an extra merge indexed prediction. Compared against VTM3.0, the proposed mechanism for NSNM-inter mode (CE10.1.1.a) can achieve -0.17% and 0.00% luma BD-rates with 7% and 7% encoding time increases and 0% and 1% decoding time increases for RA and LB, respectively. In CE10.1.1.b, when the proposed mechanism is applied to merge mode, -0.17% and -0.12% luma BD-rates with 3% and 3% encoding time increases and 2% and 2% decoding time increases for RA and LB, respectively, are reported.

# References

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

- R. Hashimoto, S. Mochizuki, “AHG5: How to use the software to evaluate memory bandwidth,” Document of Joint Video Experts Team, JVET-K0451.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).