| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0185-v1 |
| --- | --- |

| Title: | CE10-related: Syntax redundancy removal in triangle prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove syntax redundancy in triangle prediction. In VTM3.0, triangle prediction does not coexist with combined inter intra prediction (CIIP) and merge with motion vector difference (MMVD). Therefore, it is proposed to avoid signalling merge_triangle_flag, the flag that indicates if triangle merge mode is enabled for a coding unit (CU), when CIIP or MMVD is enabled for the CU. It is reported that, compared to VTM3.0, the proposed method achieves -0.07% and -0.14% luma BD-rates for RA and LB, respectively.

# Introduction

This proposal is to remove syntax redundancy between triangle prediction, CIIP, and MMVD. These three individual tools are described as follows.

## Triangle prediction

The concept of triangle prediction has been introduced in [1] and [2], and this scheme is applied to skip mode and merge mode. A skip/merge CU is split into two triangular partitions either down-right or up-right diagonally. Each triangular partition in the CU has its own uni-prediction motion vector (MV) and reference index to generate prediction samples of the triangular partition. A weighted sample prediction process is performed at the diagonal edge for the two triangular partitions. Then, transform and quantization processes are applied to the entire CU.

## Combined inter intra prediction (CIIP)

CIIP [3] is to combine one intra prediction and one merge indexed prediction, and this scheme is applied to merge mode only. In a merge CU, one flag (mh_intra_flag in JVET-L1001-v7.docx; suggested to renamed as ciip_flag) is signalled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma component, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes, and the size of the intra candidate list can be 3 or 4 depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is excluded; when the CU height is larger than the double of CU width, vertical mode is excluded. One intra prediction mode selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted averaging. For chroma components, direct mode (DM) is always applied without extra signalling. Moreover, the intra prediction mode used for CIIP can be saved for reference by the following neighbouring CUs.

## Merge with motion vector difference (MMVD)

MMVD [4] is a motion vector expression method applied to skip mode and merge mode. An mmvd_flag is signalled to indicate applying MMVD or not. Next, an mmvd_merge_flag is signalled to select one of the first two candidates in the merging candidate list to determine the prediction direction (list 0, list 1, or bi-prediction) and to locate the initial MV of each required list. Finally, an mmvd_distance_idx and an mmvd_direction_idx are signalled to derive the distance and direction from the initial MV of each required list, and the two signalled indices are shared by the two lists in case of bi-prediction.

# Proposed method

Since in VTM3.0, triangle prediction does not coexist with CIIP and MMVD, the flag that indicates whether to enable triangle prediction (i.e., merge_triangle_flag) is not signalled when CIIP or MMVD is enabled.

# Experimental results

The proposed method has been implemented on VTM3.0 [5]. Table 1 shows the results. The simulations were performed following the JVET common test conditions (CTC) [6].

Table 1.  Results of syntax redundancy removal between triangle prediction, CIIP, and MMVD

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.08% | 101% | 100% |
| Class A2 | -0.10% | -0.09% | -0.02% | 100% | 98% |
| Class B | -0.07% | -0.01% | -0.18% | 100% | 102% |
| Class C | -0.07% | -0.06% | -0.09% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.11% | 100% | 102% |
| Class D | -0.10% | -0.20% | -0.07% | 100% | 100% |
| Class F (optional) | -0.03% | -0.01% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.32% | 0.00% | 100% | 100% |
| Class C | -0.19% | 0.03% | -0.14% | 100% | 101% |
| Class E | -0.13% | 0.03% | -0.17% | 100% | 101% |
| Overall | -0.14% | -0.12% | -0.09% | 100% | 101% |
| Class D | -0.17% | 0.13% | 0.13% | 100% | 101% |
| Class F (optional) | 0.00% | -0.14% | -0.54% | 100% | 100% |

# Conclusions

This contribution proposes to remove the syntax redundancy between triangle prediction, CIIP, and MMVD. Compared against VTM3.0, the proposed method achieves -0.07% and -0.14% luma BD-rates for RA and LB, respectively.

# References

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode,” Document of Joint Video Experts Team, JVET-L0124.

- T. Poirier, F. Le Léannec, P. Bordes, “CE10 related: multiple prediction unit shapes,” Document of Joint Video Experts Team, JVET-L0208.

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” Document of Joint Video Experts Team, JVET-L0100.

- S. Jeong, M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression (Test 4.5.4),” Document of Joint Video Experts Team, JVET-L0054.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).