| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0214-v2 |
| --- | --- |

| Title: | CE3-related: Uniform chroma intra candidates modification based on DM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma Xinwei Li Jianglin Wang Junyan Huo Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | yzma@mail.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn jlwang_2@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

Abstract

This contribution presents a uniform chroma intra candidates modification based on DM. Specifically, Vertical and Horizontal are replaced by two directional modes related to DM, which are DM±N modes, where N=4, no matter DM is directional mode or not.

The simulation results show -x.xx%, -x.xx%, and -x.xx% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration. The proposed method has negligible impact on the VTM encoder and decoder complexity. It has -x.xx%, x.xx%, x.xx% BD-rates on Y, Cb, and Cr components, respectively, compared to the method based on directional DM proposed in JVET-M0213.

Introduction

In VVC draft, chroma intra coding modes include DM, LM and 4 additional default modes (Planar, Vertical, Horizontal and DC). When the chroma intra prediction mode is DM, the intra prediction mode of the co-located luma block is used. In case of DM being equal to any of the default modes, that default mode is replaced by VDIA_IDX (mode 66).

Proposed method

Chroma DM is a mode which directly uses the intra prediction mode of the co-located luma block. The separate tree for luma and chroma is used in VVC, there might be multiple luma blocks corresponding to the current chroma block. Only the mode of the center block in the region is used as DM.

This contribution proposes to use DM±4 instead of Vertical and Horizontal all the time. If the DM is mode M (0<=M<=66). Let M1 and M2 denote M+4 and M-4. M1 and M2 are obtained as follows:

N=4;

M1 = ((M + 62 - N) % 64) + 2;

M2 = (M -2 + N)% 64 + 2;

When DM is PLANAR/DC, DM+4 is mode 4/5 and DM-4 is mode 61/60 according to the above equations. Then the default mode equal to DM is replaced by VDIA_IDX (mode 66).

Chroma intra prediction modes of the proposed method is shown in Table 1.

Table 1 Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | X ( 2 <= X <= 66 ) |
| 0 | 66 | 0 | 0 |
| 1 | X-N | X-N | X-N |
| 2 | X+N | X+N | X+N |
| 3 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 |
| 7 | 0 | 1 | X |

The proposed method has negligible impact on the VTM encoder and decoder complexity.

Experimental results

The experimental results of the proposed method are presented in Table 2. And the results of the proposed method compared to Directional DM scheme are shown in Table 3. The simulations are under the JVET common test condition.

Anchor: VTM3.0

Tested: The proposed method

Table 2. The results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.18% | -0.19% |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.01% | -0.24% | -0.28% |  |  |
| Class C | -0.04% | -0.28% | -0.55% |  |  |
| Class E | -0.07% | -0.32% | -0.46% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.03% | -0.21% | -0.46% |  |  |
| Class F | 0.03% | -0.18% | -0.19% |  |  |

Anchor: Directional DM scheme

Tested: The proposed method

Table 3. The results of the proposed method compared to Directional DM scheme

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over M0213 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | -0.06% |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | 0.00% | -0.07% | -0.01% |  |  |
| Class C | -0.02% | 0.01% | -0.08% |  |  |
| Class E | -0.03% | -0.08% | -0.09% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.02% | 0.04% | -0.09% |  |  |
| Class F | 0.04% | 0.06% | 0.06% |  |  |

Conclusions

This contribution presents chroma intra candidates modification based on DM mode. The proposed method gives x.xx%, x.xx%, and x.xx% BD rate savings on Y, Cb and Cr components for All Intra configuration. Compared with Directional DM scheme, it has x.xx%, x.xx%, x.xx% BD-rates on Y, Cb, and Cr components

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).