| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0240 |
| --- | --- |

| Title: | CE2-related: Simplification of subblock-based temporal merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Sung-Chang Lim Jinho Lee Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +81-42-860-6138 hanilee@etri.re.kr sclim@etri.re.kr jinosoul@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution proposes to use a fixed spatial neighboring block of the current block, the left (A1) block, for deriving subblock-based temporal merging candidate to simplify the derivation process of collocated subblock TMVP position. With the proposed method, the average luma BD-rates are reportedly 0.00%, 0.02% and 0.00% with decoding times of 100%, 100% and 100% for RA, LDB and LDP cases, respectively. It is asserted that the proposed method simplifies the derivation process of collocated subblock TMVP position while having negligible impact on the BD-rate performance.

Introduction

Regarding the subblock-based temporal merging candidate, the following three modifications were adopted into Versatile Video Coding (VVC) Working Draft (WD) 3 [1] at the 12th JVET meeting. First, the subblock-based temporal merging candidate is moved into affine (sub-block) merge candidate list. Second, the first available spatial neighbouring motion vector (MV) among spatial merging candidates is used for deriving collocated subblock TMVP position. Third, the subblock size is fixed to 8x8 for subblock-based temporal merging candidate. Although the second adopted item reduced the complexity for identifying the collocated block compared to VVC WD2, the following issues still remain:

- The first available neighbouring MV among the spatial merging candidates is used for deriving the collocated subblock TMVP position, which means the spatial merging candidates for merge mode has to be derived although the subblock-based temporal merging candidate has been moved into sub-block merge candidate list.

- The availability checking process for four spatial neighboring blocks (A1, B1, B0, and A0) is required to derive the subblock-based temporal merging candidate. In other words, a maximum of 4 available checks are needed to derive first available neighbouring MV.

Proposed Method

It is proposed that a fixed spatial neighbouring block’s motion vector is used as temporal MV (tempMV) for a subblock-based temporal merging candidate to simplify the derivation process of collocated subblock TMVP position.

In the VVV WD 3, the tempMV for the collocated subblock TMVP position is derived as in the Table 1. It can be found that the derivation process for merging candidates from four spatial neighbouring blocks is necessary for this process.

Table 1. TempMV derivation process in VVC WD3

If ( A1 is available ) { if (A1 refers to the collocated picture as a reference picture ) tempMV = mvLXA1 else tempMV = zero MV } else if ( B1 is available ) { if (B1 refers to the collocated picture as a reference picture) tempMV = mvLXB1 else tempMV = zero MV } else if ( B0 is available ) { if (B0 refers to the collocated picture as a reference picture) tempMV = mvLXB0 else tempMV = zero MV } else if ( A0 is available ) { if (A0 refers to the collocated picture as a reference picture ) tempMV = mvLXA0 else tempMV = zero MV } else tempMV = zero MV

In the proposed method, the following neighbouring luma coding block location is used as the fixed spatial neighbouring block for tempMV as shown in the Fig 1 and tempMV is derived as in the Table 2. The remaining part of the subblock-based temporal merging candidate kept unchanged.

(xNbA1, yNbA1) = (xCb − 1, yCb + cbHeight − 1).

Table 2. TempMV derivation process in the proposed method

If ( A1 is available ) { if (A1 refers to the collocated picture as a reference picture) tempMV = mvLXA1 else tempMV = zero MV } else tempMV = zero MV

Fig. 1. The fixed spatial neighbouring block location for tempMV (A1)

Experimental Results

The proposed method was integrated on VTM-3.0 and the experiments were performed under JVET common test conditions [2]. The experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 4.7.2 compiler under CentOS Linux.

Results of proposed method

BD-rate results on top of VTM-3.0 compared to the anchor with RA, LDB and LDP configurations are shown in the Table 3. The full results are provided in the attached excel sheets.

The average BD-rates of 0.00%, 0.02% and 0.01% are observed for RA, LDB and LDP cases, respectively. There are no changes in the encoding and decoding times.

Table 3. BD-rate results of proposed method compared to VTM-3.0 anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Class B | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class C | 0.02% | -0.01% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.04% | 0.03% | 100% | 100% |
| Class F | -0.01% | -0.02% | -0.01% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.22% | 0.13% | 100% | 100% |
| Class C | -0.01% | 0.18% | 0.26% | 100% | 100% |
| Class E | 0.06% | -0.25% | -0.16% | 100% | 100% |
| Overall | 0.02% | -0.09% | 0.10% | 100% | 100% |
| Class D | -0.02% | 0.00% | -0.10% | 100% | 100% |
| Class F | -0.01% | -0.03% | -0.49% | 100% | 100% |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.25% | 100% | 100% |
| Class C | 0.01% | 0.25% | -0.13% | 100% | 100% |
| Class E | 0.00% | -0.42% | -0.45% | 100% | 100% |
| Overall | 0.00% | 0.04% | -0.05% | 100% | 100% |
| Class D | -0.02% | -0.15% | -0.13% | 100% | 100% |
| Class F | 0.01% | -0.75% | -0.36% | 100% | 100% |

Additional results of using a fixed spatial neighbouring (B1) block

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.04% | 100% | 100% |
| Class A2 | -0.02% | 0.00% | 0.06% | 100% | 100% |
| Class B | 0.01% | 0.05% | -0.03% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class D | 0.02% | -0.12% | -0.13% | 100% | 100% |
| Class F | -0.01% | -0.02% | 0.02% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.19% | 0.06% | 100% | 100% |
| Class C | 0.01% | 0.30% | 0.13% | 100% | 100% |
| Class E | 0.07% | 0.05% | -0.47% | 100% | 100% |
| Overall | 0.02% | 0.03% | -0.05% | 100% | 100% |
| Class D | 0.04% | -0.02% | 0.06% | 100% | 100% |
| Class F | 0.06% | -0.04% | 0.27% | 100% | 99% |

Additional results of using a fixed spatial neighbouring (B0) block

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.07% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Class C | 0.04% | 0.07% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.03% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.00% | 100% | 100% |
| Class F | -0.01% | -0.01% | -0.02% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.23% | -0.10% | 100% | 100% |
| Class C | 0.05% | 0.19% | 0.17% | 100% | 100% |
| Class E | 0.11% | 0.05% | 0.32% | 100% | 100% |
| Overall | 0.05% | -0.02% | 0.10% | 100% | 100% |
| Class D | -0.03% | 0.03% | 0.35% | 100% | 100% |
| Class F | 0.04% | 0.33% | -0.01% | 100% | 100% |

Additional results of using a fixed spatial neighbouring (A0) block

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.11% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.08% | 100% | 100% |
| Class B | 0.01% | 0.09% | -0.04% | 100% | 100% |
| Class C | 0.04% | 0.05% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.07% | 0.03% | 100% | 100% |
| Class D | 0.03% | -0.06% | 0.03% | 100% | 100% |
| Class F | -0.01% | -0.01% | 0.00% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.12% | 0.04% | 100% | 100% |
| Class C | 0.02% | 0.17% | 0.17% | 100% | 100% |
| Class E | 0.08% | -0.01% | -0.10% | 100% | 100% |
| Overall | 0.04% | 0.00% | 0.05% | 100% | 100% |
| Class D | 0.03% | 0.19% | 0.80% | 100% | 100% |
| Class F | 0.03% | 0.23% | -0.11% | 100% | 100% |

Additional results of always using zero motion vector

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.02% | 0.03% | 100% | 100% |
| Class A2 | 0.06% | 0.03% | 0.09% | 100% | 100% |
| Class B | 0.03% | 0.05% | -0.02% | 100% | 100% |
| Class C | 0.10% | 0.09% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.04% | 0.04% | 100% | 100% |
| Class D | 0.02% | -0.02% | 0.00% | 100% | 100% |
| Class F | 0.01% | -0.02% | -0.03% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.06% | 0.06% | 100% | 100% |
| Class C | 0.07% | 0.29% | 0.08% | 100% | 100% |
| Class E | 0.02% | 0.51% | -0.10% | 100% | 100% |
| Overall | 0.04% | 0.20% | 0.03% | 100% | 100% |
| Class D | -0.05% | -0.47% | 0.15% | 100% | 100% |
| Class F | 0.06% | -0.10% | -0.27% | 100% | 100% |

Conclusions

This contribution proposes to use a fixed spatial neighbouring block of the current block, the left (A1) block, for deriving subblock-based temporal merging candidate to simplify the derivation process of collocated subblock TMVP position.

The experimental results show that there are no significant performance changes compared to VTM-3.0 anchor. Therefore, it is recommended to adopt the proposed method to the VVC WD4.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1001, Oct. 2018.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, Oct. 2018

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).