| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0246 |
| --- | --- |

| Title: | CE2: Adaptive Motion Vector Resolution for Affine Inter Mode (Test 2.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Kai Zhang, Li Zhang, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong Yingdu Mansion F11, 48 Zhichun Road, Building B, Haidian District, 100086 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of CE2.1.2. In CE2.1.2, Adaptive Motion Vector Resolution (AMVR) is extended to affine inter mode, wherein MV precision of an affine-coded CU could be 1/16-pel, 1/4-pel or 1-pel. Simulation results reportedly show 0.23% BD-rate saving in Random Access (RA) configuration.

# Introduction

The versatile video coding test model (VTM) [1] employs AMVR for Advanced Motion Vector Prediction (AMVP) mode, wherein different MV precisions including 1/4-pel, 1-pel and 4-pel can be selected adaptively at CU-level. A syntax element is signaled to the decoder to indicate the selected MV precision when there is non-zero motion vector difference (MVD) component. Comparing with always using 1/4-pel MV precision, AMVR can achieve better compromise between MV precision and bits spent on MVD.

In affine inter mode, MVD of two/three control points are encoded for 4/6-parameter affine model. However, unlike AMVP mode, affine inter mode always uses 1/4-pel MV precision, which may be inefficient.

# Proposed method

To address the problem mentioned above, in CE2.1.2, AMVR is extended to affine inter mode and MV precision set {1/16, 1/4, 1}-pel is used [1]. If one MV precision is selected for an affine-coded block, both MVP candidates and MVD are rounded to the selected MV precision, as AMVR does.

The syntax for MV/MVD precision used for AMVP mode is reused in this proposal. No additional syntax element is added.

At the encoder side, one of the three MV precisions could be selected for an affine-coded inter block. In VTM, AMVP mode and affine inter mode are checked together in one loop. For 1/4-pel MV precision, both AMVP mode and affine inter mode are checked. However, for 1-pel MV precision and 4-pel MV precision, only AMVP mode is checked. After extending AMVR to affine inter mode, affine inter mode using 1/16-pel MV precision is checked together with AMVP mode using 1-pel MV precision, and affine inter mode using 1-pel MV precision is checked together with AMVP mode using 4-pel MV precision.

Testing all three MV/MVD precisions in affine inter mode will significantly increase the encoding complexity. To address this issue, we propose following fast algorithms:

- If affine inter mode is not selected after checking rate-distortion costs of affine merge/skip mode, merge/skip mode, 1/4-pel MV precision AMVP mode and affine inter mode, then 1/16-pel MV precision and 1-pel MV precision affine inter modes are not checked.

- Affine parameters obtained in 1/4-pel MV precision affine inter mode is reused as starting search point in 1/16-pel and 1-pel MV precision affine inter modes.

# Experimental results

The proposed method was implemented on top of reference software VTM-3.0 [2], and simulations were performed following the common test conditions [3]. The results are shown in Table 1.

Table 1: Coding performance

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.27% | -0.37% | 103% | 100% |  |  |  |  |  |
| Class A2 | -0.35% | -0.30% | -0.27% | 105% | 100% |  |  |  |  |  |
| Class B | -0.19% | -0.10% | -0.31% | 104% | 99% | -0.22% | -0.44% | -0.45% | 110% | 100% |
| Class C | -0.17% | -0.24% | -0.14% | 105% | 98% | -0.23% | 0.21% | -0.14% | 110% | 101% |
| Class E |  |  |  |  |  | -0.13% | 0.15% | -0.26% | 118% | 99% |
| Overall | -0.23% | -0.21% | -0.27% | 104% | 99% | -0.20% | -0.08% | -0.30% | 112% | 100% |
| Class D | -0.23% | -0.16% | -0.26% | 105% | 98% | -0.47% | -0.40% | -0.72% | 110% | 102% |
| Class F | -0.13% | -0.14% | -0.11% | 107% | 99% | -0.87% | -0.42% | -1.28% | 116% | 98% |

# Conclusions

CE2.1.2 can achieve 0.23%/0.20% luma BD-rate saving for RA/LB under VTM-3.0. It is recommended to adopt it into VVC.

# References

- “CE4-related: Adaptive Motion Vector Resolution for Affine Inter Mode”, H. Liu, L. Zhang, K. Zhang, Y. Wang, P. Zhao, D. Hong, Doc. JVET-L0332, Oct. 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).