| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0248 |
| --- | --- |

| Title: | AHG16: Motion compensation with padded samples for small coding units |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Jaehong Chon, Hsiao Chiang Chuang, Li Zhang, Kai Zhang, Jizheng Xu Yingdu Mansion F11, 48 Zhichun Road, Building B, Haidian District, 100086 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, bi-predicted 4x16, 16x4, 4x8 and 8x4 coding units (CU) and uni-predicted 4x4 CUs require even higher memory bandwidth than 8x8 bi-predicted CUs. This contribution proposes to fetch only some of the reference pixels and pad other reference pixels in the motion compensation process. With proposed method, it is asserted that the worst-case memory bandwidth occurs in 8x8 bi-predicted CUs which is aligned with HEVC design. Simulation results reportedly show negligible impact on coding performance.

# Introduction

In current VVC, bi-predicted 4x16, 16x4, 4x8 and 8x4 coding unit (CU) and uni-predicted 4x4 CU require higher memory bandwidth than 8x8 bi-predicted CU. The memory bandwidth required by these CUs are summarized in Table 1 for YCbCr 4:2:0 case, wherein both luma and chroma components are considered. Compared with HEVC wherein 8x8 bi-predicted CU requires the highest memory bandwidth, the worst-case memory bandwidth of VVC is increased by around 46%.

Table 1: bandwidth comparison of different CUs

| CUs | Bandwidth per luma pixel | Bandwidth / bandwidth of 8x8 bi-predicted CUs |
| --- | --- | --- |
| 8x8 Bi | ((8 + 7) × (8 + 7) + (4 + 3) × (4 + 3) × 2) × 2 / 64 = 10.09375 | 100% |
| 4x4 Uni | ((4 + 7) × (4 + 7) + (2 + 3) × (2 + 3) × 2) / 16 = 10.6875 | 106% |
| 4x8/8x4 Bi | ((4 + 7) × (8 + 7) + (2 + 3) × (4 + 3) × 2) × 2 / 32 = 14.6875 | 146% |
| 4x16/16x4 Bi | ((4 + 7) × (16 + 7) + (2 + 3) × (8 + 3) × 2) × 2 / 64 = 11.34375 | 112% |

# Proposed method

To address the problem mentioned above, this contribution proposes to fetch only some of the reference pixels and pad other pixels in the motion compensation process. More specifically, the following apply:

- 4x4 uni-predicted CU: for luma, the bottom row is padded; for chroma, no padding is required.

- 4x8/8x4 bi-predicted CU: for luma, the top row and the bottom 2 rows are padded, and the left column and the right column are padded; for chroma, the bottom row and the right column are padded.

- 4x16/16x4 bi-predicted CU: for luma, the bottom row and the right column are padded; for chroma, the bottom row is padded.

Figure 2-1: reference pixel padding in 4x4 uni-predicted CU.

An example of reference pixel padding is shown in Figure 2-1. The padding method applies when both horizonal and vertical MV components of a block (including blocks coded in affine mode) are fractional, no matter whether the block is uni-predicted or bi-predicted. With proposed method, these CUs requires lower memory bandwidth than 8x8 bi-predicted CU, as shown in Table 2.

Table 2: bandwidth comparison of different CUs with proposed method.

| CUs | Bandwidth per luma pixel | Bandwidth / bandwidth of 8x8 bi-predicted CUs |
| --- | --- | --- |
| 8x8 Bi | ((8 + 7) × (8 + 7) + (4 + 3) × (4 + 3) × 2) × 2 / 64 = 10.09375 | 100% |
| 4x4 Uni | ((4 + 7 – 1) × (4 + 7) + (2 + 3) × (2 + 3) × 2) / 16 = 10 | 99% |
| 4x8 Bi | ((4 + 7 – 2) × (8 + 7 – 3) + (2 + 3 – 1) × (4 + 3 – 1) × 2) × 2 / 32 = 9.75 | 97% |
| 8x4 Bi | ((8 + 7 – 2) × (4 + 7 – 3) + (4 + 3 – 1) × (2 + 3 – 1) × 2) × 2 / 32 = 9. 5 | 94% |
| 4x16 Bi | ((4 + 7 – 1) × (16 + 7 – 1) + (2 + 3) × (8 + 3 – 1) × 2) × 2 / 64 = 10 | 99% |
| 16x4 Bi | ((16 + 7 – 1) × (4 + 7 – 1) + (8 + 3) × (2 + 3 – 1) × 2) × 2 / 64 = 9.625 | 95% |

# Experimental results

The proposed method was implemented on top of reference software VTM-3.0 [1], and simulations were performed following the common test conditions [2]. The results are shown in Table 3. As can be seen, proposed method has negligible impact on compression performance.

Table 3: Coding performance of proposed method.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | -0.03% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.05% | 0.12% | 101% | 101% |  |  |  |  |  |
| Class B | -0.01% | 0.11% | 0.00% | 99% | 100% | 0.03% | -0.37% | 0.01% | 102% | 100% |
| Class C | 0.04% | 0.14% | 0.05% | 99% | 99% | 0.06% | 0.28% | 0.16% | 99% | 102% |
| Class E |  |  |  |  |  | 0.06% | 0.09% | -0.11% | 100% | 102% |
| Overall | 0.01% | 0.09% | 0.03% | 100% | 100% | 0.04% | -0.04% | 0.03% | 101% | 101% |
| Class D | 0.03% | 0.14% | 0.30% | 101% | 102% | 0.10% | 0.22% | 0.32% | 97% | 99% |
| Class F | 0.01% | 0.02% | 0.03% | 100% | 100% | 0.14% | 0.23% | 0.20% | 98% | 100% |

Meanwhile, we also tested disabling bi-prediction for 16x4, 4x16, 8x4 and 4x8 CUs. The results are shown in Table 4. As can be seen, disabling bi-prediction for small CUs causes large compression performance loss on low resolution test sequences, such as class D and class C.

Table 4: Coding performance of disabling bi-prediction for 16x4/4x16/8x4/4x8 CUs.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.11% | 0.08% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.17% | 0.44% | 0.62% | 100% | 99% |  |  |  |  |  |
| Class B | 0.26% | 0.48% | 0.46% | 100% | 99% | 0.24% | 0.22% | 0.52% | 100% | 101% |
| Class C | 0.40% | 0.49% | 0.55% | 100% | 100% | 0.38% | 0.67% | 0.66% | 99% | 103% |
| Class E |  |  |  |  |  | 0.12% | 0.40% | -0.19% | 99% | 100% |
| Overall | 0.23% | 0.40% | 0.44% | 100% | 99% | 0.26% | 0.41% | 0.39% | 99% | 101% |
| Class D | 0.64% | 0.63% | 0.75% | 100% | 101% | 0.58% | 1.00% | 1.02% | 98% | 104% |
| Class F | 0.19% | 0.22% | 0.28% | 100% | 101% | 0.23% | 0.49% | 0.28% | 100% | 101% |

# Conclusions

This contribution proposes a method to reduce the memory bandwidth required by small CUs. With proposed method, the worst-case memory bandwidth of VVC is same with HEVC. Meanwhile, there is negligible impact on compression performance. It is recommended to adopt it into VVC.

# References

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).