| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0254 |
| --- | --- |

| Title: | Non-CE8: Subblock Operation Removal for Chroma CPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current design of CPR, when separate tree is applied, chroma blocks inherit collocated luma blocks’ block vectors, which requires sub-block by sub-block operation. This contribution proposes to remove such a subblock-wise operation and reports a 0.26% coding gain for class TGM420.

# Introduction

In the current CPR design, when separate tree is applied and a chroma block is coded in CPR mode, a subblock by subblock block vector derivation is performed to inherit collocated luma subblock’s block vector. Thus, it needs 2x2 block operations. In this contribution, a scheme is proposed to remove such operations.

# Proposed scheme

When a Chroma block uses CPR mode, the whole chroma block is assigned with one block vector. Such a block vector is inherited from that associated with the center position of the collocated Luma region. If the collocated Luma block is not available, (-w, 0) and (0, -h) will be checked in order, to get a valid BV. If all three BVs are not valid, CPR mode is not applied for the current chroma block.

# Experimental Results

The software used is VTM3.0 CPR on, with bugfix for ticket 140, where the bug will cause inconsistent results during different runs.

Table I. Coding results for (a) AI and (b) RA cases. Anchor is VTM3.0 with bugfix for ticket 140 and with CPR on. Test is based on the same software, with the proposed modification.

- All_intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.02% | -0.01% | 101% | 97% |
| Class A2 | -0.01% | -0.08% | -0.01% | 101% | 97% |
| Class B | -0.01% | -0.05% | -0.07% | 100% | 98% |
| Class C | 0.01% | -0.10% | -0.12% | 99% | 95% |
| Class E | -0.01% | -0.05% | -0.14% | 99% | 96% |
| Overall | 0.00% | -0.06% | -0.07% | 100% | 97% |
| Class D | 0.01% | 0.11% | -0.14% | 100% | 97% |
| Class F (optional) | -0.03% | -0.21% | -0.10% | 100% | 97% |
| Class SCC 1080p | -0.26% | -0.44% | -0.51% | 100% | 100% |

- Random_access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.02% | 0.00% | -0.05% | 101% | 100% |
| Class A2 | -0.01% | 0.14% | -0.01% | 100% | 100% |
| Class B | -0.01% | -0.11% | 0.05% | 98% | 98% |
| Class C | 0.04% | -0.15% | -0.19% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.05% | 99% | 98% |
| Class D | -0.03% | -0.11% | -0.18% | 100% | 85% |
| Class F (optional) | -0.05% | -0.15% | -0.13% | 100% | 96% |
| Class SCC 1080p | -0.07% | -0.23% | -0.24% | 98% | 99% |

# Conclusions

This contribution proposes to simplify the chroma blocks’ block vector derivation process wherein the whole block is assigned with one block vector. With the simplified solution, it could even bring additional coding gains without any running time increase. It is recommended to adopt this proposal into VVC.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).