| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0260 |
| --- | --- |

| Title: | AHG17: Carriage of tile group header parameters in higher level structures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The following options are proposed in this contribution:

- PPS containing tile group header parameters.

- Tile group header parameters are divided into subsets that are selectively included either in the PPS or in the tile group header.

- Different PPSs may be activated by different tile groups of the same picture, but besides the tile group header parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

- PPS containing ALF parameters and picture header containing other tile group header parameters.

- A picture header VCL NAL unit is present and precedes all tile group VCL NAL units of the picture.

- The tile group header parameters besides ALF parameters may be included in the picture header. If not present in the picture header, they are included in the tile group header(s).

- The ALF parameters may be included in the PPS, picture header, or tile group header.

- Different PPSs may be activated by different tile groups of the same picture, but besides the ALF parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

The proponent prefers option 2, since it keeps the conceptual design of including such parameters into the PPS that may be applicable to more than one picture, whereas parameters that are likely to change on picture basis are not included in the PPS. A picture header NAL unit as in option 2 was considered simpler than a header parameter set, since the activation of a picture header takes place implicitly.

# Background: JVET-L0183

The remaining of this sub-section contain a copy of the Macau meeting minutes (JVET-L_Notes_dB.docx) for JVET-L0183:

Two options are proposed in this contribution:

- Slice header parameters are selectively included either in the PPS or in the slice header.

- Slice header parameters are selectively included either in a header parameter set (HPS) or in the slice header. Different HPSs may used in different slices of the same picture.

Option 1 was added to version 3 of the contribution. Option 2 has been been included in the contribution since version 1.

Potential bit rate savings were estimated for monoscopic cubemap-projected 360° 30-Hz video with 96 tiles, each carried in its own slice. Version 2 of the contribution contains a 9-frame example analysis indicating that the proposal reportedly provides 10.0% bit rate saving in luma Bjontegaard delta bit rate. Version 3 of the contribution contains a 32-frame simulation estimating that the proposal provides 14.9% bit rate saving in luma Bjontegaard delta bit rate.

For some use cases as discussed (e.g., viewport dependent streaming), it would be undesirable to group the tiles into large slices.

It was noted that, e.g., for ALF, if the overhead of the parameters becomes substantial, it would be better to just disable the feature than to send its parameters. Repeating ALF parameters several times in a picture would probably do more harm than disabling ALF for the picture.

It was commented that an approach we have used a number of times is to have something that can be sent at a high level with the ability to override or modify it at a lower level (e.g., slice header).

Small variations of the same concepts have come up several times in the past. It seems to be mostly just a coding efficiency and use case question what exact approach we would want to use.

It seems clear that if we keep ALF we will need a way to store its parameters to share their use in multiple VCL NAL units of a picture.

Would different ALF parameters be needed in different parts of the picture?

Would ALF parameters be stored and shared by multiple pictures or coded differentially relative to values used in other pictures?

Further study encouraged.

# Overview of the proposal

The following options are proposed in this contribution:

- PPS containing tile group header parameters.

This option is asserted to combine options 1 and 2 of JVET-L0183 and avoids the need for two parameter sets (PPS and header parameter set as in option 2 of JVET-L0183).

The option can be summarized as follows:

  - Tile group header parameters are divided into subsets that are selectively included either in the PPS or in the tile group header.

  - Different PPSs may be activated by different tile groups of the same picture, but besides the tile group header parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

- PPS containing ALF parameters and picture header containing other tile group header parameters:

  - A picture header VCL NAL unit is present and precedes all tile group VCL NAL units of the picture and all SEI NAL units associated with the tile group VCL NAL units of the picture.

  - The tile group header parameters besides ALF parameters may be included in the picture header. If not present in the picture header, they are included in the tile group header(s).

  - The ALF parameters may be included in the PPS. If they are not included in the PPS, they may be included in a picture header. If they are present neither in the PPS nor in the picture header and if ALF is enabled in SPS, they are present in the tile group header.

  - Different PPSs may be activated by different tile groups of the same picture, but besides the ALF parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

The proponent prefers option 2, since it keeps the conceptual design of including such parameters into the PPS that may be applicable to more than one picture, whereas parameters that are likely to change on picture basis are not included in the PPS. A picture header NAL unit as in option 2 was considered simpler than a header parameter set, since the activation of a picture header takes place implicitly.

In H.264/AVC and HEVC, PPSs were designed so that there is no parsing dependency on the active SPS. This enabled implementations where a PPS is parsed to an internal data structure at the time of its reception when a correct SPS might not have been activated yet. It is suspected that such implementations are not likely to be common and hence it is proposed that in VVC a parsing dependency between parameter sets could be allowed. However, including such parsing dependency in VVC should provide a substantial benefit e.g. in bit count.

The proposed syntax structures currently do not have parsing dependencies on any parameter sets. If parsing dependencies were allowed or if the syntax structures were contained in a picture header where parsing dependencies on a picture parameter set are inherently allowed, the syntax structures could be optimized based on SPS and PPS tool enabling flags.

Since the picture header always precedes the tile group VCL NAL units of the picture, it is implicitly associated with the tile group VCL NAL units. For error-prone transmission environments one or both of the following could be considered:

- Allowing repeating the same picture header VCL NAL unit within the coded picture prior to the last tile group VCL NAL unit.

- Including an identifier in the picture header VCL NAL unit and referencing the identifier explicitly from the tile group VCL NAL unit.

# Option 1: PPS containing tile group header parameters

## Subsets of tile group header parameters

### Overview

The following subsets of tile group header parameters with several syntax elements were identified from JVET-L1001-v7.docx:

- ALF parameters, alf_data( )

- Partition constraint parameters

- Miscellaneous inter coding parameters

- Deblocking filter parameters

The proposed syntax structures for each category are presented below.

Other structures that could be added in the future include:

- Reference picture set syntax elements

- Number of active reference pictures and reference picture list modification

- Prediction weights for weighted prediction

- Slice segment header extension

### Partition constraint parameters

| partition_constraint_param( ) { | Descriptor |
| --- | --- |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |

### Miscellaneous inter coding parameters

| tile_group_inter_param( ) { | Descriptor |
| --- | --- |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) |  |
| collocated_from_l0_flag | u(1) |
| six_minus_max_num_merge_cand | ue(v) |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |

### Deblocking filter parameters

| tile_group_deblocking_param( ) { | Descriptor |
| --- | --- |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |

## Syntax

### Overview

Three-level indicators for each subset of tile group header parameters are included in the PPS, where

- Value 0 specifies that the respective subset of the tile group header parameters is not present in the PPS and is always present in the tile group header.

- Value 1 specifies that the respective subset of the tile group header parameters is present in the PPS and may be overridden in the tile group header, as controlled by tile group header gating flags.

- Value 2 specifies that the respective subset of the tile group header parameters is present in the PPS and is not present in the tile group headers referring to the PPS.

### PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_alf_idc | u(2) |
| if( pps_alf_idc ) |  |
| alf_data( ) |  |
| pps_partition_constraints_idc | u(2) |
| if( pps_partition_constraints_idc ) |  |
| partition_constraint_param( ) |  |
| pps_inter_param_idc | u(2) |
| if( pps_inter_param_idc ) |  |
| tile_group_inter_param( ) |  |
| pps_deblocking_param_idc | u(2) |
| tile_group_deblocking_param( ) |  |
| … |  |

### Tile group header syntax

The change marks are relative to JVET-L1001-v7.docx. Naming change from "slice_" to "tile_group_" has not been carried out.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_address | u(v) |
| slice_type | ue(v) |
| if( partition_constraints_override_enabled_flag && pps_partition_constraints_idc < 2 ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( slice_type != I && pps_inter_param_idc < 2 ) { |  |
| if( pps_inter_param_idc = = 1 ) |  |
| tile_group_inter_param_flag | u(1) |
| if( pps_inter_param_idc = = 0 ¦ ¦ tile_group_inter_param_flag ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag && pps_alf_idc < 2 ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag && pps_deblocking_param_idc < 2 ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Option 2: PPS containing ALF parameters and picture header containing other tile group header parameters

## PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_alf_flag | u(1) |
| if( pps_alf_flag ) |  |
| alf_data( ) |  |
| … |  |

## Picture header syntax

It is suggested to specify the picture header NAL unit as a VCL NAL unit, since it is an integral part of a coded picture.

| pic_header_rbsp( ) { | Descriptor |
| --- | --- |
| pic_coding_type | u(3) |
| pic_hdr_pps_id | ue(v) |
| pic_hdr_tile_group_header_flag | u(1) |
| if( pic_hdr_tile_group_header_flag ) |  |
| tile_group_header_param( pic_coding_type ) |  |
| if( sps_alf_enabled_flag && !pps_alf_flag ) { |  |
| pic_hdr_alf_flag | u(1) |
| if( pic_hdr_alf_flag ) |  |
| alf_data( ) |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

## Tile group header syntax

The syntax agreed as part of JVET-L0686 was used as the starting point.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if( pic_hdr_tile_group_header_flag ) |  |
| tile_group_header_override_flag | u(1) |
| if( !pic_hdr_tile_group_header_flag ¦ ¦ tile_group_header_override_flag ) |  |
| tile_group_header_param( tile_group_type ) |  |
| if( sps_alf_enabled_flag && !pps_alf_flag && !pic_hdr_alf_flag ) |  |
| alf_data( ) |  |
| byte_alignment( ) |  |
| } |  |

## Tile group header parameters syntax

The change marks are relative to JVET-L1001-v7.docx.

| tile_group_header_paramslice_header( codingType ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( codingTypeslice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if (codingTypeslice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if(codingTypeslice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if(codingTypeslice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).