| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0261 |
| --- | --- |

| Title: | AHG12: On grouping of tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Aminlou Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes the following options for meet the agreed design goal of "extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)":

- Indicating MCTSs in PPS. Tiles are present in the bitstream in their raster scan order within the picture.

- Indicating MCTSs in PPS. Tiles are present in the bitstream in their raster scan order within an MCTS, and MCTSs are present in the bitstream in raster scan order according to their top-left corner within the picture.

- Sub-picture-based partitioning and decoding. Each sub-picture may refer to its own PPS and may hence have its own tile partitioning. Sub-pictures are treated like pictures in the decoding process.

The authors prefer option 3, since it is asserted to support tile grids needed in some viewport-dependent 360° streaming schemes and be more efficient than MCTSs in terms of rate-distortion performance. Moreover, it is asserted that sub-picture-based operation would be beneficial in responding to viewing orientation changes in mixed-resolution viewport-dependent 360° streaming schemes, as described in JVET-M0259.

# Introduction

The handling of JVET-L0182 in the Macau meeting lead to an agreement to have the following design goal subject to having a solution that does not have a large impact on complexity or coding efficiency:

Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)

This contribution proposes a solution to meet the agreed design goal.

The contribution proposes the following partitioning units:

| tile | Primarily a unit for parallel processing in encoders and decoders |
| --- | --- |
| tile group | Exactly one VCL NAL unit, primarily a transmission unit, particularly for low-delay transmission (RTP and such). One or more tiles, as indicated in the tile group header. |
| tile set | One or more entire tile groups, forming a rectangular set of tiles. A unit for coded-domain repositioning and merging. Two ways to realize tile sets that can be repositioned and merged are proposed in this contribution: motion-constrained tile sets (MCTSs) and sub-pictures. |

It is believed that keeping these partitioning units as separate concepts is beneficial and keeps VVC flexible for different types of applications. It is noted that in some applications there can be a one-to-one mapping of two or more of the concepts, e.g. a tile set may contain exactly one tile group. The concepts of tile and tile group are essentially the same as already agreed in the Macau meeting.

Two options for realizing MCTSs are proposed in this contribution, differing in the tile scan order:

- Tile scan within a picture: tiles are present in the bitstream in tile raster scan order within the picture.

- Tile scan within an MCTS: tiles are present in the bitstream in tile raster scan order within an MCTS, and MCTSs are present in the bitstream in raster scan order according to their top-left corner.

# Summary of options

## MCTSs using tile scan within a picture

This design has the following main features:

- The definition and syntax of tile and tile groups are kept the same as agreed in Macau (in JVET-L0686).

- Tiles appear in the bitstream in tile raster scan order within a picture, i.e. the same order as agreed in Macau (in JVET-L0686).

- To enable extracting and/or relocating tile groups, a mapping of tile addresses to tile locations can be provided in the PPS. This is similar to what was proposed in JVET-L0114.

- Motion-constrained tile sets (MCTSs) can be indicated in the PPS by providing the top-left and bottom-right tile addresses. An MCTS consists of one or more tile groups that form a rectangular set of tiles. Encoding of an MCTS shall obey similar constraints as with HEVC MCTS. Loop filtering is turned off across MCTS boundaries.

## MCTSs using tile scan within a MCTS

This option is like the previous option, but tiles appear in the bitstream in raster scan order within an MCTS, and MCTSs appear in the bitstream in CTB raster scan order of their top-left corner within a picture. In other words, tile groups of an MCTS are contiguous in decoding order.

## Sub-picture-based picture partitioning and decoding

This design has the following main features:

- Pictures are divided into sub-pictures, tile groups and tiles.

- A sub-picture is a rectangular set of tile groups that starts with a tile group that has tile_group_address equal to 0.

- Each sub-picture may refer to its own PPS and may hence have its own tile partitioning.

- Sub-pictures are treated like pictures in the decoding process.

- The reference pictures for decoding the sub-picture are generated by extracting the area collocating with the current sub-picture from the reference pictures in the decoded picture buffer. The extracted area shall be a decoded sub-picture, i.e. inter prediction takes place between sub-pictures of the same size and the same location within the picture.

- A tile group is a sequence of tiles in tile raster scan of a sub-picture.

- The definition of a tile is unchanged. In other words, a tile is a sequence of CTUs that cover a rectangular region of a picture.

For example, Figure 1 illustrates a picture that is divided into 4 sub-pictures. The sub-picture decoding order is indicated with the number (1, 2, 3, 4) outside the picture boundaries. Each sub-picture may have its own tile grid. In this example sub-picture 1 is divided into a grid of 3×2 tiles of equal width and equal height, sub-picture 2 is divided into 2×1 tiles of 3 and 5 CTUs high. Each of sub-pictures 3 and 4 has only one tile.

In the example of Figure 1 sub-picture 1 has 3 tile groups containing 1, 3, and 2 tiles, respectively. Each of sub-pictures 2, 3, and 4 has one tile group.

Figure 1. A picture with 16 by 8 coding tree blocks that is partitioned into 4 sub-pictures, the first of which is partitioned to three tile groups and the others have one tile group.

# Comparison of options

## Summary

In all options, we kept the Macau decision to enable a flexible number of tiles in tile group that is indicated in the tile group header itself (with num_tiles_in_tile_group_minus1). With this design choice, the size of a VCL NAL unit in bytes can be roughly matched to a desired value by selecting the number of tiles contained in a tile group on the fly while encoding the content for a tile group as opposed to determining tile group sizes upfront when authoring the PPS. It would be possible to append the proposed syntax with a PPS flag indicating if an MCTS or sub-picture contains exactly one tile group only and if so, derive the number of tiles in a tile group rather than indicating it in the tile group header.

The following table summarizes the options in relation to selected design goals from JVET-L0182. The table refers to the term MCTS in the design goals as a generic feature.

|  | MCTSs with tile address mapping and tile scan within a picture | MCTSs with tile address mapping and tile scan within a MCTS | Sub-picture-based picture partitioning and decoding |
| --- | --- | --- | --- |
| Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications) | Yes, with PPS and SPS changes only, provided that tile_group_address is selected uniquely for all possible merging combinations among all the bitstreams from which MCTSs can be merged. When extracting VCL NAL units, they need to be ordered in tile scan order within a picture. | Yes, with PPS and SPS changes only, provided that tile_group_address is selected uniquely for all possible merging combinations among all the bitstreams from which MCTSs can be merged. | Yes, with PPS and SPS changes only. |
| Grid for MCTSs not crossing the entire picture. | No. | No. | Yes. |
| More efficient than MCTSs in terms of rate-distortion penalty | No. | No. | Yes, when motion vectors over sub-picture boundaries allowed. |

## On the design goals

All the design goals used in the summary table were discussed in JVET-L0182 and originally in JVET-K0300. For convenience, they are summarized below.

### Extraction and repositioning

"Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g., without VLC-level modifications)" was agreed as a design goal as noted in the disposition of JVET-L0182.

### Grid not crossing the entire picture

The need is illustrated with a viewport-dependent 360° scheme offering an increased effective spatial resolution on the viewport. Schemes where tiles covering the viewport originate from a 6K (6144×3072) ERP picture or an equivalent CMP resolution, illustrated in Figure 2, with "4K" decoding capacity (HEVC Level 5.1) were included in clauses D.6.3 and D.6.4 of OMAF [1] and also adopted in the VR Industry Forum Guidelines [2]. Such resolutions are asserted to be suitable for head-mounted displays using quad-HD (2560×1440) display panel.

Encoding: The content is encoded at two spatial resolutions with cube face size 1536×1536 and 768×768, respectively. In both bitstreams a 6×4 tile grid is used and an MCTS is coded for each tile position.

Selection of streamed MCTSs: 12 MCTSs from the high-resolution bitstream are selected and the complementary 12 MCTSs are extracted from the low-resolution bitstream. Thus, a hemi-sphere (180°×180°) of the streamed content originates from the high-resolution bitstream.

Merging MCTSs to a bitstream to be decoded: The received MCTSs of a single time instance are merged into a coded picture of 1920×4608, which conforms to HEVC Level 5.1. Another option for the merged picture is to have 4 tile columns of width 768, two tile columns of width 384, and three tile rows of height 768 luma samples, resulting into a picture of 3840×2304 luma samples.

a) coded bitstreams							b) MCTSs selected for streaming		c) Picture merged 
																	   from MCTSs

Figure 2. Example MCTS-based RWMR viewport-dependent 360° streaming.

It can be observed from the dashed red lines that the presented scheme requires the use of two slices within a tile. Since VVC does not contain slices, flexibility in specifying a grid not crossing the entire picture would be beneficial in this scheme.

### More efficient than MCTSs in terms of rate-distortion penalty

The more fine-grained the MCTS grid is, the closer the viewport can be matched with high-resolution or high-quality streamed MCTSs, while the remaining areas can be covered with lower resolution or quality. However, fine-granularity MCTS grids also bring a rate-distortion cost in inter prediction. A large part of the RD efficiency loss of MCTSs is reportedly caused by constraining the motion vectors so that they do not cause references to sample locations outside the MCTS.

# Specification text

## MCTSs with tile address mapping and tile scan within a picture

### Basis text

The proposed specification text has been authored on top of the agreed specification text of JVET-L0686.

Many parts of the presented syntax are copied with minor changes from JVET-L0114. Many thanks for Huawei (Ye-Kui Wang et al.) for authoring most parts of the specification text in JVET-L0114.

### Definitions

motion-constrained tile set: one or more tile groups that form a rectangular set of tiles constrained so that the no sample values or variables outside the collocated motion-constrained tile sets of the same size, if any, in the reference pictures are referenced in the decoding processes

### PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| tile_addr_len_minus1 | ue(v) |
| mcts_flag | u(1) |
| if( mcts_flag ) { |  |
| for( i = 0; i <= num_tile_rows_minus1; i++ ) |  |
| for( j = 0; j <= num_tile_columns_minus1; j++ ) |  |
| tile_addr_val[ i ][ j ] | u(v) |
| num_mcts_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_mcts_in_pic_minus1; i++ ) { |  |
| top_left_tile_addr[ i ] | u(v) |
| bottom_right_tile_addr[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### PPS semantics

All text below is proposed in this contribution.

tile_addr_len_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_addr_val[ i ][ j ], top_left_tile_addr[ i ], and bottom_right_tile_addr[ i ], when present, in the PPS and the syntax element tile_group_address in tile group headers referring to the PPS. The value of tile_addr_len_minus1 shall be in the range of Ceil( Log2( NumTilesInPic ) ) to 15, inclusive.

mcts_flag equal to 0 specifies that tile groups may or may not form rectangular sets of tiles and usage of sample values or variables that are outside the rectangular set of tiles, if any, may or may not be constrained. mcts_flag equal to 1 specifies that the picture is partitioned to motion-constrained tile sets.

tile_addr_val[ i ][ j ] specifies the tile_group_address value of the tile of the i-th tile row and the j-th tile column. The length of tile_addr_val[ i ][ j ] is tile_addr_len_minus1 + 1 bits.

For any integer m in the range of 0 to num_tile_columns_minus1, inclusive, and any integer n in the range of 0 to num_tile_rows_minus1, inclusive, tile_addr_val[ i ][ j ] shall not be equal to tile_addr_val[ m ][ n ] when i is not equal to m or j is not equal to n.

num_mcts_in_pic_minus1 plus 1 specifies the number of MCTSs in the picture.

top_left_tile_addr[ i ] specifies the tile_group_address value of the tile located at the top-left corner of the i-th MCTS. The length of top_left_tile_addr[ i ] is tile_addr_len_minus1 + 1 bits.

bottom_right_tile_addr[ i ] specifies the tile_group_address value of the tile located at the bottom-right corner of the i-th MCTS. The length of bottom_right_tile_addr[ i ] is tile_addr_len_minus1 + 1 bits.

Regardless of the value of loop_filter_across_tiles_enabled_flag in-loop filtering operations are not performed across tile boundaries that are also MCTS boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations.

### General tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| if( mcts_flag ) |  |
| tileIdx = TileAddrToIdx[ tile_group_address ] |  |
| else |  |
| tileIdx = tile_group_address |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

### Tile group header semantics

tile_group_address specifies the tile address of the first tile in the tile group. The length of tile_group_address is tile_addr_len_minus1 plus 1 Ceil( Log2 ( NumTilesInPic ) ) bits. If mcts_flag is equal to 0, the value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive. Otherwise (mcts_flag is equal to 1), the value of tile_group_address shall be in the range of 0 to 2tile_addr_len_minus1 + 1 – 1, inclusive. The value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0. The second and later tiles, if any, in decoding order within the tile group are inferred to have tile_group_address value that is incremented by 1 relative to the tile_group_address value of the previous tile, in decoding order, of the same tile group.

### Scanning process changes

The list TileAddr[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile_group_address value, is derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ ) {
		for( y = RowBd[ j ]; y < RowBd[ j + 1 ]; y++ )	(6-7)
			for( x = ColBd[ i ]; x < ColBd[ i + 1 ]; x++ )
				TileAddr[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] = 
					mcts_flag ? tile_addr_val[ i ][ j ] : tileIdx
		if( mcts_flag )
			TileAddrToIdx[ tile_addr_val[ i ][ j ] ] = tileIdx
	}

## MCTSs with tile address mapping and tile scan within a MCTS

### General

The specification text in Section 4.1 applies unless specifically otherwise indicated below.

### PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| tile_addr_len_minus1 | ue(v) |
| mcts_flag | u(1) |
| if( mcts_flag ) { |  |
| num_mcts_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_mcts_in_pic_minus1; i++ ) { |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| tile_addr_val[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### PPS semantics

All text below is proposed in this contribution.

tile_addr_len_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_addr_val[ i ], when present, in the PPS and the syntax element tile_group_address in tile group headers referring to the PPS. The value of tile_addr_len_minus1 shall be in the range of Ceil( Log2( NumTilesInPic ) ) to 15, inclusive.

mcts_flag equal to 0 specifies that tile groups may or may not form rectangular sets of tiles and usage of sample values or variables that are outside the rectangular set of tiles, if any, may or may not be constrained. mcts_flag equal to 1 specifies that the picture is partitioned to motion-constrained tile sets.

num_mcts_in_pic_minus1 plus 1 specifies the number of MCTSs in the picture.

top_left_tile_idx[ i ] specifies the tileIdx value of the tile located at the top-left corner of the i-th MCTS. The length of top_left_tile_idx[ i ] is Ceil( Log2 ( NumTilesInPic ) ) bits. The value of top_left_tile_idx[ i ] shall be in the range of 0 to NumTilesInPic − 1, inclusive.

bottom_right_tile_idx[ i ] specifies the tileIdx value of the tile located at the bottom-right corner of the i-th MCTS. The length of bottom_right_tile_idx[ i ] is Ceil( Log2 ( NumTilesInPic ) ) bits. The value of bottom_right_tile_idx[ i ] shall be in the range of 0 to NumTilesInPic − 1, inclusive.

tile_addr_val[ i ] specifies the tile_group_address value of the top-left tile of the i-th MCTS. The length of tile_addr_val[ i ] is tile_addr_len_minus1 + 1 bits.

The variable TileAddrVal[ i ][ j ] that specifies the indicated or inferred tile_group_address value of the i-th tile column and the j-th tile row is derived as follows:

for( k = 0; k <= num_mcts_in_pic_minus1; k++ ) {
	startRow = top_left_tile_idx[ k ] / ( num_tile_columns_minus1 + 1 )
	startCol = top_left_tile_idx[ k ] % ( num_tile_columns_minus1 + 1 )
	endRow = bottom_right_tile_idx[ k ] / ( num_tile_columns_minus1 + 1 )
	endCol = bottom_right_tile_idx[ k ] % ( num_tile_columns_minus1 + 1 )
	for( j = startRow, tileAddr = tile_addr_val[ k ]; j  <=  endRow; j++ )
		for( i = startCol; i  <=  endCol; i++, tileAddr++ )			TileAddrVal[ i ][ j ] = tileAddr
}

For any integer m in the range of 0 to num_tile_columns_minus1, inclusive, and any integer n in the range of 0 to num_tile_rows_minus1, inclusive, TileAddrVal[ i ][ j ] shall not be equal to TileAddrVal[ m ][ n ] when i is not equal to m or j is not equal to n.

Regardless of the value of loop_filter_across_tiles_enabled_flag in-loop filtering operations are not performed across tile boundaries that are also MCTS boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations.

### General tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| tileIdx = tile_group_address |  |
| tileAddr = tile_group_address |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileAddr++, tileIdx++ ) { |  |
| if( mcts_flag ) |  |
| tileIdx = TileAddrToIdx[ tileAddr ] |  |
| else |  |
| tileIdx = tileAddr |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

### Tile group header semantics

tile_group_address specifies the tile address of the first tile in the tile group. The length of tile_group_address is tile_addr_len_minus1 plus 1 Ceil( Log2 ( NumTilesInPic ) ) bits. If mcts_flag is equal to 0, the value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive. Otherwise (mcts_flag is equal to 1), the value of tile_group_address shall be in the range of 0 to 2tile_addr_len_minus1 + 1 – 1, inclusive. The value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture. When tile_group_address is not present it is inferred to be equal to 0. The second and later tiles, if any, in decoding order within the tile group are inferred to have tile_group_address value that is incremented by 1 relative to the tile_group_address value of the previous tile, in decoding order, of the same tile group. When mcts_flag is equal to 1 and the tile group does not contain the top-left tile of any MCTS, tile_group_address in the tile group header shall be equal to 1 plus the inferred tile_group_address value of the last tile of the previous tile group in decoding order.

### Scanning process changes

The list TileAddr[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile_group_address value, is derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ ) {
		for( y = RowBd[ j ]; y < RowBd[ j + 1 ]; y++ )	(6-7)
			for( x = ColBd[ i ]; x < ColBd[ i + 1 ]; x++ )
				TileAddr[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] = 
					mcts_flag ? TileAddrVal[ i ][ j ] : tileIdx
		if( mcts_flag )
			TileAddrToIdx[ TileAddrVal[ i ][ j ] ] = tileIdx
	}

# Sub-picture-based picture partitioning and decoding

## Picture parameter set

The sequence parameter set contains the width and height of the entire picture, which may contain multiple sub-pictures. Thus, it is proposed to add the indication of sub-picture width and height in the picture parameter set. This could be done in any of the following ways:

- Sub-picture width and height in CTUs. This option is presented below.

- If uniform tile spacing is in use, sub-picture width and height in CTUs. Otherwise, the width of the last tile column and the height of the last tile row could be added to the PPS (i.e., the loops would have one item more).

- Sub-picture width and height in units of minimum coding block size. This option would enable finer granularity for the last tile column and the last tile row.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| multiple_subpics_enabled_flag | u(1) |
| if( multiple_subpics_enabled_flag ) { |  |
| subpic_width_in_ctus_minus1 | ue(v) |
| subpic_height_in_ctus_minus1 | ue(v) |
| } |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

multiple_subpics_enabled_flag equal to 0 specifies that a picture contains exactly one sub-picture and that all VCL NAL units of an access unit reference the same active PPS. multiple_subpics_enabled_flag equal to 1 specifies that a picture may contain more than one sub-picture and each sub-picture may reference a different active PPS.

subpic_width_in_ctus_minus1 plus 1, when present, specifies the width of the sub-picture for which this PPS is the active PPS.

subpic_height_in_ctus_minus1 plus 1, when present, specifies the height of the sub-picture for which this PPS is the active PPS.

When subpic_width_in_ctus_minus1 and subpic_height_in_ctus_minus1 are present in a PPS that is activated, the following variables are derived and override the respective variables derived from the active SPS or any earlier active PPS:

PicWidthInCtbsY = subpic_width_in_ctus_minus1 + 1

PicHeightInCtbsY = subpic_height_in_ctus_minus1 + 1

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY

PicWidthInMinCbsY = PicWidthInCtbsY * CtbSizeY / MinCbSizeY

PicHeightInMinCbsY = PicHeightInCtbsY * CtbSizeY / MinCbSizeY

PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY

PicSizeInSamplesY = PicSizeInCtbsY * CtbSizeY * CtbSizeY

PicWidthInSamplesC = PicWidthInCtbsY * CtbSizeY / SubWidthC

PicHeightInSamplesC = PicHeightInCtbsY * CtbSizeY / SubHeightC

## Sub-picture location and reference pictures

When the decoding of a coded picture is started, each CTU location in the decoded picture is marked as unoccupied. For each sub-picture in decoding order, the following applies:

- The sub-picture takes the next such unoccupied location in CTU raster scan order within a picture that is large enough to fit the sub-picture within the picture boundaries.

- Each active reference picture in reference picture lists 0 and 1 shall have a collocated sub-picture of the same size. The reference pictures for decoding the sub-picture are generated by extracting the area collocating with the sub-picture from the reference pictures in the decoded picture buffer.

## Handling of motion vectors over sub-picture boundaries

There are two options for handling motion vectors over sub-picture boundaries:

- References to sample locations outside sub-picture boundaries are saturated in the decoding process. I.e., effectively padding of boundary samples is applied.

- References to sample locations outside sub-picture boundaries are required to be avoided in the encoding.

Option 2 imposes similar constraints as MCTSs proposed above and HEVC MCTSs. Option 1 handles sub-picture boundaries like picture boundaries in the decoding process and thus has better rate-distortion performance than option 2. The proponents therefore prefer option 1.

# References

- MPEG N17563, "Second revised text of ISO/IEC FDIS 23090-2," April 2018.

- Virtual Reality Industry Forum Guidelines, draft version 1.1, May 2018. Online: https://www.vr-if.org/guidelines/, accessed 2018-06-24.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).