| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0262 |
| --- | --- |

| Title: | CE2: Affine model inheritance from single-line motion vectors (Test 2.4.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE2.4.8. In CE2.4.8, it is proposed to set the base MV to be the MV of the neighbouring 4×4 block when conducting inside CUT-row affine inheritance, to avoid storing the top-left corner position of each CU. Simulation results reportedly show 0.00% BD-rate changing under Random Access (RA) configuration.

# Introduction

At the Macau meeting, JVET-L0046/JVET-L0047/JVET-L0694 were adopted to cleanup the affine motion vector storage and reduce the extra memory required by the affine inheritance. With the current design in VTM-3.0, affine inheritance is conducted in two different ways, depending on whether affine inheritance crosses the CTU rows or not.

## Affine motion derivation

With the affine model, a MV (mvh, mvv) at the position (x, y) can be derived as

(1)

with the 4-parameter affine model, or

(2)

with the 6-parameter affine model, wherein MVbase (mvhbase, mvvbase) is a base MV at a base position (xbase, ybase), and (a, b), (a, b, c, d) represent affine parameters for the 4-parameter affine model and 6-paramter affine model, respectively. The affine parameters can be calculated as:

, 									(3)

, , 								(4)

,									(5)

where MV0 (mv0h, mv0v), MV1 (mv1h, mv1v) and MV2 (mv2h, mv2v), represent three Control Point MVs (CPMVs) at position (x0, y0), (x1, y1) and (x2, y2), respectively. (x0, y0), (x1, y1) and (x2, y2) are usually set to be the top-left, top-right and bottom-left corner of an affine coded block with size equal to w×h, so Lx is set to be w and Ly is set to be h.

It should be noted that the base MV is not necessary to be one of the CPMVs, although it is set to be the CPMV MV0 at the top-left corner (x0, y0) in VTM-3.0.

## Affine inheritance crossing CTU rows

Fig. 1 Affine inheritance crossing CTU rows.

When the affine model of the current CU is inherited from a neighbouring 4×4 block in the above CTU row, MVs stored in the regular MV buffer are accessed. Fig. 1 shows an example when the current CU applies affine merge mode, inheriting the affine model from a neighbouring block B0. In this case, the codec needs to store the width of the CU covering B0 (Wn in Fig. 1), MVs of the bottom-left 4×4 block (N0) and bottom-right 4×4 block (N1) of the CU (MVN0 and MVN1 in Fig. 1), and the base position which is the bottom-right coordinator (xN0, yN0) of N0. In this example, MVN0 is used as the base MV.

## Affine inheritance inside a CTU row

When the affine model of the current CU is inherited from a neighbouring 4×4 block in the current CTU row, CPMVs stored in a special CPMV buffer are accessed. The current CU can inherit the 4-parameter or the 6-parameter affine model from the neighbouring CU.

## Problems of affine inheritance inside CTU-row

For a 4×4 block inside a CTU, three CPMVs for two reference lists, the width, the height and the top-left coordinate of the CU containing the 4×4 block are required to be stored.

# Proposed methods

This contribution only addresses the affine inheritance inside a CTU row. When the current CU inherits the affine model from a neighbouring 4×4 block B, the MV of B is set as the base MV and the center position of B is set to be the base position. The top-left coordinate of a CU no longer needs to be stored. For each 8×8 block, 10 bits (or 2 bytes in a byte-alignment implementation) memory can be saved.

# Experimental Results

The proposed methods have been implemented on top of VTM-3.0. Results of RA configuration are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for Test 2.4.8

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | 0.04% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.02% | -0.07% | 0.01% | 100% | 100% |  |  |  |  |  |
| Class B | -0.01% | 0.05% | 0.01% | 100% | 99% | 0.02% | -0.20% | 0.27% | 100% | 102% |
| Class C | 0.03% | 0.00% | 0.03% | 100% | 99% | 0.06% | 0.45% | 0.05% | 100% | 106% |
| Class E |  |  |  |  |  | 0.00% | 0.34% | -0.68% | 100% | 102% |
| Overall | 0.00% | -0.02% | 0.02% | 100% | 99% | 0.03% | 0.15% | -0.04% | 100% | 103% |
| Class D | -0.01% | -0.14% | 0.00% | 100% | 99% | 0.13% | -0.07% | 0.23% | 99% | 97% |
| Class F | 0.00% | -0.05% | -0.03% | 99% | 99% | 0.13% | 0.66% | 0.17% | 99% | 99% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).