| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0266 |
| --- | --- |

| Title: | CE2-related: History-based affine merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, history-based affine merge candidates are proposed to be inserted into the sub-block-based affine merge candidate list. After decoding an affine-coded CU, the affine parameters are added to a history table. When constructing the sub-block prediction affine merge candidate list, the affine parameters stored in the history table are combined with MVs of neighbouring blocks to generate the history-based affine merge candidates. Simulation results reportedly show -0.13% BD-rate changing under Random Access (RA) configuration with almost no impact on encoding and decoding time.

# Introduction

History-based Motion Vector Prediction (HMVP) has been adopted in VTM-3.0. HMVP candidates from a HMVP table may be put into the regular merge candidate list. After decoding an inter-coded CU, the motion information is pushed into a history table.

Besides the regular merge mode, a sub-block-based merge mode has also been adopted in VTM-3.0. A sub-block-based merge candidate list may include ATMVP merge candidate, affine merge candidate inherited from neighbouring blocks, affine merge candidates constructed by spatial/temporal neighbouring blocks, and zero candidates in order. However, HMVP is not used to construct the sub-block-based merge candidate list in VTM-3.0.

# Proposed methods

It is proposed to add History-based Affine Merge Candidates (HAMC) into the sub-block-based merge candidate list. After decoding an affine-coded CU, a set of affine parameters {a, b, c, d} for the two lists and the associated reference indices are put into an affine parameter history table.

An HAMC is derived by combining a set of affine parameters stored in the table and the MV of a neighbouring 4×4 block served as the base MV. In a formulating way, the MV of the current block at position (x, y) is calculated as:

,

where (mvhbase, mvvbase) represents the MV of a neighbouring 4×4 block, (xbase, ybase) represents the center position of the neighbouring 4×4 block. (x, y) can be the top-left, top-right and bottom-left corner of the current block to obtain the CPMVs.

HAMCs derived from stored affine parameters and MVs from spatial neighbouring blocks are put into the sub-block-based after the first constructed affine merge candidate. For each set of stored affine parameters, the first valid neighbouring 4×4 block with the same inter-prediction direction and reference indices as those associated with the set of affine parameters is used to derive the HAMC.

HAMCs derived from stored affine parameters and MVs from temporal neighbouring block are put into the sub-block-based before the zero candidate. For each set of stored affine parameters, the TMVP is scaled to the reference picture the parameters referring to, to derive the HAMC.

In this contribution, each parameter is stored as an 8-bit signed integer. Up to 6 affine parameter sets are stored. Therefore, the affine parameter history table is quite small, i.e., only 6×(8×4×2+8)=432bits (42 bytes).

# Experimental Results

The proposed method has been implemented on top of VTM-3.0. Results of RA configuration are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for HAMC

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.20% | -0.11% | 100% | 99% |
| Class A2 | -0.22% | -0.11% | -0.14% | 100% | 101% |
| Class B | -0.13% | -0.04% | -0.18% | 100% | 100% |
| Class C | -0.06% | -0.02% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.08% | -0.09% | 100% | 100% |
| Class D | -0.09% | -0.19% | -0.15% | 99% | 98% |
| Class F | -0.04% | -0.07% | -0.07% | 97% | 99% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).