| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0275_v3 |
| --- | --- |

| Title: | Non-CE6 : On transform skip conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu Seoul 06772, South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this document, it is proposed to use the number of pixels for transform skip enable condition instead of width and height of the current block so that transform skip mode can be more generally applied. Specifically, it shows benefits when transform skip enabled size is extended. The experimental result is reportedly shown that Y BD-rate changes in current CTC condition (i.e., equivalent to applying transform skip up to 16 pixels) are 0.00%, 0.00%, 0.02% in AI, RA, LDB, respectively. However, when the transform skip is allowed up to 64 pixels, it is reportedly shown that Y BD-rate changes are -0.04%, -0.07%, -0.07% in AI, RA, LDB, respectively. Note that if the transform skip is extended to larger block sizes (i.e., 8x8), more block types can be enabled using the proposed scheme such as 4x16 and 16x4 in luma component.

Problem statement

In the current VVC standard [1], the transform skip can be enabled when the block width is smaller or equal to 4 and the block height is smaller or equal to 4, respectively. However, this restricts the blocks for transform skip even though the area of the block is the same. For example, in the current spec, block size 2x8 or 8x2 is not allowed in spite of the same area as 4x4 blocks. Since the current standard has opportunity to have variety of block shapes, square and non-square blocks, the proposed method can encourage elongated blocks to be coded with transform-skipped.

Test results

The proposed method has been implemented using VTM-3.0 [2] as software base and experimentally evaluated according to the common test conditions [3], in addition to SCC sequences used in CE8. For tests results in Table 4, Table 5, (and LDB Table), VTM-3.0 with transform skip size extended to 8x8 is used as an anchor.

Table 1 Transform skip with proposed method (All intra)

Table 2 Transform skip with proposed method (Random access)

Table 3 Transform skip with proposed method (Low delay B)

Table 4 Transform skip extended to 8x8 with proposed method (All intra)

Table 5 Transform skip extended to 8x8 with proposed method (Random access)

Table 6 Transform skip extended to 8x8 with proposed method (Low delay B)

Table 7 Transform skip extended to 8x8 with proposed method (All intra), anchor : JVET-M0269 (Extended TS to 8x8)

Table 8 Transform skip extended to 8x8 with proposed method (Random access), anchor : JVET-M0269 (Extended TS to 8x8)

Table 9 Transform skip extended to 8x8 with proposed method (Low delay B), anchor : JVET-M0269 (Extended TS to 8x8)

Proposed spec

- 7.3.4.12 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( transform_skip_enabled_flag && ( cIdx ! = 0 ¦ ¦ tu_mts_flag[ x0 ][ y0 ] = = 0 ) && ( log2TbWidth <= 2 ) && ( log2TbHeight <= 2 ) ( log2TbWidth + log2TbHeight <= 4) ) |  |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| … |  |

Conclusion

In this document, it is proposed that the transform skip is considered using the number of pixels of the current block. The proposed scheme allows thin or fat blocks which have the same area with square to be coded with transform-skipped, which is desirable to the current standard. Moreover, if the condition of the transform skip size is increased, more block size types can be enabled. Therefore, it is recommended that the condition of the transform skip is modified with the number of pixels of blocks, not each size of the width and height.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).