| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0276 |
| --- | --- |

| Title: | CE10-related: MPM list alignment between CIIP and intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li, Shanshe Wang, Wen Gao (Peking University) Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu (Bytedance) | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | Peking University and Bytedance Inc. |  |  |

_____________________________

# Abstract

Combined Inter-Intra mode Prediction (CIIP) utilizes a new MPM list construction process to select intra prediction mode which is different from that developed for intra mode coding. In this contribution, it is proposed to reuse the MPM list construction process for intra mode for CIIP. Only the first four or first one mode of the intra MPM list are allowed for coding a CIIP coded block. Simulation results reportedly show that the unification of MPM list achieves 0.02% and 0.01% BD rate savings for RA Main10 with four and one allowed modes, respectively.

# Introduction

Combined inter-intra mode prediction is adopted in the 3rd VVC meeting [1]. In CIIP, a different MPM list from intra mode is constructed and the intra mode is derived by indexing the constructed MPM list. More specifically, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes, and the size of the MPM candidate list can be 3 or 4 depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is exclusive of the intra mode list and when the CU height is larger than the double of CU width, vertical mode is removed from the intra mode list.

The MPM list for CIIP is constructed based on neighboring intra blocks and combined prediction blocks. The original intra modes from these neighboring blocks are firstly derived. If the original intra modes are not one of planar, DC, vertical, and horizontal modes, then these modes are rounded to the closest modes of planar, DC, vertical, and horizontal. Such a strategy require computations for MPM list construction and introduces block dependency.

# Proposed methods

Instead of constructing the additional MPM list in the CIIP process, we take fully use of the existing intra MPM lists, and then, the first K intra mode candidates are selected. As such, the selection modes can be easily signaled by MPM index with truncated unary code. With one candidate (i.e., K = 1), there is no need to signal MPM index. In this contribution, we provide two sets of simulation results with K equal to 4 and 1, respectively.

In particular, for the newly introduced angular intra modes (horizontal and vertical modes are excluded), equal weights are applied.

# Experimental Results

The proposed method is implemented on VTM-3.0 [2]. The simulations were performed following the JVET CTC [3]. Results of K set to 4 and 1 under Random Access (RA) configuration and Low-delay B (LB) configuration are summarized in the Table 1 and Table 2, respectively. More detailed results can be found in the attached spreadsheets.

Table 1. Results of the proposed method compared to VTM-3.0 (K = 4)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.02% | 100% | 101% |
| Class A2 | -0.05% | -0.16% | -0.05% | 101% | 101% |
| Class B | -0.02% | 0.02% | -0.05% | 101% | 103% |
| Class C | 0.01% | 0.04% | 0.07% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.01% | 100% | 102% |
| Class D | -0.03% | 0.09% | -0.05% | 100% | 100% |
| Class F | 0.01% | -0.07% | 0.01% | 100% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.29% | 0.06% | 100% | 100% |
| Class C | 0.02% | 0.30% | 0.15% | 100% | 99% |
| Class E | -0.12% | -0.39% | -0.10% | 100% | 99% |
| Overall | -0.02% | -0.12% | 0.05% | 100% | 100% |
| Class D | 0.01% | 0.34% | -0.01% | 100% | 101% |
| Class F | 0.10% | 0.11% | 0.51% | 100% | 98% |

Table 2. Results of the proposed method compared to VTM-3.0 (K = 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.00% | 99% | 100% |
| Class A2 | -0.02% | 0.03% | 0.11% | 100% | 101% |
| Class B | -0.03% | 0.11% | 0.05% | 100% | 102% |
| Class C | 0.04% | 0.05% | 0.10% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.06% | 0.07% | 99% | 101% |
| Class D | 0.02% | -0.02% | -0.04% | 99% | 100% |
| Class F | 0.01% | 0.02% | 0.07% | 99% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.13% | 0.25% | 99% | 100% |
| Class C | 0.11% | 0.46% | 0.36% | 99% | 99% |
| Class E | 0.08% | 0.62% | 0.16% | 99% | 99% |
| Overall | 0.10% | 0.36% | 0.26% | 99% | 100% |
| Class D | 0.06% | 0.00% | 0.73% | 100% | 100% |
| Class F | 0.11% | 0.14% | 0.19% | 98% | 98% |

# Conclusions

This contribution proposes a simplified CIIP method wherein the logic of MPM list for intra mode is reused for CIIP. One or four intra modes derived from the intra MPM list are allowed for CCIP mode. It is recommended to consider adoption of the proposed method to the next version of VTM.

# Reference

- “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode”, Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei, Doc. JVET-L0100, Oct. 2018.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).