| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MO, 9–18 Jan. 2019 | Document: JVET-M0281 v1 |
| --- | --- |

| Title: | CE4.1.5: Inter motion predictor pruning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, F. Galpin, T. Poirier | Email: | antoine.robert@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes an uniformization of AMVP motion predictor list construction and an early and simple pruning of the Merge pair-wise candidates to avoid redundancy in the lists of predictors. The proposed process has been implemented in the JVET VTM3.0. The reported simulation results show an average luma BD-rate gain of -0.02% in RA configuration and of -0.02% in LDB configuration. The encoding and decoding times stay identical to the VTM-3.0 ones. The worth case complexity in terms of integer comparisons required by the pruning is increased by 24 comparisons in addition to the 174 actual ones in Merge mode but some pair-wise candidate calculations are discarded.

# Introduction

In the current AMVP and Merge motion predictor lists’ constructions, it is possible to encounter the same predictor several times. This reduces the achievable performances.

At the last meeting in Macao, several modifications concerning pruning have been adopted:

- In AMVP, when performing the AMVR rounding, the ¼-pel rounding should be performed at the same time, and a pruning process has been added for the temporal candidate.

- In Merge, a simpler pruning process of the spatial candidates has been adopted as well as it has been removed for the temporal candidate.

This contribution proposes a harmonization of the rounding processes in AMVP and a simple motion predictor pruning process for Merge pair-wise candidates that allow reducing the redundancy in the lists of predictors. It is also possible to clean and speed-up these predictor lists’ constructions.

# Description

In AMVP, a motion predictor is a motion vector dedicated to a particular reference picture, and a list must contain 2 motion predictors. The process is repeated for each reference frame of each reference frame list.

It has been adopted that when performing the AMVR rounding, the ¼-pel rounding should be performed at the same time. This rounding step takes place just before the existing pruning. But for motion predictors that do not use AMVR, the needed ¼-pel rounding is performed at the end of the list construction.

This contribution then proposes to perform all the rounding operations before pruning even if AMVR is off, i.e. either ¼-pel or AMVR and ¼-pel rounding is done before any pruning.

Figure 1 represents the candidates considered during the AMVP list creation, their number over the list size, and the worth case complexity analysis where the number of motion vectors involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector).

The associated worth case complexity in terms of integer comparisons is not changed, it stays at 16 comparisons per reference frame.

Figure 1: Proposed AMVP list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

In Merge mode, a motion predictor is one or two motion vector(s) with its associated reference picture, and a list must contain 6 motion predictors.

The pruning process of the spatial candidates has been simplified and the one of the temporal candidate has been removed. But, for pair-wise candidates, no pruning process exists.

This contribution then proposes to perform an early and simple pruning of the pair-wise candidates. For each used pair (A, B), the pair-wise candidate is not calculated:

- If both A and B are bi-directional and both motion vectors are equal,

- If both A and B are uni-directional or only A is bi-directional and motion vectors of the common list are equal,

- If only B is bi-directional and motion vectors and reference frame of the common list are equal.

Figure 2 represents the candidates considered during the Merge list creation, their number over the list size, and the worth case complexity analysis where the number of motion vectors and associated reference frame indexes involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector and 1 per reference frame index).

The associated worth case complexity in terms of integer comparisons is increased by only 24 comparisons on top of the 174 actual ones (+12%), but it allows discarding some pair-wise candidate calculations.

Figure 2: Proposed Merge list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

## CE4.1.5 test a): AMVP

Table 1 and Table 2 show the simulation results of the proposed harmonization of the AMVP predictor list construction in random access and low delay B configurations. In both configurations, the proposed harmonization allows keeping the BD-rate performances with the same complexity (in encoding/decoding time and in number of integer comparisons requested by pruning).

Table 1: Simulation results of the proposed harmonization of the AMVP motion predictor list in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.10% | -0.01% | 100% | 100% |
| Class B | -0.01% | 0.02% | -0.01% | 100% | 101% |
| Class C | -0.01% | 0.06% | -0.02% | 99% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.02% | 0.07% | -0.04% | 99% | 97% |
| Class F (optional) | 0.00% | -0.02% | -0.02% | 100% | 102% |

Table 2: Simulation results of the proposed harmonization of the AMVP motion predictor list in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | -0.24% | -0.03% | 99% | 100% |
| Class C | -0.03% | 0.28% | 0.06% | 100% | 100% |
| Class E | -0.01% | -0.15% | -0.06% | 99% | 100% |
| Overall | -0.01% | -0.04% | -0.01% | 100% | 100% |
| Class D | -0.04% | -0.14% | -0.20% | 99% | 100% |
| Class F (optional) | -0.02% | -0.12% | -0.13% | 100% | 98% |

## CE4.1.5 test b): Merge

Table 3 and Table 4 show the simulation results of the proposed pruning process of the Merge pair-wise candidates in random access and low delay B configurations. In both configurations, the proposed pruning process allows improving the BD-rate performances with the same complexity in encoding/decoding time and with a slightly increased number of integer comparisons requested by pruning.

Table 3: Simulation results of the proposed pruning of the Merge pair-wise candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.01% | 100% | 100% |
| Class A2 | -0.03% | 0.07% | 0.09% | 99% | 99% |
| Class B | -0.03% | 0.01% | -0.01% | 100% | 101% |
| Class C | -0.02% | -0.01% | -0.06% | 99% | 99% |
| Overall | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.08% | -0.05% | 99% | 98% |
| Class F (optional) | 0.00% | 0.01% | 0.04% | 100% | 100% |

Table 4: Simulation results of the proposed pruning of the Merge pair-wise candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.04% | -0.12% | -0.05% | 99% | 101% |
| Class C | 0.05% | 0.19% | 0.04% | 100% | 100% |
| Class E | -0.04% | 0.28% | -0.31% | 99% | 99% |
| Overall | 0.02% | 0.09% | -0.09% | 99% | 100% |
| Class D | -0.03% | -0.07% | 0.49% | 99% | 99% |
| Class F (optional) | -0.09% | -0.36% | -0.41% | 99% | 100% |

## CE4.1.5 test c): combined AMVP and Merge

Table 5 and Table 6 show the simulation results of the combination of AMVP and Merge modifications of motion predictor list constructions in random access and low delay B configurations. In both configurations, the proposed modifications allow improving the BD-rate performances for all sequence classes with the same complexity in encoding/decoding time and with a slightly increased number of integer comparisons requested by pruning in Merge.

Table 5: Simulation results of the proposed AMVP and Merge modifications of predictor lists in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.00% | 100% | 99% |
| Class A2 | -0.02% | -0.03% | 0.01% | 100% | 99% |
| Class B | -0.02% | 0.01% | -0.03% | 100% | 102% |
| Class C | -0.03% | -0.03% | -0.01% | 99% | 100% |
| Overall | -0.02% | -0.03% | -0.01% | 100% | 100% |
| Class D | -0.01% | -0.03% | -0.14% | 99% | 98% |
| Class F (optional) | -0.01% | 0.00% | -0.01% | 100% | 101% |

Table 6: Simulation results of the proposed AMVP and Merge modifications of predictor lists in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.02% | -0.07% | -0.06% | 100% | 101% |
| Class C | 0.00% | 0.21% | 0.41% | 99% | 100% |
| Class E | -0.13% | 0.05% | -0.18% | 99% | 99% |
| Overall | -0.02% | 0.06% | 0.07% | 99% | 100% |
| Class D | -0.06% | 0.52% | 0.25% | 99% | 100% |
| Class F (optional) | -0.07% | -0.08% | -0.20% | 100% | 99% |

# Conclusion

This contribution proposes an uniformization of the rounding in the AMVP motion predictor list construction and an early and simple pruning of the Merge pair-wise candidates associated with some cleaning. Simulation results reportedly show that the proposed method can bring an average luma BD-rate gain of -0.02% for RA and of -0.02% for LDB. The complexity in encoding and decoding time stays the same for RA and LDB. The worth case complexity in terms of integer comparisons required by the pruning is increased by 24 comparisons in addition to the 174 actual ones in Merge mode but some pair-wise candidate calculations are discarded.
It is suggested that the uniformization and motion predictor pruning proposed in this contribution should be adopted into the reference test model.

# References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting, Macao, CN, October 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1024, 12th Meeting, Macao, CN, October 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).