| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MO, 9–18 Jan. 2019 | Document: JVET-M0283 v1 |
| --- | --- |

| Title: | CE10-related: Reduction of the motion predictor pruning in Triangle Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, F. Galpin, T. Poirier | Email: | antoine.robert@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution describes two simple motion predictor pruning processes to be applied in Triangle Merge modes. The proposed processes have been implemented in the JVET VTM3.0. For the first version of pruning reduction, the reported simulation results keep the BD-rate performances in RA and LDB configurations. Then, for the second and more drastic reduction, the reported simulation results show an average luma BD-rate loss of 0.03% in RA configuration and of 0.14% in LDB configuration. The encoding and decoding times stay identical to the VTM-3.0 ones with both versions. The worth case complexity in terms of integer comparisons required by the pruning is decreased from 366 to 204 comparisons for the first version (-44%) and to only 36 comparisons for the second version (-90%).
Combined with the proposals of JVET-M0281 CE4.1.5 and JVET-M0282 CE2.2.8 that performs motion predictor pruning for AMVP and Merge modes and for Affine AMVP and Affine Merge modes respectively, the reported simulation results show, with the first version, an average luma BD-rate gain of -0.04% in RA configuration and of -0.04% in LDB configuration and, with the second version, the same BD-rate performances for RA configuration and an average luma BD-rate loss of 0.08% in LDB configuration. The encoding and decoding times also stay identical to the VTM-3.0 ones with both versions. The total worth case complexity in terms of integer comparisons required by the pruning in all coding modes is reduced, at decoding, from 366 to 204 comparisons with the first version (-44%) and to 198 comparisons with the second version (-46%), and, at encoding, from 856 to 828 or 660 comparisons with the two versions respectively (-3% or -23%).

# Introduction

In the current Triangle Merge motion predictor lists’ constructions, it is not possible to encounter the same predictor several times since a full pruning process is applied.

This contribution proposes a harmonization and some simpler motion predictor pruning processes for the Triangle Merge mode that allow keeping the redundancy in the lists of predictors low.

# Description

In Triangle Merge mode, a motion predictor is one motion vector with its associated reference picture, and a list must contain 5 motion predictors.

The motion predictors are uni-directional parts picked in a pre-list of candidates composed of spatial and temporal candidates. A full pruning process is applied when collecting the first spatial candidates. Then, when constructing the motion predictor list, a full pruning is also performed where motion vectors and POC of reference frame are compared.

This contribution then proposes to:

- Harmonize first spatial candidates pruning as in Merge mode since these candidates are the same as in Merge,

- Perform a simple pruning of the first temporal candidates together,

- Remove the pruning of uni-directional candidates since the pre-list is already (partially) pruned,

- Remove the pruning of averaged candidates.

In a second, more drastic, version where all prunings of the motion predictor list construction are removed, it is, in addition, proposed to:

- Remove the pruning of L0 parts of bi-directional candidates,

- Remove the pruning of L1 parts of bi-directional candidates.

Figure 1 represents the candidates considered during the Triangle Merge list creation, their number over the list size, and the worth case complexity analysis where the number of motion vectors and associated reference frame indexes involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector and 1 per reference frame index).

The associated worth case complexity in terms of integer comparisons is reduced by:

- 162 comparisons and goes from 366 to 204 comparisons, for the first version (-44%),

- 330 comparisons and goes from 366 to only 36 comparisons, for the second version (-90%).

Figure 1: Proposed Triangle Merge list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

This proposal will be further combined with the proposals of JVET-M0281 CE4.5.1 [3] and JVET-M0282 CE2.2.8 [4].

CE4.5.1 [3] proposes a harmonization of the rounding processes in AMVP and an early and simple motion predictor pruning process for Merge pair-wise candidates. The associated worth case complexity in terms of integer comparisons is:

- Not changed for AMVP, it stays at 16 comparisons per reference frame,

- Increased by only 24 comparisons in top of the 174 existing ones for Merge (+12%), but it allows discarding some pair-wise candidate calculations.

Figure 2 represents the candidates considered during the AMVP and Merge list creations, their number over the corresponding list size, and the worth case complexity analysis where the number of motion vectors and associated reference frame indexes for Merge involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector and 1 per reference frame index).

Figure 2: Proposed CE4.1.5 AMVP and Merge list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

CE2.2.8 [4] also suggests a harmonization and some simple motion predictor pruning processes for Affine AMVP and Affine Merge modes. The worth case complexity in terms of integer comparisons is:

- Increased to 18 comparisons per reference frame for Affine AMVP (compared to 16 in AMVP),

- Increased by only 14 comparisons on top of the 78 existing ones in Affine Merge (+15%).

Figure 3 represents the candidates considered during the Affine AMVP and Affine Merge list creations, their number over the corresponding list size, and the worth case complexity analysis where the number of motion vectors and associated reference frame indexes for Affine Merge involved in the pruning of motion predictors are counted to get the total number of integer comparisons (2 per motion vector and 1 per reference frame index).

Figure 3: Proposed CE2.2.8 Affine AMVP and Affine Merge list construction modification (and complexity in comparisons)
[black: VTM-3.0 – orange: proposed modification]

The full worth case complexity analysis in terms of integer comparisons of these combined proposals show that:

- When using the first version of the pruning reduction in Triangle Merge, the total number of comparisons is reduced from 366 to 204 comparisons for a decoder (-44%), and is stable even reduced by 28 comparisons (-3%) for an encoder that explores all the coding modes,

- When using the second version of the pruning reduction in Triangle Merge, the total number of comparisons is reduced from 366 to 198 comparisons for a decoder (-46%), and is reduced from 856 to 660 comparisons (-23%) for an encoder that explores all the coding modes.

Figure 4: Complexity analysis in integer comparisons

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

## First Triangle Merge pruning reduction

Table 1 and Table 2 show the simulation results of the first version of the proposed pruning reduction in Triangle Merge mode in random access and low delay B configurations. In both configurations, the proposed pruning reduction allows keeping the BD-rate performances with the same complexity in encoding/decoding time but with the number of integer comparisons requested by pruning nearly divided by two (204 comparisons instead of 366).

Table 1: Simulation results of the first proposed pruning reduction in the Triangle Merge mode in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.07% | 0.03% | 100% | 100% |
| Class A2 | -0.01% | -0.03% | 0.03% | 100% | 99% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.03% | 0.08% | 99% | 98% |
| Overall | 0.00% | 0.02% | 0.03% | 100% | 100% |
| Class D | 0.02% | 0.01% | 0.15% | 100% | 98% |
| Class F (optional) | 0.00% | 0.01% | 0.00% | 100% | 100% |

Table 2: Simulation results of the first proposed pruning reduction in the Triangle Merge mode in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.02% | -0.10% | 0.18% | 100% | 100% |
| Class C | 0.03% | 0.13% | 0.04% | 100% | 100% |
| Class E | -0.04% | 0.60% | -0.18% | 99% | 99% |
| Overall | 0.01% | 0.15% | 0.04% | 100% | 100% |
| Class D | -0.01% | 0.32% | 0.26% | 99% | 100% |
| Class F (optional) | 0.07% | 0.15% | 0.00% | 100% | 100% |

## Second Triangle Merge pruning reduction

Table 3 and Table 4 show the simulation results of the second version of the proposed pruning reduction in Triangle Merge mode in random access and low delay B configurations. In both configurations, the proposed pruning reduction slightly decreases the BD-rate performances with the same complexity in encoding/decoding time but with the number of integer comparisons requested by pruning nearly removed (36 comparisons instead of 366).

Table 3: Simulation results of the second proposed pruning reduction in the Triangle Merge mode in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.06% | 100% | 101% |
| Class A2 | 0.00% | 0.06% | 0.10% | 100% | 99% |
| Class B | 0.01% | 0.06% | -0.02% | 100% | 101% |
| Class C | 0.06% | 0.10% | 0.09% | 99% | 99% |
| Overall | 0.03% | 0.06% | 0.05% | 100% | 100% |
| Class D | 0.08% | 0.01% | 0.07% | 99% | 98% |
| Class F (optional) | 0.02% | 0.05% | 0.01% | 100% | 100% |

Table 4: Simulation results of the second proposed pruning reduction in the Triangle Merge mode in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.09% | 0.10% | 0.33% | 100% | 101% |
| Class C | 0.14% | 0.27% | 0.37% | 100% | 100% |
| Class E | 0.22% | 0.67% | 0.36% | 99% | 99% |
| Overall | 0.14% | 0.29% | 0.35% | 100% | 100% |
| Class D | 0.11% | 0.01% | 0.18% | 100% | 100% |
| Class F (optional) | 0.07% | 0.22% | 0.53% | 99% | 99% |

## Combination with CE4.1.5 and CE2.2.8

Table 5 and Table 6 show the simulation results in random access and low delay B configurations of the combination of the proposal with CE4.1.5 test c) of JVET-M0281 [3] and CE2.2.8 test c) of JVET-M0282 [4]. In both configurations, the proposed modifications allow improving the BD-rate performances for all sequence classes with the same complexity in encoding/decoding time and with a slightly reduced number of integer comparisons requested by pruning (204 instead of 366 for a decoder, and 828 instead of 856 for an encoder).

Table 5: Simulation results of the first proposed pruning reduction in the Triangle Merge mode
combined with CE4.5.1 and CE2.2.8 in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.12% | 0.01% | 101% | 99% |
| Class A2 | -0.05% | 0.00% | 0.13% | 100% | 99% |
| Class B | -0.03% | 0.08% | -0.03% | 101% | 101% |
| Class C | -0.04% | -0.06% | 0.09% | 100% | 100% |
| Overall | -0.04% | -0.01% | 0.04% | 101% | 100% |
| Class D | -0.05% | -0.21% | -0.05% | 99% | 97% |
| Class F (optional) | 0.03% | 0.04% | 0.06% | 102% | 102% |

Table 6: Simulation results of the first proposed pruning reduction in the Triangle Merge mode
combined with CE4.5.1 and CE2.2.8 in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.03% | -0.16% | 0.23% | 100% | 101% |
| Class C | -0.03% | 0.44% | -0.02% | 100% | 101% |
| Class E | -0.07% | 0.17% | 0.06% | 99% | 99% |
| Overall | -0.04% | 0.12% | 0.11% | 100% | 100% |
| Class D | -0.10% | 0.20% | 0.48% | 100% | 99% |
| Class F (optional) | 0.02% | 0.17% | -0.60% | 100% | 99% |

Table 7 and Table 8 show the simulation results in random access and low delay B configurations of the combination of the proposal with CE4.1.5 test c) of JVET-M0281 [3] and CE2.2.8 test c) of JVET-M0282 [4]. In both configurations, the proposed modifications allow keeping the BD-rate performances with the same complexity in encoding/decoding time and with a reduced number of integer comparisons requested by pruning (198 instead of 366 for a decoder, and 660 instead of 856 for an encoder).

Table 7: Simulation results of the second proposed pruning reduction in the Triangle Merge mode
combined with CE4.5.1 and CE2.2.8 in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 100% | 100% |
| Class A2 | -0.02% | 0.10% | 0.06% | 100% | 99% |
| Class B | -0.01% | 0.00% | -0.13% | 101% | 101% |
| Class C | 0.03% | 0.12% | 0.08% | 100% | 100% |
| Overall | 0.00% | 0.06% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.01% | 99% | 98% |
| Class F (optional) | 0.06% | 0.06% | 0.05% | 101% | 102% |

Table 8: Simulation results of the second proposed pruning reduction in the Triangle Merge mode
combined with CE4.5.1 and CE2.2.8 in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.05% | -0.22% | 0.05% | 100% | 101% |
| Class C | 0.10% | 0.40% | 0.23% | 100% | 100% |
| Class E | 0.10% | 0.32% | -0.20% | 100% | 100% |
| Overall | 0.08% | 0.12% | 0.05% | 100% | 100% |
| Class D | 0.02% | 0.51% | 0.77% | 100% | 99% |
| Class F (optional) | 0.12% | 0.43% | 1.55% | 100% | 99% |

# Conclusion

This contribution proposes a reduction of the pruning in the Triangle Merge motion predictor list constructions with an associated cleaning. Two versions are provided, one dividing by nearly 2 the number of integer comparisons needed by the pruning, and another keeping only 10% of the comparisons. Simulation results reportedly show that the first proposed version can keep the average BD-rate performances for RA and LDB, and that the second proposed version can slightly decrease the average luma BD-rate by 0.03% for RA and by 0.14% for LDB. The complexity in encoding and decoding time stays the same for RA and LDB for both versions. The worth case complexity in terms of integer comparisons required by the pruning is decreased from 366 to 204 comparisons for the first version (-44%), and even more decreased to only 36 comparisons for the second version (-90%).
Combined with the proposals of JVET-M0281 CE4.1.5 and JVET-M0282 CE2.2.8 that performs motion predictor pruning for AMVP and Merge modes and for Affine AMVP and Affine Merge modes respectively, simulation results reportedly show, with the first version of pruning reduction, an average luma BD-rate gain of -0.04% for RA and -0.04% for LDB and, with the second version of pruning reduction, the same BD-rate performances for RA and an average luma BD-rate loss of 0.08% for LDB. Simulations also show that the complexity in encoding and decoding time stays the same for RA and LDB for both versions. The total worth case complexity in terms of integer comparisons required by the pruning in all coding modes is reduced, at decoding, from 366 to 204 comparisons with the first version (-44%) and to 198 comparisons with the second version (-46%), and, at encoding, from 856 to 828 or 660 comparisons with the two versions respectively (-3% or -23%).
It is suggested that the full proposed combination of motion predictor pruning that brings the same BD-rate performances as VTM-3.0 with a reduced worth case complexity in terms of integer comparisons needed by the pruning (half at decoder and one quarter at encoder) should be adopted into the reference test model.

# References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting, Macao, CN, October 2018.

[2] C. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1030, 12th Meeting, Macao, CN, October 2018.

[3] A. Robert, F. Le Léannec, F. Galpin, T. Poirier, “CE4.5.1: Inter motion predictor pruning”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0281, 13th Meeting, Marrakesh, MO, January 2019.

[4] A. Robert, F. Le Léannec, F. Galpin, T. Poirier, “CE2.2.8: Affine motion predictor pruning”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0282, 13th Meeting, Marrakesh, MO, January 2019.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).