| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18Jan. 2019 | Document: JVET-M0291 |
| --- | --- |

| Title: | CE 4: Extension on MMVD (Test 4.2.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents method of MMVD extension based on VTM-3.0. Limited area MV offset are selected to be candidates. For solution 1, it has 0.07%/0.03% lost for RA/LB compared to VTM-3.0 anchor with around 5% decoding time saving. For solution 2, it has 0.02% lost for RA and 0.07%gain for LBrespectively compared to VTM-3.0 anchor with around 3% decoding time saving.

Introduction

MMVD re-uses merge candidate. Among the merge candidates, two candidate can be selected, and choice one to bestarting point,MVD is expressed as motion magnitude and motion direction.

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

Table 1. Distance IDX

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent of the four directions as shown below.

Table 2. Direction IDX

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

Proposed improvements

- Extension on MMVD

- Solution 1

For the merge candidates, three candidate can be selected, and choice one to be the starting point.

Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

Table 3. Distance IDX

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel |

- Solution 2

For solution 2, 4 new diagonal directionMV offsets for each distance is induced as figure 1 shows.Distance of diagonal offsets are half of vertical and horizontal direction distance of exited offsets.

Figure 1New offset candidates for each distance(red ones)

Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

Table 3. Distance IDX

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel |

.

For solution 2, Bug of MMVD flag signalling (When MMVD mode is selected, no need to transfer triangle flag or Intra-Inter combined mode flag)is fixed.

Experimental Results

The proposed method has been implemented on VTM-3.0[1], and allthe tests were conducted under the common testconditions[2]. The experimental results of themerge offset extensioncompared to VTM-3.0 are shown in Table1.

Table 1. Experimental results of Solution 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22% | 0.17% | 0.30% | 99% | 98% |
| Class A2 | 0.09% | 0.25% | 0.25% | 99% | 97% |
| Class B | 0.02% | 0.19% | 0.03% | 99% | 93% |
| Class C | 0.02% | 0.09% | 0.08% | 102% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.17% | 0.14% | 100% | 95% |
| Class D | -0.05% | -0.19% | 0.20% | 101% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | 0.06% | 0.31% | 100% | 95% |
| Class C | 0.00% | 0.11% | 0.29% | 101% | 98% |
| Class E | 0.13% | 0.26% | 0.04% | 101% | 99% |
| Overall | 0.03% | 0.13% | 0.24% | 100% | 97% |
| Class D | 0.00% | 0.46% | 0.09% | 101% | 99% |

Table1 shows the gain for solution 1.

Table 2. Experimental results of Solution 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.28% | 0.31% | 0.30% | 98% | 98% |
| Class A2 | 0.08% | 0.32% | 0.26% | 99% | 98% |
| Class B | -0.03% | 0.14% | 0.03% | 100% | 98% |
| Class C | -0.14% | 0.01% | 0.06% | 103% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.17% | 0.14% | 100% | 97% |
| Class D | -0.23% | -0.13% | 0.05% | 104% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.08% | -0.03% | 0.23% | 99% | 91% |
| Class C | -0.10% | -0.03% | 0.12% | 102% | 98% |
| Class E | 0.00% | 0.49% | -0.17% | 101% | 96% |
| Overall | -0.07% | 0.10% | 0.10% | 100% | 94% |
| Class D | -0.23% | -0.48% | 0.13% | 101% | 98% |

Table2 shows the gain for solution 2.

Reference

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/VVCSoftware_VTM.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, China, Oct. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).