| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0292 |
| --- | --- |

| Title: | CE6: Reduced Secondary Transform (RST) (test 6.5.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution, reports test results of CE 6-5.1 on reduced secondary transform (RST). According to the request of the previous 12th JVET meeting, all tests were performed without normative change of MTS signaling, which are described as the following:

- Test 1:  (A),

- Test 2:  (A) + (B),

- Test 3:  (A) + (B) + (C),

- Test 4:  (A) + (B) + (D),  where

| Feature | Description |
| --- | --- |
| (A) | 4 transform sets (instead of 35). 2 transforms per set. |
| (B) | Restricted RST to allow maximum 8 multiplications/sample |
| (C) | RST is disabled for 4x4 TU. |
| (D) | 16x48 matrices are employed instead of 16x64 ones. |

The experimental results of all CE 6-5.1 tests are summarized as follows:

As Test 1 results, with 128% (AI), 108% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate changes are -1.59% (AI), -0.88% (RA), and -0.24% (LD).

As Test 2 results, with 128% (AI), 108% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate changes are -1.40% (AI), -0.76% (RA), and -0.21% (LD).

As Test 3 results, with 125% (AI), 107% (RA), and 103% (LD) encoding time compared to VTM anchor, BD-rate changes are -1.25% (AI), -0.69% (RA), and -0.18% (LD).

As Test 4 results, with 129% (AI), 107% (RA), and 104% (LD) encoding time compared to VTM anchor, BD-rate changes are -1.34% (AI), -0.71% (RA), and -0.18% (LD).

The memory usage of Test 1, Test 2, and Test 3 accounts to 10KB and that of Test 4 to 8KB.

Background on Secondary Transform

In JEM, secondary transform is applied between forward primary transform and quantization (at encoder) and between de-quantization and inverse primary transform (at decoder side). As shown in Figure 1, 4x4 (or 8x8) secondary transform is performed depends on block size. For example, 4x4 secondary transform is applied for small blocks (i.e., min (width, height) < 8) and 8x8 secondary transform is applied for larger blocks (i.e., min (width, height) > 4) per 8x8 block.

Figure 1  Secondary transform in JEM

Application of a non-separable transform is described as follows using input as an example. To apply the non-separable transform, the 4x4 input block X

X=X00X01X10X11X02X03X12X13X20X21X30X31X22X23X32X33		     	            	(1)

is first represented as a vector X:

X=X00X01X02X03X10X11X12X13X20X21X22X23X30X31X32X33T	(16)

The non-separable transform is calculated as F=T∙X, where F indicates the transform coefficient vector, and T is a 16x16 transform matrix. The 16x1 coefficient vector F is subsequently re-organized as 4x4 block using the scanning order for that block (horizontal, vertical or diagonal). The coefficients with smaller index will be placed with the smaller scanning index in the 4x4 coefficient block. There are totally 35 transform sets and 3 non-separable transform matrices (kernels) per transform set are used. The mapping from the intra prediction mode to the transform set is pre-defined. For each transform set, the selected non-separable secondary transform candidate is further specified by the explicitly signalled secondary transform index. The index is signalled in a bit-stream once per Intra CU after transform coefficients.

Proposed Method

Reduced Secondary Transform (RST)

The RST was introduced in JVET-K0099 [1] and 4 transform set (instead of 35 transform sets) mapping introduced in JVET-L0133 [2], which is still used in CE6-5.1. In this contribution, 16x64 (or 16x48 for Test 4) and 16x16 matrices are employed for 8x8 and 4x4 blocks, respectively. For notational convenience, the 16x64 (or 16x48) transform is denoted as RST8x8 and the 16x16 one as RST4x4.

Figure 2  Proposed Reduced Secondary Transform (RST)

RST computation

The main idea of a Reduced Transform (RT) is to map an N dimensional vector to an R dimensional vector in a different space, where R/N (R < N) is the reduction factor.

The RT matrix is an R×N matrix as follows:

where the R rows of the transform are R bases of the N dimensional space. The inverse transform matrix for RT is the transpose of its forward transform. The forward and inverse RT are depicted in Figure 3.

Figure 3 Forward and inverse Reduced Transform

In this contribution, the RST8x8 with a reduction factor of 4 (1/4 size) is applied. Hence, instead of 64x64, which is conventional 8x8 non-separable transform matrix size, 16x64 direct matrix is used. In other words, the 64×16 inverse RST matrix is used at the decoder side to generate core (primary) transform coefficients in 8×8 top-left regions. The forward RST8x8 uses 16×64 (or 8x64 for 8x8 block) matrices so that it produces non-zero coefficients only in the top-left 4×4 region within the given 8×8 region. In other words, if RST is applied then the 8×8 region except the top-left 4×4 region will have only zero coefficients. For RST4x4, 16x16 (or 8x16 for 4x4 block) direct matrix multiplication is applied.

An inverse RST is conditionally applied when the following two conditions are satisfied:

- Block size is greater than or equal to the given threshold (W>=4 && H>=4)

- Transform skip mode flag is equal to zero

If both width (W) and height (H) of a transform coefficient block is greater than 4, then the RST8x8 is applied to the top-left 8×8 region of the transform coefficient block. Otherwise, the RST4x4 is applied on the top-left min(8, W) × min(8, H) region of the transform coefficient block.

If RST index is equal to 0, RST is not applied. Otherwise, RST is applied, of which kernel is chosen with the RST index. The RST selection method and coding of the RST index are explained in 1.1.2) and 1.4), respectively.

Furthermore, RST is applied for intra CU in both intra and inter slices, and for both Luma and Chroma. If a dual tree is enabled, RST indices for Luma and Chroma are signaled separately. For inter slice (the dual tree is disabled), a single RST index is signaled and used for both Luma and Chroma.

RST selection

A RST matrix is chosen from four transform sets, each of which consists of two transforms. Which transform set is applied is determined from intra prediction mode as the following:

- If one of three CCLM modes is indicated, transform set 0 is selected.

- Otherwise, transform set selection is performed according to the following table:

Table 1  The transform set selection table

| IntraPredMode | Tr. set index |
| --- | --- |
| IntraPredMode < 0 | 1 |
| 0 <= IntraPredMode <= 1 | 0 |
| 2 <= IntraPredMode <= 12 | 1 |
| 13 <= IntraPredMode <= 23 | 2 |
| 24 <= IntraPredMode <= 44 | 3 |
| 45 <= IntraPredMode <= 55 | 2 |
| 56 <= IntraPredMode | 1 |

The index to access Table 1, IntraPredMode, have a range of [-14, 83], which is a transformed mode index used for wide angle intra prediction.

Simplification method ( Test 2 )

The same simplification method as proposed in [2] is applied in this contribution, which limits worst case number of multiplications per sample less than or equal to 8.

If we use RST8x8 and RST4x4, worst case in terms of multiplication count occurs when all TUs consist of 4x4 TU or 8x8 TU. Therefore, top 8x64 and 8x16 matrices (in other words, first 8 transform basis vectors from the top in each matrix) are applied to 8x8 TU and 4x4 TU, respectively.

In the case of blocks larger than 8x8 TU, worst case does not occur so that RST8x8 (i.e. 16x64 matrix) is applied to top-left 8x8 region. For 8x4 TU or 4x8 TU, RST4x4 (i.e. 16x16 matrix) is applied to only top-left 4x4 region excluding the other 4x4 regions in order to avoid worst case happening. In the case of 4xN or Nx4 TU (N ≥ 16), RST4x4 is applied to two adjacent top-left 4x4 blocks each.

With the aforementioned simplification, the worst case number of multiplications becomes 8 per sample.

RST matrices of reduced dimension ( Test 4 )

In Test 4, 16x48 matrices are applied instead of 16x64 with the same transform set configuration, each of which takes 48 input data from three 4x4 blocks in a top-left 8x8 block excluding right-bottom 4x4 block (Figure 4). With the help of the reduced dimension, memory usage for storing all RST matrices is reduced from 10KB to 8KB with reasonable performance drop.

Figure 4  An example of forward RST8x8 process with 16x48 matrix

RST Signalling

The forward RST8x8 with R =16 uses 16×64 matrices so that it produces non-zero coefficients only in the top-left 4×4 region within the given 8×8 region. In other words, if RST is applied then the 8×8 region except the top-left 4×4 region generates only zero coefficients. As a result, RST index is not coded when any non-zero element is detected within 8x8 block region other than top-left 4×4 (which is depicted in Figure 5) because it implies that RST was not applied. In such a case, RST index is inferred to be zero.

Figure 5  Scanning the position 17 to 64 for non-zero element

Experimental results

The proposed method has been implemented based on VTM 3.0 reference software according to common test conditions defined in JVET-L1010 [3] and Core Experiment description in JVET-L1026 [4].

Test 1: (A)

(A) 4 transform sets and 2 transforms per set

Results with inter MTS off (CTC)

Inter MTS on

Test 2: (A) + (B)

(A) 4 transform sets and 2 transforms per set

(B) Simplification of max. 8 multiplications/pixel

Results with inter MTS off  (CTC)

Results with inter MTS on

Test 3: (A) + (B) + (C)

(A) 4 transform sets and 2 transforms per set

(B) Simplification of max. 8 multiplications/pixel

(C) RST disabled for 4x4 TU

Results with inter MTS off  (CTC)

Results with inter MTS on

Test 4: (A) + (B) + (D)

(A) 4 transform sets and 2 transforms per set

(B) Simplification of max. 8 multiplications/pixel

(D) 16x48 matrices

Results with inter MTS off  (CTC)

Complexity metric

| Item | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Operation counts | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 16 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 768 (M), 720(A), 48(S) Worst Case multiplication per sample: 8 |
| Memory requirements | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*48*8 /(8*1024)= 0.75 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(0.75+ 0.25) = 8 KB |
| Bit-precision to represent transform coefficients | 8 bits | 8 bits | 8 bits | 8 bits |
| Precision of arithmetic operations during transform computation | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | No | No | No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM |
| List of all combinations of transforms block size and transform type used for secondary transformation | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: Half RST4x4 for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: No RST for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: Half RST4x4 for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | N/A | N/A | N/A | N/A |

Conclusion

In the proposed RST, it is reportedly observed that BD-rate/encoding time/decoding time was -1.59%/128%/98% (AI), -0.88%/108%/100% (RA), and -0.24%/104%/101% (LD) for 4 transform set configuration of two RST candidates per set without normative change of MTS signaling. Also, -1.40%/128%/97% (AI), -0.76%/108%/100% (RA), and -0.21%/104%/98% (LD) were achieved for simplified configuration restricting maximum number of multiplications per pixel on top of the former one. As reported, substantial BD-rate is achieved with reasonable memory cost and encoding time increase. It is strongly recommended that the proposed RST should be adopted in the next VTM.

Reference

- M. Salehifar et al., “CE 6.2.6: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0099, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- M. Koo et al., “CE 6-2.1: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0133, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-L1026, the 12th JVET meeting, Oct. 2018, Macao, CN.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).