| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0293 |
| --- | --- |

| Title: | CE10: Simplification on Combined Inter-Intra Prediction with Size Restriction (Test 10.1.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiwei Xu Biao Wang Haitao Yang Jianle Chen Man-Shu Chiang Chih-Wei Hsu Yuwen Huang Shawmin Lei | Tel: Email: | +8613758193753 xuweiwei3@huawei.com biao.wang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com man-shu.chiang@mediatek.com cw.hsu@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | Huawei Technologies. Co., Ltd; MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal simplifies the Combined Inter-Intra Prediction (CIIP) adopted in Macao meeting from three aspects. First, the four intra modes is reduced to only Planar mode. Second, the Planar prediction process is performed by skipping PDPC. Third, the number of weights is reduced. In addition, the proposed simplifications are combined with the test in CE10.1.4, where the size of blocks applied with combined inter-intra prediction is restricted no larger than 1024 samples. Two Tests CE10.1.5.a and CE10.1.5.b are performed: CE10.1.5.a uses only Planar mode in CIIP, with equal weights, and applies the size restriction; CE10.1.5.b uses only Planar mode in CIIP, skip the PDPC step, and applies the size restriction. In addition, CE10.1.5.a applies equal weights for all configurations while CE10.1.5.b applies equal weights except for low delay P, where (2, 6) weights are used with 2 applied for intra.

It is reported that, compared to VTM3.0, the luma BD-rate of proposed CE10.1.5.a are 0.02% in RA, and 0.11% in LDB with encoding time slightly decreased and decoding time remains the same; the luma BD-rate of proposed CE10.1.5.b are 0.06% in RA, and 0.07% in LDB with encoding and decoding time remains the same, respectively.

# Introduction

Two simplifications for the combined Inter-Intra prediction are tested.

## CE10.1.5.a: combined inter-intra prediction with only planar mode for intra and limitation of big CU usage

When the combined inter-intra prediction is applied, the final prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select the usage of intra planar mode when the flag is true. For chroma component, DM is always applied without extra signaling. When the cu size is bigger than 1024, combined inter-intra prediction will not be used.

## CE10.1.5.b: Combined Inter-intra prediction with PDPC skipping for intra prediction and limitation of big CU usage

When the combined inter-intra prediction is applied, the final prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select the usage of intra planar mode when the flag is true. For chroma component, DM is always applied without extra signaling. All intra modes use unified weights of (4:4) for the whole CU except for low delay configuration, where (2:6) weight is used with weighting factor of intra prediction as 2. When the cu size is bigger than 1024, combined inter-intra prediction will not be used.

# Simulation results

The contribution has been implemented on top of VTM3.0. The test condition descripted in JVET-L1010 [1] is used for verifying the contribution. The summary results are provided in the following tables.

## CE10.1.5.a

On average, the BD-rate differences are 0.02% in RA, and 0.11% in LDB, respectively.

Table 1 Coding performance over VTM3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.00% | 0.06% | 97% | 100% |
| Class A2 | 0.00% | 0.22% | 0.20% | 98% | 100% |
| Class B | -0.01% | 0.24% | 0.04% | 98% | 100% |
| Class C | 0.02% | 0.13% | 0.20% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.16% | 0.12% | 98% | 100% |
| Class D | -0.01% | -0.13% | 0.04% | 99% | 100% |
| Class F | 0.00% | 0.05% | 0.06% | 98% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.16% | 0.55% | 98% | 100% |
| Class C | 0.10% | 0.45% | 0.45% | 99% | 100% |
| Class E | 0.11% | 0.06% | -0.35% | 98% | 101% |
| Overall | 0.11% | 0.23% | 0.29% | 98% | 100% |
| Class D | 0.05% | 0.22% | 0.10% | 99% | 100% |
| Class F | 0.14% | -0.01% | 0.11% | 98% | 99% |

## CE10.1.5.b

On average, the BD-rate differences are 0.06% in RA, and 0.07% in LDB, respectively.

Table 2 Coding performance over VTM3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.07% | 0.00% | 98% | 100% |
| Class A2 | 0.04% | 0.02% | 0.11% | 98% | 100% |
| Class B | 0.03% | 0.10% | -0.05% | 98% | 100% |
| Class C | 0.08% | 0.08% | 0.11% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.07% | 0.03% | 98% | 100% |
| Class D | 0.03% | -0.10% | 0.13% | 99% | 100% |
| Class F | -0.02% | -0.05% | 0.01% | 98% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.06% | 0.14% | 98% | 99% |
| Class C | 0.12% | 0.45% | 0.35% | 99% | 100% |
| Class E | -0.01% | 0.08% | -0.58% | 98% | 100% |
| Overall | 0.07% | 0.15% | 0.03% | 98% | 99% |
| Class D | 0.08% | 0.15% | 0.42% | 99% | 99% |
| Class F | -0.24% | 0.32% | -0.19% | 98% | 98% |

# Conclusions

This contribution reports the results of two simplifications for combined inter-intra prediction. Compared to VTM3.0 anchor, the average BD-rate differences are 0.02%~0.06% in RA, and 0.07%~0.11% in LDB.

We recommend to accept the proposed methods in VVC VTM as the simplification well justifies the negligible coding loss.

# Reference

[1]“JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET-L1010, Nov. 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc.may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).