| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, Morocco, 9–18 Jan. 2019 | Document: JVET-M0294 |
| --- | --- |

| Title: | CE10-related: modification for blocks applied with combined inter-intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Anand Meher Kotra, Semih Esenlik, Han Gao, Jianle Chen | Tel: Email: | biao.wang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes two modifications for blocks applied with combined inter-intra prediction (for simplification, these blocks are named as combined prediction blocks). The first modification occurs to MPM (Most Probable Mode) list construction, where combined prediction blocks are considered as inter blocks always. The second modification is applied for deblocking filter, where combined prediction blocks are considered as intra blocks in boundary strength derivation. Two solutions are proposed: the first solution applies modification only for MPM list construction, the second solution applies both modification of MPM list construction and boundary strength derivation. The BD-rate results over the VTM3.0 anchor are:

- -0.01% (RA), 0.01% (LDB), -0.04% (LDP) for luma component with the first solution

- 0.00% (RA), 0.00% (LDB), -0.07% (LDP) for luma component with the second solution

AI results are the same to the anchor as modifications have no effect on I frames, wherein no combined prediction blocks exist.

# Introduction and Background

In VTM3.0, two different MPM list constructions are used for combined prediction blocks and intra blocks. When constructing MPM list for combined prediction blocks, if their neighboring blocks are combined prediction blocks, then these neighboring combined prediction blocks’ intra modes are used to construct the MPM list. However, when constructing MPM list for intra blocks, if their neighboring blocks are combined prediction blocks, they are marked as not available, and no intra mode of combined prediction blocks will be used for MPM list construction of intra blocks.

When neighboring combined prediction blocks are used for MPM list construction, combined prediction blocks need to store intra mode for the current block’s MPM list construction. Addition memory would be required to store intra mode for combined prediction blocks.

In addition to MPM list construction, the boundary strength derivation in VTM3.0 is not adapted for combined prediction blocks wherein intra prediction is performed. As intra prediction is introduced in combined prediction blocks, residual data might be evident and introduce block artifact. In fact, a statistic for all bitstreams encoded as specified in Common Test Configurations (CTC) [1] shows that non-zero coefficients exists as long as a block is a combined prediction block. To reduce the block artifact introduced by combined prediction blocks, the boundary strength derivation in the deblocking filter might need to be adapted.

# Proposed Methods

In this proposal, two modification are performed for combined prediction blocks, one occurs to MPM list construction, and the other boundary strength derivation.

For the modification of MPM list construction, when constructing MPM list for combined prediction blocks, if their neighboring blocks are combined prediction blocks, then these neighboring combined prediction blocks are marked as unavailable, and no intra modes from these neighboring combined prediction blocks will be used for MPM list construction. In contrast, these neighboring combined prediction blocks’ intra modes are used to construct the MPM list in VTM3.0. One advantage of the proposed design is that no extra memory is required for combined prediction blocks to store intra modes. In VTM3.0, storing intra modes is necessary for combined prediction blocks because those intra mode information is used for its neighboring combined prediction blocks’ MPM list construction.

With respect to deblocking filter, the boundary strength derivation process is simplified for combined prediction blocks. As long as one of the two associated blocks (P and Q) is a combined prediction block, the corresponding boundary strength is set to 2. The boundary strength derivation process is modified as shown in Figure 1. In contrast, the boundary strength for combined prediction blocks in VTM 3.0 is determined based on residual information (coefficients), reference picture indices, and motion vector etc. since combined prediction blocks are not intra blocks. The derivation workflow in VTM 3.0 is depicted as shown in Figure 2.

Figure 1: adapted boundary strength derivation, MHIntra indicates a block is a combined prediction block.

Figure 2: boundary strength derivation in VVC 3.0.

The modification for boundary strength derivation might increase the requirement for line memory buffer, as a flag (indicating whether a block is a combined prediction block or not, such as MHIntra flag in Figure 1) needs to be stored for blocks above the current CTU when the horizontal boundaries across CTUs. In order to avoid using additional line memory, when a horizontal boundary across CTUs, only the Q block’ flag is checked for Bs derivation, as shown in Figure 3.

P

CTU row boundary

Current CU

Q

Only block Q’s flag is checked

Figure 3: only Q block is checked when a horizontal boundary across CTUs for saving line memory in Bs derivation

# Experimental Results

Two tests are performed to evaluate the proposed approaches:

- Test1: The first simulation reflects the effect when only the modification for MPM list construction is used.

- Test2: The second simulation shows results when both modification for MPM list construction and deblocking filter are applied.

Both simulations use VTM3.0 as the anchor and the Common Test Conditions [1] are used.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.03% | 100% | 100% |
| Class A2 | -0.02% | 0.02% | 0.09% | 100% | 100% |
| Class B | -0.01% | 0.02% | -0.01% | 100% | 100% |
| Class C | 0.00% | -0.02% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.01% | -0.05% | 0.02% | 100% | 100% |
| Class F | 0.02% | -0.01% | -0.02% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.25% | -0.11% | 100% | 99% |
| Class C | 0.02% | 0.25% | 0.01% | 100% | 99% |
| Class E | 0.01% | 0.11% | -0.21% | 100% | 101% |
| Overall | 0.01% | 0.01% | -0.10% | 100% | 99% |
| Class D | -0.03% | 0.17% | 0.14% | 100% | 100% |
| Class F | -0.01% | 0.08% | -0.05% | 100% | 100% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.31% | 0.19% | 100% | 98% |
| Class C | -0.02% | 0.13% | -0.10% | 100% | 100% |
| Class E | -0.11% | -0.04% | -0.91% | 100% | 101% |
| Overall | -0.04% | 0.16% | -0.18% | 100% | 99% |
| Class D | 0.00% | -0.42% | 0.04% | 100% | 99% |
| Class F | 0.06% | -0.48% | -0.43% | 100% | 100% |

Table 1: RA, LDB, and LDP results when modification is only applied for MPM list construction. No results for AI as proposal has no effect on I frames.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.21% | -0.34% | 101% | 100% |
| Class A2 | -0.01% | -0.06% | 0.02% | 100% | 101% |
| Class B | 0.00% | -0.05% | -0.30% | 101% | 102% |
| Class C | 0.02% | -0.15% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.11% | -0.20% | 100% | 101% |
| Class D | 0.01% | -0.05% | -0.19% | 100% | 101% |
| Class F | 0.02% | -0.11% | -0.07% | 100% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.62% | -0.37% | 100% | 101% |
| Class C | -0.02% | -0.07% | -0.17% | 100% | 101% |
| Class E | -0.01% | -0.11% | -0.30% | 100% | 103% |
| Overall | 0.00% | -0.31% | -0.29% | 100% | 101% |
| Class D | 0.02% | -0.11% | -0.10% | 100% | 101% |
| Class F | -0.02% | 0.09% | 0.05% | 100% | 102% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.09% | -0.47% | 100% | 100% |
| Class C | -0.05% | -0.20% | -0.47% | 100% | 101% |
| Class E | -0.17% | -0.23% | -1.41% | 100% | 102% |
| Overall | -0.07% | -0.16% | -0.70% | 100% | 101% |
| Class D | -0.03% | -0.74% | -0.51% | 100% | 101% |
| Class F | 0.09% | -0.47% | 0.13% | 100% | 101% |

Table 2: RA, LDB, and LDP results when both modification for MPM list construction and deblocking filter are applied. No results for AI as proposal has no effect on I frames

# Conclusion

The proposal reports the results of adaptions for blocks with combined inter-intra prediction. The test of adaption in MPM list construction shows no coding loss and can save memory storage for blocks with combined inter-intra prediction. The test of adaption in both MPM list construction and deblcoking filter shall reduces the block artifacts and shows obvious coding gain in chroma components.

# Reference

[1]“JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET-L1010, Nov. 2018.

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu doc. JVET-K1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-K1023, September 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).