| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0299-v1 |
| --- | --- |

| Title: | CE11: Deblocking for 4 x N, N x 4 and 8 x N and N x 8 block boundaries that not are aligned with 8x8 grid (test 11.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Zhi Zhang, Rickard Sjöberg, | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
|  | Anand Meher Kotra, Jianle Chen, Semih Esenlik, Biao Wang, Han Gao | Tel: Email: | Anand.meher.kotra@huawei.com jianle.chen@huawei.com |
|  | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Ericsson, Huawei Technologies. Co., Ltd, MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution document reports the results of the core experiment CE11.2.1, which is a joint proposal from Ericsson, Huawei and MediaTek Inc. VTM-3.0 [2] does not apply deblocking filter for some edges belonging to the blocks whose size is Nx4, 4xN blocks and also for some blocks whose size is Nx8, 8xN where N value can be upto 64 samples. This test applies deblocking on 4x4 grid to not only allow deblocking of Nx4, 4xN, Nx8 and 8xN blocks aligned with the current 8x8 sample grid but also for the Nx4, 4xN, Nx8, 8xN blocks that are not aligned on the current 8x8 sample grid. The number of samples to read and modify during deblocking filtering is limited to allow for parallel friendly processing. For a given edge, if at least one of the blocks sharing the edge has a size of 4 samples (orthogonally) then the VTM-3.0 weak deblocking filter is chosen to be applied. Furthermore, the weak filter is constrained to only modify one sample on either side of the edge.

Subjective analysis shows that when compared to the Anchor (VTM-3.0), the proposed test improves the subjective quality of sequences.

Objective results show luma BD-Rate gain with negligible run-time changes.

Over VTM 3.0 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of -0.05%, -0.23%, -0.21%, -0.36% is achieved with negligible run-time changes.

Over VTM 3.0 Anchor with ALF ON (AI, RA, LDB, LDP): Luma BD-Rate of 0.02%, -0.12%, -0.14%, -0.18% is achieved with negligible run-time changes.

# Introduction

VTM 3.0 performs deblocking on an 8 x 8 grid. Therefore deblocking is only performed for all the CU and TU edges which overlap with the 8 x 8 grid. Due to the VTM 3.0 partitioning scheme (BinaryTree (BT) and Triple Tree (TT)), there may be some long CU edges which may not overlap with the 8 x 8 grid but still have sufficient blocking artifacts. The scenario is depicted in the Fig. 1 below. The edge shown in red color belongs to a 4 x 16 block and current VTM 3.0 deblocking does not perform deblocking on this edge, as it does not overlap with an 8 x 8 sample grid.

Figure 1. VTM 3.0 performs deblocking on 8 x 8 sample grid. Few 4 x N edges are not filtered

# Proposed Method

As shown in Fig. 2, proposed method enforces the Luma weak filter with one or no sample modification, when any of the blocks sharing the edge has a size of 4 samples. (i.e. width = 4 samples for vertical edge, height = 4 samples for horizontal edge).

The different steps performed for a vertical edge are illustrated below:

- Check if filter on/off condition is evaluated to true i.e. d < Beta, where d = d0 + d3 and d0 = dp0 + dq0 and d3= dp3 + dq3) is evaluated to true for the respective edge. If true then

- Check if the one of the blocks has a block width of 4 samples, if true, then

- Enforce weak filter with maximum one sample modification. Therefore the following condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8)≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

Figure 2.  Weak filter with one sample modification for 4xN, Nx4 and 8xN, Nx8 blocks.

# Complexity Evaluation

Complexity of the proposal is evaluated based on the following criteria described in the CE11 document, JVET-L1031 [1].

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - For blocks size >=8 samples: it is the same as in VTM-3.0. Except when the edge is shared with a block whose size is 4 samples. Then only one sample is modified

  - For block size < 8 samples: one sample is modified

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks size >=8 samples: 4 samples are used (same as in VTM 3.0)

  - for block size < 8 samples: Three samples are only used.

- Number of filtering operations:

  - The maximum number of filter operations per edge, is still the same as in VTM 3.0

- Number of line buffers: 4 lines (same as in VTM 3.0)

- Parallelization capabilities: All edges overlapping with 4 x 4 grid can be deblocked in parallel.

- Luma only (Chroma filter is not changed)

# Subjective Evaluation

Fig. 3a below shows a blocking artifact (highlighted in red oval region), observed on VTM-3.0 for the following configuration: Johnny sequence, POC 509, configuration is LDP, QP 34, with ALF OFF. The artifact results because of the multi-type tree partitioning as shown in Fig. 3b. As shown in Fig. 3c, the block size is 8 x 16 samples for the corresponding edge. Fig. 3d shows the 8 x 8 sample grid and the corresponding edge not overlapping with the grid.

Fig. 4a shows that the corresponding edge is deblocked and the block artifact is not observable anymore, Fig. 4b, 4c, show the corresponding partitioning structure and the Fig. 4d shows the 4 x 4 sample grid which ensures that the given block edge is deblocked.

| 3a) 8 x 16 CU not deblocked | 3b) Partitioning information | 3c) depth information | 3d) 8 x 8 sample grid missing the edge |
| --- | --- | --- | --- |

| 4a) 8 x 16 CU also deblocked | 4b) Partitioning information | 4c) depth information | 4d) 4 x 4 sample grid enables deblocking |
| --- | --- | --- | --- |

Figure 5 shows a visual example for Kimono sequence, POC 236, configuration is LDB, QP 34, with ALF OFF. Red circles in Figure 5a and Figure 5c shows the visual quality improvement between the deblocking grid at 8x8 and 4x4. Figure 5c shows the edges filtered only at 4x4 grid.

| 5a) Deblocking on 8x8 sample grid | 5b) Red lines show the edges filtered at 4x4 sample grid and not covered by 8x8 sample grid | 5c) Deblocking on 4x4 sample grid |
| --- | --- | --- |

Figure 6 shows another visual example for KristenAndSara sequence, POC 302, configuration is LDP, QP 37, with ALF OFF. Red circles in Figure 6a and Figure 6b shows the visual quality improvement between the deblocking grid at 8x8 and 4x4.

| 6a) Deblocking on 8x8 sample grid | 6b) Deblocking on 4x4 sample grid |
| --- | --- |

# Experimental Results

The proposed method is implemented on top of VTM-3.0 [2] and the simulation is performed under common test conditions as described in JVET-L1010 [3].

The results are presented for both ALF ON and ALF OFF conditions as suggested to be reported by the CE11 description document [1].

The results are as follows:

# Conclusion

This contribution presents the results for CE test 11.2.1. The proposed method applies deblocking on 4x4 grid to ensure that all blocks whose size is 4xN, Nx4, 8xN, Nx8 are deblocked and not only those that are aligned with the 8x8 sample grid. To ensure parallel processing, the method uses constrained version of the VTM-3.0 weak filter. This filter only modifies one sample on either side of the edge, when one of the blocks sharing the edge has a size of 4 samples.  The proposed method substantially improves the subjective quality when compared to VTM-3.0. Objective results show a Luma BD-Rate gain of upto of 0.4% for LDP configuration (ALF off) without any increase in complexity. It is suggested to adopt this technique to the VVC draft specification and the VTM reference software.

# References

- Description of Core Experiment 11 (CE11): Deblocking, Document JVET-L1031, A. Norkin, A.M. Kotra

- Versatile Video Coding (Draft 3), Document JVET-L01001_v10, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-L1010

# Patent rights declaration(s)

Ericsson, Huawei Technologies Co., Ltd and MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).