| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0303 |
| --- | --- |

| Title: | CE6: Shape adaptive transform selection (Test 3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document provides results for CE6 Test 3.1 on shape adaptive transform selection. The proposed approach selects transforms used in residual coding based on the shape of the transform unit. In the proposed configuration of the tool, the default transform is DST7 for the shorter dimension of a transform block and DCT2 otherwise. The default transforms are used for intra blocks when MTS flag is 0 or when MTS is switched off. DST7 is also used for the square luma Intra blocks if MTS is switched off. The adaptivity is enabled only for Intra blocks and only if block uses spatial prediction (disabled for cross-component predicted chroma blocks) and when the shorter luma dimension is either smaller than equal to 32 (Test 3.1a) or 16 (Test 3.1b).

When MTS is off, the proposed configuration (Test 3.1b) provides -1.61 %, -0.71 % and -0.14 % bitrate impact for AI, RA and LDB configurations, respectively. When intra MTS is on (CTC configuration), the bitrate impacts are -0.06%, -0.03 % and 0.02 %, respectively. When also inter MTS is applied, the bitrate impacts are -0.06%, -0.04 % and -0.03 % for AI, RA and LDB, respectively

# Introduction

As distribution of the intra prediction error signal appears to correlate with the shape of the block, it is proposed to select different transforms for different directions based on the width and height of the transform block. More precisely, it is proposed to use DST7 for the shorter side of the transform block and DCT2 for the longer side of the block. For square blocks it is proposed to use DCT2 for both horizontal and vertical transforms when MTS is enabled for the sequence and DST7 if MTS is disabled for the sequence. The shape adaptivity is used only for intra blocks that are predicted with spatial prediction modes (and disabled for cross-component predicted blocks).

In the case the shape adaptive selection would yield a DST7 with more than 16 points for luma, or more that 8 points for chroma, the adaptivity is not used and DCT2 is selected instead (Test 3.1b configuration). Also limiting the shape adaptivity to 32-point transform in luma and 16-point for chroma was tested as part of the CE in Test 3.1a. But results being nearly identical in both tests, the Test 3.1b with shorted DST7 transforms is proposed for adoption.

If multiple transform selection (MTS) is enabled for the sequence, the shape adaptiveness is applied in the case when MTS flag is zero and VTM would use DCT2 for both horizontal and vertical directions. In the case MTS flag is one, the VTM transform selection process is followed. All the three transforms used (DCT2, DST7 and DCT8) are identical to the ones defined in the current VTM.

Specification text describing the exact process if provided in the next section.

The proposal is not introducing any new transforms, thus it is reported to have no effect on worst case complexity of VTM.

# Original proposed text

8.4.4	Transformation process for scaled transform coefficients

8.4.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-11 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ]. Variables trAdaptHorEnabled and trAdaptVerEnabled are derived as follows:

- if sps_mts_shape_adapt_enabled_flag && CuPredMode[ xTbY ][ yTbY ] == MODE_INTRA && !( cIdx > 0 && IntraPredModeC[ xTbY ][ yTbY ] > 66 ):

trAdaptSizeMin = 4

trAdaptSizeMax = cIdx == 0 ? 16 : 8

trAdaptHorEnabled = nTbW >= trAdaptSizeMin && nTbW <= trAdaptSizeMax ? 1 : 0

trAdaptVerEnabled = nTbH >= trAdaptSizeMin && nTbH <= trAdaptSizeMax ? 1 : 0

- otherwise:

trAdaptHorEnabled = 0

trAdaptVerEnabled = 0

Variables trAdaptHor and trAdaptVer are derived as follows:

- if sps_mts_intra_enabled_flag:

trAdaptHor = trAdaptHorEnabled && nTbW < nTbH ? 1 : 0

trAdaptVer = trAdaptVerEnabled && nTbH < nTbW ? 1 : 0

- otherwise:

trAdaptHor = trAdaptHorEnabled && nTbW <= nTbH ? 1 : 0

trAdaptVer = trAdaptVerEnabled && nTbH <= nTbW ? 1 : 0

…

Table 8-11 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ]

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | trAdaptHor | trAdaptVer |
| 0 | 1 | 1 |
| 1 | 2 | 1 |
| 2 | 1 | 2 |
| 3 | 2 | 2 |

# Proposed text

The proposed text below reflects the request expressed in the meeting to have a separate high-level flag to switch between operating modes using only DCT2 transforms and the mixed transform cases (both implicit and explicit MTS).

#### 7.x.x.x Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_cclm_enabled_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| … |  |
| } |  |

#### 7.x.x.x Sequence parameter set RBSP semantics

…

sps_mts_enabled_flag equal to 1 specifies that sps_explicit_mts_intra_enabled_flag is present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is present in the sequence parameter set RBSP syntax. sps_mts_enabled_flag equal to 0 specifies that sps_explicit_mts_intra_enabled_flag is not present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is not present in the sequence parameter set RBSP syntax.

sps_explicit_mts_intra_enabled_flag equal to 1 specifies that tu_mts_flag may be present in the residual coding syntax for intra coding units. sps_mts_intra_enabled_flag equal to 0 specifies that tu_mts_flag is not present in the residual coding syntax for intra coding units. When not present, the value of sps_mts_intra_enabled_flag is inferred to be equal to 0.

sps_explicit_mts_inter_enabled_flag equal to 1 specifies that tu_mts_flag may be present in the residual coding syntax for inter coding units. sps_mts_inter_enabled_flag equal to 0 specifies that tu_mts_flag is not present in the residual coding syntax for inter coding units. When not present, the value of sps_mts_inter_enabled_flag is inferred to be equal to 0.

…

8.5.4	Transformation process for scaled transform coefficients

8.4.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable implicitTrTypeSel is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA:

implicitTrTypeSel = 1

- Otherwise:

implicitTrTypeSel = 0

The variable trAdaptEnabled is derived as follows:

- If implicitTrTypeSel == 1 && !( cIdx > 0 && IntraPredModeC[ xTbY ][ yTbY ] > 66 ):

trAdaptEnabled = 1

- Otherwise:

trAdaptEnabled = 0

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If implicitTrTypeSel is equal to 1 and cIdx is equal to 0:

trTypeHor = trAdaptEnabled && nTbW <= nTbH && nTbW >= 4 && nTbW <= 16 ? 1 : 0

trTypeVer = trAdaptEnabled && nTbH <= nTbW && nTbH >= 4 && nTbH <= 16 ? 1 : 0

- Otherwise, if implicitTrTypeSel is equal to 1 and cIdx is larger than 0:

trTypeHor = trAdaptEnabled && nTbW < nTbH && nTbW >= 4 && nTbW <= 8 ? 1 : 0

trTypeVer = trAdaptEnabled && nTbH < nTbW && nTbH >= 4 && nTbH <= 8 ? 1 : 0

- Otherwise, trTypeHor and trTypeVer are derived in Table 8-11 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ].

…

# Test results

Full results are provided in the attached Excel sheets. There are three sets of results in both Test 3.1a and Test 3.1b (EMTFast encoder parameter has been set equal to EMT encoder parameter):

- EMT 0: MTS is off for both anchor and test

- EMT 1: MTS is on for Intra, off for Inter for both anchor and test (Common Test Conditions test)

- EMT 3: MTS is on for Intra, on for Inter for both anchor and test

Summary of the results for the proposed Test 3.1b is provided below.

| MTS / EMT: |  | 0 | 1 | 3 |
| --- | --- | --- | --- | --- |
|  | Y | -1.61 % | -0.06 % | -0.06 % |
|  | U | -2.11 % | -0.02 % | -0.02 % |
| AI | V | -2.12 % | -0.08 % | -0.08 % |
|  | Enc | 101 % | 100 % | 100 % |
|  | Dec | 111 % | 99 % | 99 % |
|  | Y | -0.71 % | -0.03 % | -0.04 % |
|  | U | -0.81 % | 0.05 % | 0.01 % |
| RA | V | -0.84 % | 0.02 % | 0.01 % |
|  | Enc | 100 % | 100 % | 101 % |
|  | Dec | 102 % | 101 % | 100 % |
|  | Y | -0.14 % | 0.02 % | -0.03 % |
|  | U | -0.02 % | 0.18 % | 0.06 % |
| LB | V | -0.33 % | -0.01 % | -0.25 % |
|  | Enc | 100 % | 99 % | 100 % |
|  | Dec | 100 % | 99 % | 101 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).