| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0304 |
| --- | --- |

| Title: | CE6-related: 2-mode MTS with shape adaptive transform selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document proposes to reduce MTS (Multiple Transform Selection) modes from 5 to 2 and apply the shape adaptivity aspect of CE6.3.1 on top of the two MTS modes.

The 2-mode MTS proposed in this contribution is identical to the earlier proposals by e.g. LGE (JVET-L0292) and Ericsson (JVET-L0489) and uses only DCT2-DCT2 and DST7-DST7 pairs.

The shape adaptivity aspect is identical to Test 3.1b of CE6 (JVET-M0303), which applies DST7 to the shorter side of the transform block when shape adaptivity is allowed to be performed. In this case it is proposed to be allowed if spatial intra prediction is used and the shorter side of the block is between 4 and 16 samples for luma and 4 and 8 samples for chroma.

The 2-mode MTS alone is reported to have a BD-rate impact of 0.27 %, 0.11 % and 0.10 % for AI, RA and LDB configurations, respectively. The encoder runtimes are reported as 64 %, 95 % and 98 % in those categories.

The 2-mode MTS together with the shape adaptive transform selection aspect is reported to have a BD-rate impact of 0.20 %, 0.10 % and 0.08 % for AI, RA and LDB configurations, respectively. The encoder runtimes are reported as 65 %, 95 % and 98 % in those categories.

When MTS is turned off, the proposed shape adaptive transform selection approach (identical to the CE6 Test 3.1b) provides -1.61 %, -0.71 % and -0.14 % bitrate impact for AI, RA and LDB configurations, respectively.

# Introduction

As distribution of the intra prediction error signal appears to correlate with the shape of the block, it is proposed to select different transforms for different directions based on the width and height of the transform block. More precisely, it is proposed to use DST7 for the shorter side of the transform block and DCT2 for the longer side of the block. For square blocks it is proposed to use DCT2 for both horizontal and vertical transforms when MTS is enabled for the sequence and DST7 if MTS is disabled for the sequence. The shape adaptivity is used only for intra blocks that are predicted with spatial prediction modes (and disabled for inter predicted blocks and cross-component predicted blocks).

In the case the shape adaptive selection would yield a DST7 with more than 16 points for luma, or more that 8 points for chroma, the adaptivity is not used and DCT2 is selected instead.

If multiple transform selection (MTS) is enabled for the sequence, the shape adaptiveness is applied in the case when MTS flag is zero and VTM would use DCT2 for both horizontal and vertical directions. In the case MTS flag is one, the DST7 is used in both directions.

Both transforms used (DCT2, DST7) are identical to the ones defined in the current VTM. As the proposal is not introducing any new transforms, it is reported to have no effect on worst case complexity of VTM.

Specification text describing the exact process if provided in the next section.

# Proposed text

7.3.4.11		Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_mts_flag[ x0 ][ y0 ] && ( cIdx = = 0 ) ) |  |
| mts_idx[ x0 ][ y0 ][ cIdx ] | ae(v) |
| … |  |

8.4.4	Transformation process for scaled transform coefficients

8.4.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-11 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ]. Variables trAdaptHorEnabled and trAdaptVerEnabled are derived as follows:

- if sps_mts_shape_adapt_enabled_flag && CuPredMode[ xTbY ][ yTbY ] == MODE_INTRA && !( cIdx > 0 && IntraPredModeC[ xTbY ][ yTbY ] > 66 ):

trAdaptSizeMin = 4

trAdaptSizeMax = cIdx == 0 ? 16 : 8

trAdaptHorEnabled = nTbW >= trAdaptSizeMin && nTbW <= trAdaptSizeMax ? 1 : 0

trAdaptVerEnabled = nTbH >= trAdaptSizeMin && nTbH <= trAdaptSizeMax ? 1 : 0

- otherwise:

trAdaptHorEnabled = 0

trAdaptVerEnabled = 0

Variables trAdaptHor and trAdaptVer are derived as follows:

- if sps_mts_intra_enabled_flag:

trAdaptHor = trAdaptHorEnabled && nTbW < nTbH ? 1 : 0

trAdaptVer = trAdaptVerEnabled && nTbH < nTbW ? 1 : 0

- otherwise:

trAdaptHor = trAdaptHorEnabled && nTbW <= nTbH ? 1 : 0

trAdaptVer = trAdaptVerEnabled && nTbH <= nTbW ? 1 : 0

…

Table 8-11 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ]

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | trAdaptHor | trAdaptVer |
| 0 | 1 | 1 |
| 1 | 2 | 1 |
| 2 | 1 | 2 |
| 3 | 2 | 2 |

# Test results

Full results are provided in the attached Excel sheets. There are three sets of results included in the summary table below:

- Test 1: MTS is off for both anchor and test; shape adaptive transform selection is used for the test (results of CE6.3.1b)

- Test 2: 2-mode MTS is used with shape adaptive transform selection (CTC anchor)

- Test 3: 2-mode MTS is used without shape adaptive transform selection (CTC anchor)

Results for tests with “Inter MTS” enabled (EMT configuration parameter of VTM-3 set to 3) are provided as attached additional Excel documents.

| Test |  | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| MTS / EMT |  | 0 | 1 | 1 |
| Shape adapt |  | 1 | 1 | 0 |
|  | Y | -1.61 % | 0.20 % | 0.27 % |
|  | U | -2.11 % | -0.19 % | -0.24 % |
| AI | V | -2.12 % | -0.26 % | -0.22 % |
|  | Enc | 101 % | 65 % | 64 % |
|  | Dec | 111 % | 101 % | 100 % |
|  | Y | -0.71 % | 0.10 % | 0.11 % |
|  | U | -0.81 % | -0.06 % | -0.10 % |
| RA | V | -0.84 % | -0.06 % | -0.12 % |
|  | Enc | 100 % | 95 % | 95 % |
|  | Dec | 102 % | 101 % | 101 % |
|  | Y | -0.14 % | 0.08 % | 0.10 % |
|  | U | -0.02 % | 0.18 % | 0.18 % |
| LB | V | -0.33 % | 0.22 % | -0.01 % |
|  | Enc | 100 % | 98 % | 98 % |
|  | Dec | 100 % | 96 % | 95 % |

# Summary

Two things are proposed:

Proposal 1: adopt the 2-mode MTS as it appears to provide a good trade-off between complexity and coding efficiency.

Proposal 2: adopt the shape adaptive transform selection tool as it appears to have a positive impact on coding efficiency when 2-mode MTS is enabled; and especially desirable effects when MTS is disabled.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).