| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18Jan. 2019 | Document: JVET-M0306 |
| --- | --- |

| Title: | CE 9: DMVR Simplifications (Test 9.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents DMVR simplifications based onVTM3.0 with DMVR tool. Firstly, only 4 integer precision surround check for certain condition, followed by MRSAD mean value calculation by sampling process, then use SAD calculation to replace MRSAD. The proposed technologies have 0.00%/ 0.03%, 0.11% lost f respectively compared to VTM3.0 with DMVR as anchor.

Introduction

In current DMVR, 5 point offset for integer checking and 4 point offset for half, and use MRSAD for cost calculation

Proposed improvements

Check reduction for integer (test A)

If cross (UP DOWN LEFT RIGHT) check is done and cost of them are all not less than central one, skip the fifth point check for integer.

Figure 1. Flowchart for integer precision checking.

Sampling for mean value calculation (test B)

In current DMVR, MRSAD needs to calculate mean value of prediction blocks. Proposed use down sampling process to get sub-mean value, which replaces original mean value calculation.

Figure 2.  sub-mean value for MRSAD

Use SAD instead of MRSAD (test C)

Proposed to use SAD calculation to get cost value instead of MRSAD for integer and half precision check.

Experimental Results

The proposed method has been implemented on VTM3.0 with DMVR[1], and allthe tests were conducted under the common testconditions[2]. The experimental results of DMVR simplificationscompared to VTM3.0 with DMVR are shown in Table1-3.

TEST A

Table 1. Experimental results of check reduction for integer(test A)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.04% | 100% | 101% |
| Class A2 | 0.00% | 0.01% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.11% | -0.03% | 100% | 100% |
| Class C | 0.01% | 0.03% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.02% | 100% | 100% |
| Class D | 0.00% | -0.11% | 0.07% | 100% | 100% |

TEST B

Table 2. Experimental results of sub-mean value calculation(test B)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.04% | 0.01% | -0.01% | 100% | 99% |
| Class B | 0.04% | -0.05% | -0.01% | 100% | 99% |
| Class C | 0.01% | -0.05% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.03% | -0.02% | 100% | 99% |
| Class D | -0.02% | 0.00% | 0.01% | 100% | 101% |

TEST C

Table 3. Experimental results of using SAD instead of MRSAD(test C)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.02% | 0.03% | 99% | 99% |
| Class A2 | 0.11% | 0.07% | 0.10% | 99% | 99% |
| Class B | 0.13% | -0.02% | 0.04% | 99% | 99% |
| Class C | 0.09% | 0.06% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.03% | 0.03% | 99% | 99% |
| Class D | 0.02% | -0.01% | 0.03% | 99% | 99% |

Table1-3 show the separate gain for check reduction for integer, sampling for mean value calculation and use SAD instead of MRSAD separately.

Reference

[1]  https://vcgit.hhi.fraunhofer.de/JVET-L-CE9/VVCSoftware_VTM/commits/CE9.2_base_sw.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, China, Oct. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).