| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18Jan. 2019 | Document: JVET-M0306 |
| --- | --- |

| Title: | CE 9: DMVR Simplifications (Test 9.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents DMVR simplifications based on VTM3.0. Firstly, only 4 integer precision surround check for certain condition, followed by MRSAD mean value calculation by sampling process, then use SAD calculation to replace MRSAD. The proposed technologies have 0.71%/ 0.68%, 0.60% gain respectively compared to VTM3.0 anchor.

Introduction

In current DMVR, 5 point offset for integer checking and 4 point offset for half, and use MRSAD for cost calculation

Proposed improvements

Check reduction for integer (test A)

If cross (UP DOWN LEFT RIGHT) check is done and cost of them are all not less than central one, skip the fifth point check for integer.

Figure 1. Flowchart for integer precision checking.

Sampling for mean value calculation (test B)

In current DMVR, MRSAD needs to calculate mean value of prediction blocks. Proposed use down sampling process to get sub-mean value, which replaces original mean value calculation.

Figure 2.  sub-mean value for MRSAD

Use SAD instead of MRSAD (test C)

Proposed to use SAD calculation to get cost value instead of MRSAD for integer and half precision check.

Experimental Results

The proposed method has been implemented on VTM3.0[1], and all the tests were conducted under the common test conditions[2]. The experimental results of DMVR simplifications compared to VTM3.0 are shown in Table1/3/5. The experimental results of DMVR simplifications compared to VTM3.0 with DMVR anchor are shown in Table2/4/6.

TEST A

Table 1. Experimental results of check reduction for integer(test A_vtm3.0 anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.67% | -0.87% | 106% | 113% |
| Class A2 | -0.98% | -1.12% | -1.15% | 108% | 115% |
| Class B | -0.57% | -0.76% | -0.75% | 108% | 116% |
| Class C | -0.77% | -0.87% | -0.91% | 109% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.71% | -0.84% | -0.90% | 108% | 116% |
| Class D | -0.74% | -1.02% | -0.86% | 110% | 121% |

Table 2. Experimental results of check reduction for integer(test A_vtm3.0 with DMVR anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0 with DMVR] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.04% | 100% | 101% |
| Class A2 | 0.00% | 0.01% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.11% | -0.03% | 100% | 100% |
| Class C | 0.01% | 0.03% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.02% | 100% | 100% |
| Class D | 0.00% | -0.11% | 0.07% | 100% | 100% |

TEST B

Table 3. Experimental results of sub-mean value calculation(test B_vtm3.0 anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | -0.62% | -0.84% | 106% | 112% |
| Class A2 | -0.94% | -1.12% | -1.14% | 108% | 115% |
| Class B | -0.53% | -0.71% | -0.74% | 108% | 114% |
| Class C | -0.77% | -0.96% | -0.94% | 109% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.68% | -0.84% | -0.89% | 108% | 115% |
| Class D | -0.76% | -0.91% | -0.91% | 110% | 122% |

Table 4. Experimental results of sub-mean value calculation(test B_vtm3.0 with DMVR anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0 with DMVR] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.04% | 0.01% | -0.01% | 100% | 99% |
| Class B | 0.04% | -0.05% | -0.01% | 100% | 99% |
| Class C | 0.01% | -0.05% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.03% | -0.02% | 100% | 99% |
| Class D | -0.02% | 0.00% | 0.01% | 100% | 101% |

TEST C

Table 5. Experimental results of using SAD instead of MRSAD(test C_vtm3.0 anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.59% | -0.80% | 105% | 111% |
| Class A2 | -0.87% | -1.06% | -1.04% | 107% | 116% |
| Class B | -0.44% | -0.67% | -0.69% | 107% | 115% |
| Class C | -0.69% | -0.84% | -0.92% | 108% | 117% |
| Class E |  |  |  |  |  |
| Overall | -0.60% | -0.78% | -0.84% | 107% | 115% |
| Class D | -0.72% | -0.92% | -0.89% | 109% | 120% |

Table 6. Experimental results of using SAD instead of MRSAD(test C_vtm3.0 with DMVR anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0 with DMVR] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.02% | 0.03% | 99% | 99% |
| Class A2 | 0.11% | 0.07% | 0.10% | 99% | 99% |
| Class B | 0.13% | -0.02% | 0.04% | 99% | 99% |
| Class C | 0.09% | 0.06% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.03% | 0.03% | 99% | 99% |
| Class D | 0.02% | -0.01% | 0.03% | 99% | 99% |

Reference

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/VVCSoftware_VTM.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, China, Oct. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).