| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0310 |
| --- | --- |

| Title: | CE2-related: Using shorter-tap filter for 4x4 sized partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com Chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution uses 6-tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine. it reduces worst case memory bandwidth of 4x4 sized CU by 67% and complexity of affine to 65% number of multiplications and 62% number of additions. Experimental result shows 0.08% luma BD-rate change in RA and 0.14% luma BD-rate change in LD_B. Ignorable change in encoding and decoding time are observed.

# Algorithm description

In this contribution 6-tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine CU. Interpolation filter is shown in Table 4.

The first purpose is to reduce memory bandwidth of 4x4 sized uni-pred CU to be smaller than that of 8x8 bi-pred MC. At the same time, memory bandwidth of 4x4 sized CU is reduced to 67%. The second purpose is to reduce worst computational complexity of affine CU to be smaller than that of 8x8 bi. By using 6-tap for 4x4 sub-block of affine CU, complexity is reduced from 130% to 85% (number of multiplications) and 81% (number of additions) compared with 8x8 bi-pred MC. And worst case complexity of affine CU is reduced to 65% (multiplications) and 62% (number of additions). Details are given in table 2-3.

Table 1: memory bandwidth using difference sizes of filter tap for 4x4 sized uni-pred CU

| filter size | memory bandwidth | ratio to 8 tap |
| --- | --- | --- |
| 8 | 121 | - |
| 6 | 81 | 67% |

| CU size (W×H) | Number of luma samples to be fetched per luma sample (W + N) × (H + N) / (W × H) |
| --- | --- |
| 8×8 Bi-prediction 8 tap | 7.03 |
| 4×4 uni-prediction 8 tap | 7.56 |
| 4×4 Uni-prediction 6 tap | 5.06 |

Table 2: number of multiplications using difference sizes of filter tap for affine CU

| W | H | (1) use 8x8 sub-blocks (8 tap) | (2) use 4x4 sub-blocks (8 tap) | (2)/(1) | (3) use 4x4 sub-blocks (6 tap) | (3)/(1) | (3)/(2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 753,664 | 983,040 | 130% | 638,976 | 85% | 65% |
| 128 | 64 | 376,832 | 491,520 | 130% | 319,488 | 85% | 65% |
| 64 | 128 | 376,832 | 491,520 | 130% | 319,488 | 85% | 65% |
| 128 | 32 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 32 | 128 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 128 | 16 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 16 | 128 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 128 | 8 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 8 | 128 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 64 | 64 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 64 | 32 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 32 | 64 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 64 | 16 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 16 | 64 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 64 | 8 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 8 | 64 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 32 | 32 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 32 | 16 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 16 | 32 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 32 | 8 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 8 | 32 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 16 | 16 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 16 | 8 | 5,888 | 7,680 | 130% | 4,992 | 85% | 65% |
| 8 | 8 | 2,944 | 3,840 | 130% | 2,496 | 85% | 65% |

Table 3: number of additions using difference sizes of filter tap for affine CU

| W | H | (1) use 8x8 sub-blocks (8 tap) | (2) use 4x4 sub-blocks (8 tap) | (2)/(1) | (3) use 4x4 sub-blocks (6 tap) | (3)/(1) | (3)/(2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 659,456 | 860,160 | 130% | 532,480 | 81% | 62% |
| 128 | 64 | 329,728 | 430,080 | 130% | 266,240 | 81% | 62% |
| 64 | 128 | 329,728 | 430,080 | 130% | 266,240 | 81% | 62% |
| 128 | 32 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 32 | 128 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 128 | 16 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 16 | 128 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 128 | 8 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 8 | 128 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 64 | 64 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 64 | 32 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 32 | 64 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 64 | 16 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 16 | 64 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 64 | 8 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 8 | 64 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 32 | 32 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 32 | 16 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 16 | 32 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 32 | 8 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 8 | 32 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 16 | 16 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 16 | 8 | 5,152 | 6,720 | 130% | 4,160 | 81% | 62% |
| 8 | 8 | 2,576 | 3,360 | 130% | 2,080 | 81% | 62% |

Table 4: interpolation filter

| MV | tap filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 64 | 0 | 0 | 0 |
| 1/16 | 1 | -3 | 63 | 4 | -2 | 1 |
| 1/8 | 1 | -5 | 62 | 8 | -3 | 1 |
| 3/16 | 2 | -8 | 60 | 13 | -4 | 1 |
| 1/4 | 3 | -10 | 58 | 17 | -5 | 1 |
| 5/16 | 3 | -11 | 52 | 26 | -8 | 2 |
| 3/8 | 2 | -9 | 47 | 31 | -10 | 3 |
| 7/16 | 3 | -11 | 45 | 34 | -10 | 3 |
| 1/2 | 3 | -11 | 40 | 40 | -11 | 3 |
| 9/16 | 3 | -10 | 34 | 45 | -11 | 3 |
| 5/8 | 3 | -10 | 31 | 47 | -9 | 2 |
| 11/16 | 2 | -8 | 26 | 52 | -11 | 3 |
| 6/8 | 1 | -5 | 17 | 58 | -10 | 3 |
| 13/16 | 1 | -4 | 13 | 60 | -8 | 2 |
| 7/8 | 1 | -3 | 8 | 62 | -5 | 1 |
| 15/16 | 1 | -2 | 4 | 63 | -3 | 1 |

# Experimental Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | -0.04% | 100% | 100% |
| Class A2 | 0.09% | 0.09% | 0.20% | 100% | 98% |
| Class B | 0.05% | 0.08% | -0.10% | 100% | 99% |
| Class C | 0.16% | 0.10% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.06% | 0.02% | 100% | 99% |
| Class D | 0.25% | 0.15% | 0.38% | 100% | 100% |
| Class F | 0.05% | -0.04% | 0.03% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.08% | 0.17% | 100% | 101% |
| Class C | 0.22% | 0.45% | 0.35% | 100% | 101% |
| Class E | 0.09% | 0.17% | 0.50% | 100% | 101% |
| Overall | 0.14% | 0.16% | 0.31% | 100% | 101% |
| Class D | 0.42% | 0.48% | 0.46% | 100% | 101% |
| Class F (optional) | 0.19% | 0.02% | -0.17% | 100% | 101% |

# Conclusions

In this contribution, 6-tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine. it reduces worst case memory bandwidth of 4x4 sized CU by 67% and complexity of affine to 65% regarding number of multiplications and 62% regarding number of additions. Experimental result shows 0.08% luma BD-rate change in RA and 0.14% luma BD-rate change in LD_B. Ignorable change in encoding and decoding time are observed.

It is suggested to adopt proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Additional information:

Use 4-tap filter for 4x4 sized partitions:

4 tap interpolation filter

| MV | tap filter |  |  |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 64 | 0 | 0 |
| 1/16 | -2 | 62 | 4 | 0 |
| 1/8 | -2 | 58 | 10 | -2 |
| 3/16 | -4 | 56 | 14 | -2 |
| 1/4 | -4 | 54 | 16 | -2 |
| 5/16 | -6 | 52 | 20 | -2 |
| 3/8 | -6 | 46 | 28 | -4 |
| 7/16 | -4 | 42 | 30 | -4 |
| 1/2 | -4 | 36 | 36 | -4 |
| 9/16 | -4 | 30 | 42 | -4 |
| 5/8 | -4 | 28 | 46 | -6 |
| 11/16 | -2 | 20 | 52 | -6 |
| 6/8 | -2 | 16 | 54 | -4 |
| 13/16 | -2 | 14 | 56 | -4 |
| 7/8 | -2 | 10 | 58 | -2 |
| 15/16 | 0 | 4 | 62 | -2 |

simulation results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | -0.10% | 100% | 100% |
| Class A2 | 0.30% | 0.06% | 0.08% | 100% | 98% |
| Class B | 0.35% | 0.16% | 0.02% | 100% | 99% |
| Class C | 0.65% | 0.33% | 0.29% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.35% | 0.13% | 0.08% | 100% | 99% |
| Class D | 1.27% | 0.68% | 0.91% | 100% | 98% |
| Class F | 0.12% | 0.08% | 0.15% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.65% | -0.04% | 0.33% | 100% | 99% |
| Class C | 0.90% | 0.71% | 0.56% | 100% | 98% |
| Class E | 0.73% | 0.45% | -0.05% | 100% | 100% |
| Overall | 0.75% | 0.33% | 0.31% | 100% | 99% |
| Class D | 2.48% | 1.24% | 1.46% | 99% | 98% |
| Class F (optional) | 0.51% | 0.42% | 0.24% | 100% | 100% |