| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan 2019 | Document: JVET-M0317 |
| --- | --- |

| Title: | CE10-related: Simplification of triangular prediction unit mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jingya Li Chong Soon Lim | Tel: Email: | +65-65505487 ruling.liao@sg.panasonic.com jingya.li@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic corporation |  |  |

_____________________________

# Abstract

In this contribution, three modifications are proposed for the triangular prediction unit mode. Firstly, the derivation process of candidate list for triangular prediction unit is unified with that for normal merge mode. Secondly, the redundant triangular prediction unit mode flag, which is signaled when either merge motion vector difference mode or multi-hypothesis intra mode is used, is removed. Thirdly, the de-blocking filter is not applied to transform unit boundaries within the coding unit coded in triangular prediction unit mode when there is no residue. With all these modifications, it is reported that as compared to VTM-3.0, xx% B-D rate reduction with xx% encoding time and xx% decoding time is achieved in random access configuration. xx% B-D rate reduction with xx% encoding time and xx% decoding time is achieved in low delay B configuration.

# Introduction

In the Macao meeting, the triangular prediction unit mode is adopted to VVC. For a coding unit (CU) coded in the triangular prediction unit mode, it is split into two triangular prediction units. Each triangular prediction unit in the CU is inter-predicted using its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list. An adaptive weighting process is performed to the diagonal edge after predicting the triangular prediction units. Then, the transform and quantization process are applied to the whole CU. It is noted that the motion vectors of the triangular prediction units are stored in 4×4 grid, either uni-prediction or bi-prediction motion vector is stored depending on the position of the 4×4 grid in the CU.

Although the triangular prediction unit mode provides B-D rate reduction, there are still several aspects can be improved.

Firstly, the candidate list for the triangular prediction unit mode is derived from spatial and temporal neighboring blocks with the full pruning process, which is not aligned with that for normal regular merge mode.

Secondly, generating a bi-prediction motion vector for storage may including motion vector scaling, which increases hardware complexity.

Thirdly, a redundant triangular prediction mode flag is signaled when the current CU is coded in merge motion vector difference mode (MMVD) or multi-hypothesis intra mode (MHIntra).

Fourthly, a CU larger than 64×64 is split into multiple transform units (TU). If the CU is coded using triangular prediction mode, the deblocking filter is applied to TU boundaries even though there is no coefficient. As shown in Figure 1, the red boundaries are deblocked since different motion vectors are stored.

Figure 1: An illustration of deblocking filter applied to a 128×128 CU coded in triangular prediction unit mode

# Proposed technology

## Simplification and unification of candidate list derivation

Firstly, to simplify the motion vector storage process for triangular prediction unit mode, one of the triangular prediction units must be predicted from L0 and another one must be predicted from L1. With this simplification, the motion vector scaling used in generating bi-prediction motion vector can be removed.

Secondly, to align the candidate list derivation of triangular prediction unit mode with that of normal merge mode, the triangular candidate list is directly derived from the first 4 candidates in normal merge candidate list. If the candidate is a bi-prediction one, it is directly copied to the triangular candidate list. If the candidate is a uni-prediction one from L0, the reference index of L1 is set equal to 0 and the motion vector of L1 is derived from that of L0. If the reference pictures of L0 and L1 is from the same time direction, the motion vector of L1 is set equal to that of L0. Otherwise, the motion vector of L1 is set equal to the invert of that of L0. Similarly, the same process is applied if the candidate is from L1.

Two uni-prediction motion vectors of the two triangular prediction units are predicted from either L0 or L1 motion vectors of four bi-prediction candidates in the triangular list. The two uni-prediction motion vectors cannot be predicted from the same bi-prediction candidate. One example is shown in Figure 2. If the first triangular prediction unit is predicted from L0 motion vector of the 1st bi-prediction candidate. Another triangular prediction unit can only predict from L1 motion vector of the 2nd, 3rd or 4th bi-prediction candidate. An index ranging from 0 to 47 is used to indicate which two motion vectors are used to predict the triangular prediction units.

Figure 2: An example of predicting two uni-prediction motion vectors of the triangular prediction units

## Redundant flag removal

The triangular prediction unit flag is not signaled when MMVD or MHIntra mode is used to code a CU.

## Bugfix of deblocking filter

When there is no coefficient, the deblocking filter is disabled for the TU boundaries within a CU coded in triangular prediction unit mode.

# Simulation results

The following experiments were conducted using the VTM-3.0 software with the JVET common test conditions [1] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. Table 1 to Table 4 show the results of three modifications.

Table 1: Results of simplification and unification of candidate list derivation

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.05% | 0.08% | 102% |  |
| Class A2 | -0.03% | -0.04% | 0.03% | 102% |  |
| Class B | -0.02% | 0.00% | -0.04% | 102% |  |
| Class C | 0.11% | 0.11% | 0.18% | 102% |  |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.06% | 102% |  |
| Class D | -0.05% | 0.05% | 0.02% | 102% |  |
| Class F (optional) | 0.05% | 0.05% | 0.03% | 102% |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.08% | 0.24% | 0.08% | 102% |  |
| Class E | 0.08% | 0.63% | 0.08% | 102% |  |
| Overall |  |  |  |  |  |
| Class D | 0.16% | 0.31% | 0.05% | 102% |  |
| Class F (optional) |  |  |  |  |  |

Table 2: Results of redundant flag removal

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.08% | 100% | 98% |
| Class A2 | -0.10% | -0.09% | -0.02% | 100% | 97% |
| Class B | -0.07% | -0.01% | -0.18% | 99% | 96% |
| Class C | -0.07% | -0.06% | -0.09% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.11% | 99% | 97% |
| Class D | -0.10% | -0.20% | -0.07% | 99% | 100% |
| Class F (optional) | -0.03% | -0.01% | -0.03% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.32% | 0.00% | 99% | 99% |
| Class C | -0.19% | 0.03% | -0.14% | 99% | 99% |
| Class E | -0.13% | 0.03% | -0.17% | 99% | 99% |
| Overall | -0.14% | -0.12% | -0.09% | 99% | 99% |
| Class D | -0.17% | 0.13% | 0.13% | 99% | 100% |
| Class F (optional) | 0.00% | -0.14% | -0.54% | 99% | 99% |

Table 3: Results of deblocking filter bug fix

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | 0.06% | 0.05% | 99% | 99% |
| Class E | 0.03% | 0.16% | 0.53% | 99% | 99% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.01% | 0.02% | 99% | 100% |
| Class F (optional) | 0.03% | -0.08% | -0.13% | 99% | 98% |

Table 4: Results of all three modifications

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.01% | -0.15% | 102% |  |
| Class C | 0.03% | 0.14% | 0.18% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.13% | -0.01% |  |  |
| Class F (optional) | 0.05% | 0.01% | 0.04% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.11% | 0.05% | 0.19% |  |  |
| Class E | -0.05% | 0.20% | -0.52% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.05% | 0.41% | -0.30% | 102% |  |
| Class F (optional) |  |  |  |  |  |

# Conclusion

This contribution proposes three modifications to the triangular prediction unit mode. It is reported that xx% and xx% B-D rate reduction is achieved in random access configuration and low delay B configuration, respectively. We recommend to adopt all these three modifications to VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, October 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).