| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0319-v2 |
| --- | --- |

| Title: | CE6: MTS for non-square CUs (test 6.3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung, Dongcheol Kim, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of CE6-3.3. In JVET-L0195, it is proposed to apply MTS to the shorter side of 64xN and Nx64 non-square blocks with N smaller than or equal to 32. It is reportedly shown that CE6-3.3a can achieve 0.00%, -0.04% and 0.02% luma BD-rate on average with 100%, 101% and 100% encoding time under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, compared to VTM-3.0.

As an additional test (CE6-3.3b), the proposed method is tested with the maximum binary tree (BT) and ternary tree (TT) size increased to 64 for I-slices in order to verify the performances of the proposed method in I-slices. Experimental results show that -0.65%, -0.18% and 0.00% luma BD-rate on average with 154%, 103% and 101% encoding time under AI, RA and LDB configurations, respectively, compared to VTM-3.0.

Proposed method

In VTM 3 [1], MTS can efficiently code residual signal using optimal kernel selection between the DST-VII and DCT-VIII for blocks with both width and height smaller than or equal to 32. The proposed method extends the block size condition for applying MTS in order to achieve better energy compaction in large non-square CUs. For 64xN and Nx64 blocks with N smaller than or equal to 32, one of DST-VII / DCT-VIII can be applied to the shorter side while only DCT-II can be applied to the longer side.

To verify the proposed method in Core Experiment (CE 6) [2], two tests (CE6-3.3a and CE6-3.3b) are defined in Table 1 as follows:

Table 1. Description for CE6-3.3

| Test # | Description |
| --- | --- |
| 6-3.3a | For 64xN and Nx64 luma transform blocks (N < 64), MTS can be applied to the shorter side. |
| 6-3.3b | For 64xN and Nx64 luma transform blocks (N<64), MTS can be applied to the shorter side, with the increased max_BT and max_TT sizes for I-slices to 64. |

Experimental results

The proposed method is implemented on top of VTM-3.0 and tested under the common test condition (CTC) [3] and CTC with inter MTS enabled. Table 2 and Table 3 show the experimental results for CE6-3.3a and CE6-3.3b, respectively.

Table 2. Coding performance of CE6-3.3a

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0+Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0+Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.33% | -0.37% | 102% | 100% | -0.10% | -0.27% | -0.17% | 105% | 101% |
| Class A2 | -0.03% | -0.07% | -0.01% | 101% | 100% | -0.04% | -0.02% | 0.03% | 103% | 100% |
| Class B | -0.03% | -0.04% | -0.20% | 101% | 100% | -0.04% | -0.11% | -0.10% | 104% | 100% |
| Class C | -0.02% | 0.03% | -0.05% | 100% | 100% | -0.01% | 0.13% | 0.10% | 103% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.09% | -0.16% | 101% | 100% | -0.04% | -0.06% | -0.03% | 104% | 100% |
| Class D | 0.01% | -0.02% | 0.05% | 100% | 102% | -0.02% | -0.06% | 0.05% | 102% | 102% |
| Class F | -0.01% | -0.04% | 0.00% | 100% | 100% | -0.01% | -0.01% | 0.01% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0+Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.35% | 0.21% | 101% | 100% | -0.07% | 0.13% | -0.27% | 104% | 101% |
| Class C | 0.03% | 0.10% | 0.14% | 100% | 99% | 0.01% | 0.05% | 0.15% | 103% | 99% |
| Class E | 0.01% | 0.40% | -0.13% | 100% | 100% | 0.01% | 0.11% | -0.54% | 104% | 100% |
| Overall | 0.02% | -0.02% | 0.10% | 100% | 100% | -0.02% | 0.10% | -0.20% | 104% | 100% |
| Class D | 0.00% | 0.18% | -0.07% | 100% | 99% | 0.00% | 0.21% | -0.39% | 101% | 100% |
| Class F | 0.06% | 0.10% | 0.06% | 100% | 95% | -0.12% | -0.01% | -0.40% | 103% | 100% |

Table 3. Coding performance of CE6-3.3b

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0+Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.38% | -1.90% | -2.02% | 204% | 99% | -1.38% | -1.90% | -2.02% | 204% | 99% |
| Class A2 | -0.69% | -1.75% | -1.34% | 163% | 99% | -0.69% | -1.75% | -1.34% | 163% | 99% |
| Class B | -0.59% | -1.69% | -1.96% | 153% | 99% | -0.59% | -1.69% | -1.96% | 153% | 99% |
| Class C | -0.15% | -0.57% | -0.70% | 123% | 101% | -0.15% | -0.57% | -0.70% | 123% | 101% |
| Class E | -0.66% | -1.83% | -1.82% | 149% | 100% | -0.66% | -1.83% | -1.82% | 149% | 100% |
| Overall | -0.65% | -1.51% | -1.56% | 154% | 100% | -0.65% | -1.51% | -1.56% | 154% | 100% |
| Class D | -0.08% | -0.36% | -0.34% | 126% | 100% | -0.08% | -0.36% | -0.34% | 126% | 100% |
| Class F | -0.22% | -0.73% | -0.72% | 128% | 100% | -0.22% | -0.73% | -0.72% | 128% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0+Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.75% | -0.76% | 104% | 100% | -0.34% | -0.71% | -0.61% | 107% | 101% |
| Class A2 | -0.29% | -0.89% | -0.61% | 103% | 100% | -0.29% | -0.89% | -0.62% | 105% | 100% |
| Class B | -0.17% | -0.82% | -1.22% | 103% | 100% | -0.16% | -0.87% | -1.03% | 105% | 100% |
| Class C | -0.02% | -0.22% | -0.11% | 101% | 100% | -0.02% | -0.21% | -0.27% | 103% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.18% | -0.66% | -0.71% | 103% | 100% | -0.18% | -0.66% | -0.66% | 105% | 100% |
| Class D | -0.04% | -0.65% | -0.14% | 102% | 101% | -0.01% | -0.36% | -0.04% | 103% | 100% |
| Class F | -0.18% | -0.33% | -0.24% | 104% | 100% | -0.17% | -0.34% | -0.19% | 106% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0+Emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.62% | -0.23% | 101% | 99% | -0.11% | -0.08% | -0.33% | 104% | 101% |
| Class C | 0.06% | 0.15% | 0.04% | 101% | 100% | 0.03% | -0.09% | 0.15% | 103% | 100% |
| Class E | -0.05% | -1.07% | -1.93% | 100% | 102% | 0.00% | -1.00% | -1.80% | 104% | 101% |
| Overall | 0.00% | -0.48% | -0.57% | 101% | 100% | -0.03% | -0.31% | -0.54% | 104% | 101% |
| Class D | 0.02% | 0.12% | -0.23% | 100% | 99% | -0.01% | 0.04% | -0.43% | 101% | 99% |
| Class F | 0.00% | -0.47% | -0.40% | 100% | 95% | -0.18% | -0.09% | -0.60% | 103% | 104% |

As additional test results for CE6-3.3b, Table 4 shows the experimental results for CE6-3.3b when the maximum BT and TT sizes for I-slices in the anchor are increased to 64.

Table 4. Coding performance of CE6-3.3b when the anchor is set to increase the maximum BT / TT sizes for I-slices to 64

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0+MAX_BT_TT_64 |  |  |  |  |  | Over VTM-3.0+MAX_BT_TT_64+Emt3 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.28% | -0.33% | 106% | 101% | -0.21% |  | -0.28% | -0.33% | 106% | 101% |
| Class A2 | -0.09% | -0.09% | -0.11% | 105% | 101% | -0.09% |  | -0.09% | -0.11% | 105% | 101% |
| Class B | -0.07% | -0.07% | -0.18% | 104% | 101% | -0.07% |  | -0.07% | -0.18% | 104% | 101% |
| Class C | 0.00% | 0.01% | -0.01% | 102% | 101% | 0.00% |  | 0.01% | -0.01% | 102% | 101% |
| Class E | -0.08% | -0.10% | -0.06% | 104% | 101% | -0.08% |  | -0.10% | -0.06% | 104% | 101% |
| Overall | -0.08% | -0.10% | -0.13% | 104% | 101% | -0.08% |  | -0.10% | -0.13% | 104% | 101% |
| Class D | -0.01% | -0.10% | -0.02% | 101% | 100% | -0.01% |  | -0.10% | -0.02% | 101% | 100% |
| Class F | 0.03% | 0.01% | -0.07% | 102% | 101% | 0.03% |  | 0.01% | -0.07% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0+MAX_BT_TT_64 |  |  |  |  |  | Over VTM-3.0+MAX_BT_TT_64+Emt3 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.37% | -0.53% | 102% | 101% | -0.16% |  | -0.29% | -0.38% | 105% | 101% |
| Class A2 | -0.06% | 0.03% | -0.12% | 101% | 100% | -0.07% |  | 0.13% | -0.02% | 104% | 100% |
| Class B | -0.05% | -0.07% | -0.20% | 101% | 100% | -0.05% |  | 0.04% | -0.05% | 104% | 100% |
| Class C | 0.01% | 0.07% | 0.11% | 101% | 100% | 0.00% |  | -0.03% | 0.05% | 103% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.08% | -0.17% | 101% | 100% | -0.06% |  | -0.03% | -0.08% | 104% | 100% |
| Class D | 0.00% | -0.06% | 0.05% | 101% | 100% | 0.01% |  | 0.02% | 0.23% | 102% | 100% |
| Class F | -0.07% | 0.07% | 0.08% | 101% | 100% | -0.03% |  | 0.09% | 0.13% | 103% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0+MAX_BT_TT_64 |  |  |  |  |  | Over VTM-3.0+MAX_BT_TT_64+Emt3 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.35% | -0.03% | 101% | 100% | -0.09% |  | 0.17% | 0.05% | 104% | 101% |
| Class C | -0.04% | -0.11% | -0.16% | 101% | 100% | 0.04% |  | 0.06% | -0.01% | 103% | 101% |
| Class E | 0.02% | -0.60% | -0.54% | 100% | 100% | 0.04% |  | -0.43% | -0.11% | 104% | 100% |
| Overall | -0.01% | -0.33% | -0.20% | 101% | 100% | -0.01% |  | -0.02% | -0.01% | 104% | 101% |
| Class D | 0.02% | 0.24% | -0.19% | 101% | 99% | -0.05% |  | 0.19% | -0.16% | 101% | 100% |
| Class F | 0.04% | -0.09% | -0.24% | 101% | 100% | -0.17% |  | 0.12% | -0.50% | 103% | 100% |

Complexity analysis

Combinations of transform type and transforms block sizes

In summary, Table 5 shows the combinations of transform type and transforms block sizes for CE6-3.3.

Table 5. Combinations of transform type and transforms block sizes in CE6-3.3a and CE6-3.3b

| Block size | MTS_CU_flag | Primary transform type |  |
| --- | --- | --- | --- |
|  |  | Horizontal | Vertical |
| width <= 32 && height <= 32 | 0 | DCT-II | DCT-II |
|  | 1 | DST-VII / DCT-VIII | DST-VII / DCT-VIII |
| width <= 32 && height == 64 | 0 | DCT-II | DCT-II |
|  | 1 | DST-VII / DCT-VIII | DCT-II |
| width == 64 && height <= 32 | 0 | DCT-II | DCT-II |
|  | 1 | DCT-II | DST-VII / DCT-VIII |
| Others | 0 | DCT-II | DCT-II |

Other complexity metrics

- Operation counts: Same as VTM 3.

- Memory requirements for storing transform cores (in bytes): Same as VTM 3.

- The minimum bit-precision required by the transforms: Same as VTM 3.

- Analysis of implementation and arithmetic commonalities: Same as VTM 3.

- Worst-case analysis (e.g., multiplications per pixels): Same as VTM 3.

Conclusion

This contribution proposes to apply MTS to the shorter side of 64xN and Nx64 blocks with N smaller than or equal to 32 in order to achieve better energy compaction for large non-square CUs. It is reportedly shown that CE6-3.3a can achieve 0.00%, -0.04% and 0.02% luma BD-rate on average with 100%, 101% and 100% encoding time for AI, RA and LDB configurations, respectively, compared to VTM-3.0. Especially, regarding high resolution sequences in Class A1, 0.11% of luma BD-rate saving is observed. It is recommended to adopt the method tested in CE6-3.3a to VVC.

References

[1] J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVET-L1002, Macao, CN, October 2018

[2] A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-L1026, Macao, CN, October 2018

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software. reference configurations for SDR video,” JVET-L1010, Macao, CN, October 2018

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).