| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0322 |
| --- | --- |

| Title: | CE13-related: In-loop filters disabled across face discontinuities on PHEC with 2-pixel padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun Xuchang Huangfu Lu Yu | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn huangfuxc@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This proposal uses in-loop filters disabled across face discontinuities on PHEC (padding of 2 samples around face row) to solve the subjective issue of HEC. Test results show that proposed method yields -0.35% in BD-rate (using end-to-end Y-WS-PSNR) reduction on average in comparison to that of in-loop filters disabled across face discontinuities on PHEC (padding of 4 samples around face row) while keeping similar subjective results.

Introduction

Only padding method needs more padding pixels to avoid dyeing since deblocking filter at most revises 3 pixels. And to avoid blending with the dyed pixels, if the distance between the corresponding pixel and the discontinuous edge is less than padding width, blending is not allowed in 360Lib.

In-loop filters disabled across face discontinuities can avoid wrongly using in-loop filters in face row boundaries, padding pixels in CE 13, test 1.1 (b) [1] are only used for blending to smooth seam artifacts. There will be no dyeing pixels so that blending should be allowed on all redundant pixels in 360Lib. To improve coding efficiency, padding of 2 samples around face row is used. We found the subjective results of in-loop filters disabled across face discontinuities with 2 or 4 pixels padding are similar.

Disable the following codes in THybridEquiAngularCubeMap.cpp if in-loop filters are disabled across face discontinuities.

if((pos3D.faceIdx == 4 || pos3D.faceIdx == 0 || pos3D.faceIdx == 5 || pos3D.faceIdx == 3) && pos3D.y >= m_sVideoInfo.iFaceHeight-SVIDEO_HEC_PADDING_WIDTH)

continue;

if(pos3D.faceIdx == 1 && pos3D.x < SVIDEO_HEC_PADDING_WIDTH)

continue;

if(pos3D.faceIdx == 2 && pos3D.y < SVIDEO_HEC_PADDING_WIDTH)

continue;

Experimental results

The experiments are conducted according to JVET common test conditions (CTC) [2]. The results of the proposed in-loop filters disabled across face discontinuities on PHEC (padding of 2 samples around face row) are shown below against PHEC and PHEC with in-loop filters disabled (CE test 1.1(b)).

Table II BD-rate using end-to-end WS-PSNR for the proposed method

|  | Anchor: PHEC |  |  | Anchor：CE test 1.1 (b) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.23% | -0.63% | 0.06% | -0.36% | -0.14% | -0.29% |
| ChairLift | -0.38% | -0.24% | -0.44% | -0.31% | -0.12% | -0.34% |
| KiteFlite | -0.35% | -0.22% | -0.27% | -0.32% | -0.24% | -0.14% |
| Harbor | -0.38% | -0.08% | -0.23% | -0.28% | 0.25% | -0.14% |
| Trolley | -0.49% | -0.43% | -0.36% | -0.45% | -0.31% | -0.12% |
| GasLamp | -0.42% | -0.24% | -0.38% | -0.35% | -0.09% | -0.19% |
| Balboa | -0.24% | -0.78% | -0.17% | -0.28% | -0.36% | -0.23% |
| Broadway | -0.22% | -0.64% | -0.12% | -0.29% | -0.58% | 0.01% |
| Landing2 | -0.46% | -2.08% | -1.61% | -0.41% | -0.72% | -0.62% |
| BranCastle2 | -0.46% | -0.36% | -0.52% | -0.48% | -0.52% | -0.63% |
| Class S1 | -0.37% | -0.31% | -0.27% | -0.35% | -0.11% | -0.20% |
| Class S2 | -0.34% | -0.96% | -0.60% | -0.36% | -0.54% | -0.37% |
| Overall | -0.36% | -0.57% | -0.40% | -0.35% | -0.28% | -0.27% |

Conclusion

In this contribution, in-loop filters disabled across face discontinuities on PHEC with 2-pixel padding is proposed. It has 0.35% coding gain compared to in-loop filters disabled across face discontinuities on PHEC with 4-pixel padding while keeping similar subjective results.

Reference

- P. Hanhart, J.-L. Lin and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1033, Oct. 2018.

- P. Hanhart, J. Boyce, K. Choi and J. Lin, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1012, Oct. 2018.

Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).