| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0324 |
| --- | --- |

| Title: | CE3-related: Modified Chroma Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeyoon Park (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 jiyoonpark@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

- Abstract

This contribution proposes a modified coding method for chroma intra prediction modes. While the current version of VVC differentiates the four chroma intra modes of planar, DC, horizontal, vertical prediction modes using 2 bit fixed length codes, this contribution proposes to differentiate them using the unary max coding by modifying the binarization and association in Table 9-7 and Table 9-8. With respect to VTM-3.0 as the anchor, the proposed method shows that an average BDBR of 0.01% (Y), -0.07% (Cb), and -0.08% (Cr) for AI; and 0.06% (Y), -0.20% (Cb), and -0.22%(Cr) for RA, respectively, with no run-time change.

Introduction

When the intra prediction mode of the chroma block is encoded according to the current version of VVC [1], the four modes of planar, DC, horizontal, vertical prediction modes are differentiated by 2 bit FLC, so their bin strings are 11000, 11001, 11010, 11011. Although the planar mode is seen most frequently occurring among thme, the four modes are assigned bin strings of the same length. The preceding 3 bit prefix of ‘110’ differentiates those four cases from DM mode, LM_Chroma (including MDLM-L and MDLM_T).

Figure 1. Binary Strings of Chroma Intra Prediction Modes

When the corresponding luma block has IntraPredModeY among planar (0), vertical (50), horizontal (18), DC (1) and the chroma block is not encoded as the DM mode, the VDIA (66) mode replaces the mode indicated by IntraPredModeY as shown in Table 1 and Table 2.

<Table 1> Specification of IntraPredModeC[ xCb ][ yCb ] when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

<Table 2> Specification of IntraPredModeC[ xCb ][ yCb ] when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

Proposed coding of chroma intra mode

In this contribution, we propose three modifications to the current VVC [1] regarding chroma intra mode coding.

(1). Modified association of the value of intra_chroma_pred_mode = 4

Table 9-7 and Table 9-8 in VVC [1] specify the binarization for intra_chroma_pred_mode respectively when sps_cclm_enabled_flag is equal to 0 and 1. It is noted that the DM mode is associated with intra_chroma_pred_mode = 4 in Table 9-7, but it is assigned to intra_chroma_pred_mode = 7 in Table 9-8. For easier readability and consistency of specification, it is proposed to associate the DM mode with the same value of intra_chroma_pred_mode of 4 irrespective of sps_cclm_enabled_flag value as below.

- intra_chroma_pred_mode = 4   DM mode

- intra_chroma_pred_mode = 7   LM_chroma mode

According to this proposed modification, LM_chroma is assigned the bin string ‘0’ while the DM mode is assigned the bin string ‘10’. It is further noted that the proposed change has no non-trivial performance loss since LM_chorma occurs quite often in many cases, and sometime more frequently than the DM mode.

(2). Modified Binarization for intra_chroma_pred_mode regarding intra_chroma_pred_mode = 0,1,2,3

The second proposed modification is the bin strings associated to intra_chroma_pred_mode = 0, 1, 2, 3 in Table 9-7 and Table 9-8. By noting the dominant occurring probability of the planar mode and that DC mode also likely occurs more frequently than the vertical or horizontal modes, it is proposed to assign variable length codewords of 0/10/110/111 following the prefix 110 to the intra_chroma_pred_mode = 0, 1, 2, 3. The proposed changes are as below.

< OLD>

Table 9-7 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 0 |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |

Table 9-8 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 7 | 0 |
| 4 | 10 |
| 5 | 1110 |
| 6 | 1111 |
| 0 | 11000 |
| 1 | 11001 |
| 2 | 11010 |
| 3 | 11011 |

<NEW>

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 0 |
| 0 | 10 |
| 1 | 110 |
| 2 | 1110 |
| 3 | 1111 |

Table 9-7 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

Table 9-8 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 7 | 0 |
| 4 | 10 |
| 5 | 1110 |
| 6 | 1111 |
| 0 | 1100 |
| 1 | 11010 |
| 2 | 110110 |
| 3 | 110111 |

(3). Modification of Association of intra_chroma_pred_mode for non DM mode cases

As seen in Tables 1 and 2, the VDIA (66) mode replaces the mode indicated by IntraPredModeY when IntraPredModeY is among planar (0), vertical (50), horizontal (18), DC (1) but the chroma block is not encoded as the DM mode. Since the bin strings of the four modes are now not of equal length, the replaced mode of VDIA (which is least probable) should be assigned the longest bin string, that is, 111 with its prefix being either 1 or 110, respectively for sps_cclm_enabled_flag to be equal to 0 or 1. This modification is shown in Tables 3 and 4.

< Table 3 > Proposed bin string when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode | VVC3.0 | Proposed scheme |  |  |
| --- | --- | --- | --- | --- |
|  |  | IntraPredModeY |  |  |
|  |  | Planar | DC | The others |
| 4 | 0 | 0 | 0 | 0 |
| 0 | 100 | 1111 | 10 | 10 |
| 1 | 101 | 110 | 110 | 1110 |
| 2 | 110 | 1110 | 1110 | 1111 |
| 3 | 111 | 10 | 1111 | 110 |

< Table 4 > Proposed bin string when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode | Proposed scheme |  |  |
| --- | --- | --- | --- |
|  | IntraPredModeY |  |  |
|  | Planar | DC | The others |
| 7 | 0 | 0 | 0 |
| 4 | 10 | 10 | 10 |
| 5 | 1110 | 1110 | 1110 |
| 6 | 1111 | 1111 | 1111 |
| 0 | 110111 | 1100 | 1100 |
| 1 | 11010 | 11010 | 110110 |
| 2 | 110110 | 110110 | 110111 |
| 3 | 1100 | 110111 | 11010 |

The value of intra_chroma_pred_mode out of Table 3 and Table 4 is modified as in Table 5 and Table 6 before referring to Table 1 and Table 2.

< Table 5 > Proposed remapping of intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

| Bin String | IntraPredModeY |  |  |
| --- | --- | --- | --- |
|  | Planar | DC | The Others |  |
| 0 | 4 | 4 | 4 |  |
| 10 | 3 | 0 | 0 |  |
| 110 | 1 | 1 | 3 |  |
| 1110 | 2 | 2 | 1 |  |
| 1111 | 0 | 3 | 2 |  |

< Table 6 > Proposed remapping of intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Bin String | IntraPredModeY |  |  |
| --- | --- | --- | --- |
|  | Planar | DC | The Others |  |
| 0 | 7 | 7 | 7 |  |
| 10 | 4 | 4 | 4 |  |
| 1110 | 5 | 5 | 5 |  |
| 1111 | 6 | 6 | 6 |  |
| 1100 | 3 | 0 | 0 |  |
| 11010 | 1 | 1 | 3 |  |
| 110110 | 2 | 2 | 1 |  |
| 110111 | 0 | 3 | 2 |  |

Experiment Result

Experiment is conducted to evaluate the proposed method under CTC condition [2] as below.

  Anchor: VTM-3.0

   Test: VTM-3.0 + SSKU_Chroma_LM_DM_VLC

(test results are in the attached excel file JVET-M0324.xlsm)

Conclusion

In intra prediction mode coding of the chroma block, its compression efficiency of mode coding can be improved by assigning variable length codewords to differentiate the four chroma intra modes of planar, DC, Horizontal, vertical prediction modes.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th Meeting, Macao, CN, 3–12 Oct. 2018.

- Frank Bossen, Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video” JVET-L1010, Oct. 2018.

Patent rights declaration(s)

Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).