| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0335 |
| --- | --- |

| Title: | Non-CE8: modification on SbTMVP process regarding with CPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM-3.0, fetched motion vector is not allowed to refer current picture for the subblock of SbTMVP because the subblock is forced to have default motion vector and it could use fetched motion information which use current picture as reference picture and this default motion could be used for other subblock. This contribution proposes that the subblock motion vector derivation process of SbTMVP is to be applied only if the fetched motion vector for center subblock is coded as inter prediction and the reference picture is not current picture. The proposal shows 0.00% and 0.00% BD-rate changes for RA and LDB configuration respectively with no encoding/decoding complexity change.

Problem Statement

In VTM-3.0, Fetched motion vector does not refer current picture for the subblock of SbTMVP in terms of harmonization between ATMVP and CPR. If fetched subbock motion use the current picture as reference, the subblock is forced to have default motion vector where default motion vector is obtained by center subblock motion information of current PU. However, the derivation process of the default motion vector do not have same restriction. In other words, default motion information could use fetched motion information which has current picture as reference picture and this default motion could be used for other subblock.

Proposed method

In this contribution, the subblock motion vector derivation process of SbTMVP is applied only if the fetched motion vector for center subblock is coded as inter prediction and the reference picture is not current picture.

Experimental result

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 (CPR on) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 (CPR on) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).