| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0338 |
| --- | --- |

| Title: | Non-CE2 : Simplified neighboring spatial coding unit derivation for SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This proposal presents a simplification for SbTMVP derivation process. In order to reduce average and worst case complexity for SbTMVP. This proposal suggests to consider one fixed spatial left neighboring unit to displace spatial neighbor coding unit derivation process for SbTMVP motion vector derivation process.

Problem Statement

At 12th JVET meeting, JVET-L0198[1] proposed simplification of SbTMVP candidate derivation process and was adopted for complexity reduction. One of proposed method is the simplified derivation process of the collocated block for SbTMVP which is one of merge candidates in VTM-2.0. The proposed method of JVET-L0198 reduces complexity because the derivation process is reused by spatial neighboring merge candidates. However SbTMVP is decided as a sub-block merge candidate before other sub-block merge candidate, inherited affine candidates and constructed affine candidates. In VTM-3.0, SbTMVP requires additional derivation process for merging candidates from neighbouring coding units as depicted in Figure 1 because current SbTMVP do not have reusable constructed spatial candidate.

Figure 1. Derivation process for subblock motion vector component and reference indices in VTM-3.0

Proposed method

The proposed method is to use only one fixed position spatial candidates for reference in order to reduce worst case complexity of SbTMVP motion vector derivation process as shown in Figure 2. From this proposed method, derivation process of merging candidates from neighbouring coding units can be removed by fixed left spatial coding unit.

Figure 2. Proposed derivation process for subblock motion vector component and reference indices

Experimental result

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Class B | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class C | 0.02% | -0.01% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.04% | 0.03% | 100% | 99% |
| Class F | -0.01% | -0.02% | -0.01% | 100% | 100% |

Additional experimental result

This proposal reports additional experimental result about SbTMVP derivation based on HMVP.

- Case 1: last motion element of HMVP is used for SbTMVP motion derivation

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 100% | 100% |
| Class A2 | -0.02% | 0.02% | 0.07% | 100% | 100% |
| Class B | 0.00% | -0.02% | -0.03% | 100% | 100% |
| Class C | -0.01% | -0.03% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.02% | 100% | 100% |
| Class D | 0.02% | -0.16% | 0.03% | 100% | 99% |
| Class F | 0.00% | 0.03% | 0.03% | 100% | 100% |

And also reports alternative spatial candidate position which is used for CP1 derivation as shown in figure 3.

Figure 3. Alternative spatial candidate position

- Case 2: Top-left(B2) spatial neighbor is used for SbTMVP motion derivation

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.00% | 100% | 99% |
| Class F | 0.01% | 0.00% | 0.01% | 100% | 100% |

- Case 3: Above-left(B3) spatial neighbor is used for SbTMVP motion derivation

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.00% | 100% | 99% |
| Class F | 0.01% | 0.00% | 0.01% | 100% | 100% |

- Case 4: Left-top(A2) spatial neighbor is used for SbTMVP motion derivation

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.03% | -0.03% | 0.00% | 100% | 99% |
| Class F | 0.01% | 0.00% | 0.01% | 100% | 100% |

Reference

- S.Wang, X. Zheng, X. Zhang, S.Wang, S.Ma, “CE4-related: Simplification of ATMVP candidate derivation,” JVET-L0198, Macao, CN, October 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).