| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0346 |
| --- | --- |

| Title: | CE4-related: Non-square compression unit for temporal motion data storage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Jing Cui1, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to reduce the temporal motion vector storage with a factor of 2 and the coding performance with respect to the granularity of MV storage is reported in two tests:

- Test(a): 8x16 unit based temporal motion compression

- Test(b): 16x8 unit based temporal motion compression

Simulation results for Test (a) shows 0.06% BD-rate loss in VTM Random access configuration and 0.23% BD-rate loss in Low delay B configuration.

Introduction

In current design, the compression unit size of temporal motion vector storage is set to 8x8. Compared to HEVC, the compression unit of which is 16x16, 4 times memory is needed in VTM due to some sub-block tools have been added. Aiming at saving as much memory as possible, this contribution suggests using 8x16 granularity for temporal motion vector compression and change the derivation position from top-left to center, according to the experimental results.

Proposed Method

In order to save the memory needed for temporal motion vector storage, this contribution proposes 8x16 and 16x8 unit temporal motion vector compression method to current design.

R	educe temporal MV data by a factor of 2 (8x16/16x8)

In current design, the granularity of temporal MV compression is set as 8x8, as shown in Fig. 1. Since large CUs might exist when the content of the video sequence is smooth or the motion is relatively simple, 8x8 unit compression for temporal MV isn't effective enough. In this contribution, a non-square temporal MV compression unit, which extends one of edge of original compression unit from 8 to 16, is proposed. The non-square temporal MV compression unit is illustrated at Fig.2.

Figure 1. Current temporal MV compression method

- (b)

Figure 2. Proposed temporal MV compression method: (a) 8x16 unit (b) 16x8 unit

Change the derivation position of temporal MV compression

In current design, the derivation position is set to top-left of the compression unit during the temporal MV compression process. This proposal suggests changing the derivation from top-left to center since the MV of center position tends to be more accurate, which is shown in Fig. 3.

Figure 3. Proposed derivation position for temporal MV compression

Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the three tests on top of the VTM-3.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets.

Coding performance and complexity of Test (a) 8x16 unit based temporal motion compression)

Coding performance and complexity of Test (b) (16x8 unit based temporal motion compression)

From the coding performance of test (a) and test (b), it can be observed that 8x16 compression unit’s performance is slight higher than 16x8 unit.

Conclusion

This contribution proposed a non-square temporal motion vector compression unit that can further reduce the motion data storage to the half as the original size. Simulation results for the test of 8x16 compression unit shows 0.06% BD-rate loss in Random access configuration and 0.23% BD-rate loss in Low delay B configuration on top of VTM3.0. Compared to minor coding lost, the benefit for motion data storage reduction is obvious. Therefore, it is recommended to adopt the proposed temporal motion vector compression method to the next version of VVC working draft.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).