| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0353-v3 |
| --- | --- |

| Title: | Non-CE: Simplification of ALF coefficients merge |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, the simplification of ALF (Adaptive Loop Filter) coefficients merge is proposed. In VTM-3.0, ALF coefficients can be merged after deriving 25 sets of luma filter coefficients in order to reduce signaling bits overhead in encoder side. In the merge process, pairs of classification index to be merged are identified by an exhaustive search. In this contribution, three variants are presented, and they can simplify the process and provide the similar coding efficiency: variant1: AI 0.07%, RA 0.10% LDB 0.18%, variant2: AI 0.08%%, RA 0.11% LDB 0.15%, variant3: AI 0.07%, RA 0.09% LDB 0.17%, respectively.

Introduction

In current ALF[1], each 4x4 block is categorized into one out of 25 classes for luma component. The classification index is derived based on its directionality and activity for which 1-D Laplacian calculation is used. In the encoder, 25 classes can be merged into smaller number of classes to reduce the signaling overhead of ALF coefficients. In the merge process, there are two steps after the derivation of 25 sets of ALF coefficients as shown in Figure 1. In the 1st step, the best pattern is identified per reduced number of classification (24~1). The example is shown in Figure 2. Merged coefficients bits and its distortion are checked for whole pairs of class index. If the pattern is identified for reduced number of classification (24~1), the best pattern can be chosen by RD check, merged coefficients bits and its distortion, among 25 different classifications in the 2nd step. The reduced coefficients and merged information, which indicates the correspondence between original and merged class index, are signalled. The example of the merged information, in which the reduced number of filters is 5, is shown in Figure 1. The 1st step is highly complex, especially for real-time encoder. This contribution addresses the issue and proposes to simplify the process by preparing pre-defined merged pattern per reduced number of classification. There are three variants tested for pre-defined merged pattern and the details are presented in the following section.

Figure 1: Search for best combination of merged class at 25 classes

Figure 2: An example of merge pattern per reduced number of classification

Proposed technology

Two features, directionality and activity, are used for ALF classification. The directionality D consists of direction and its strength as shown in Table 1.

| Value of D | Direction | Strength |
| --- | --- | --- |
| 0 | N/A | Small |
| 1 | h, v | Middle |
| 2 | h, v | Large |
| 3 | d0, d1 | Middle |
| 4 | d0, d1 | Large |

Table 1: Correspondence between D and Direction/Strength

The directionality D and activity value A is calculated as follows, respectively:

Step 1.	If both gh,vmax≤t1∙gh,vmin and gd0,d1max≤t1∙ gd0,d1min are true, D is set to 0.

Step 2.	If gh,vmaxgh,vmin>gd0,d1maxgd0,d1min, continue from Step 3; otherwise continue from Step 4.

Step 3.	If gh,vmax>t2∙gh,vmin, D is set to 2; otherwise D is set to 1.

Step 4.	If gd0,d1max>t2∙gd0,d1min, D is set to 4; otherwise D is set to 3.

A=k=i-2i+3l=j-2j+3Vk,l+Hk,l

A is further quantized to the range of 0 to 4, inclusively, and the quantized value is denoted as A. Finally, the class index C is derived as follows:

C=5D+A

Variant 1

Staring from original classification, the strength, middle and large, is merged alternatively in both h/v and d0/d1. Then the direction, h/v and d0/d1, is merged. Then the strength, small and middle/large, is merged. Finally, the activity is merged toward from small to large one. The detailed derivation and all merged patterns are shown in Figure 3 and Figure 4, respectively.

Figure 3: Derivation of merged pattern in variant 1

Figure 4: All merged patterns in variant 1

In addition, there is no need for signaling of the merge information which indicates the correspondence between original and merged class index since the pattern is provided per reduced number of classification. Therefore, the syntax with regard to the merge can be removed as shown in Table 2.

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_chroma_idc | tu(v) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if ( alf_luma_coeff_delta_flag ) { |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++ ) |  |
| alf_luma_coeff_flag[ sigFiltIdx ] | u(1) |
| } |  |

Table 2: Proposed modifications of syntax table

Variant 2

Staring from original classification, the direction, h/v and d0/d1, is merged alternatively in strength, middle and large. Then the strength, middle and large, is merged. Then the strength, small and middle/large, is merged which is the same as in variant 1. Finally, the activity is merged toward from small to large one which is also the same as in variant 1. The detailed derivation and all merged patterns are shown in Figure 5 and Figure 6, respectively.

Figure 5: Derivation of merged pattern in variant 2

Figure 6: All merged patterns in variant 2

Variant 3

Staring from original classification, the strength, small and middle, is merged alternatively in both h/v and d0/d1. Then the strength, small/middle and large is merged alternatively in both h/v and d0/d1. Finally, the activity is merged toward from small to large one which is the same as in variant 1 and 2. The detailed derivation and all merged patterns are shown in Figure 7 and Figure 8, respectively.

Figure 7: Derivation of merged pattern in variant 3

Figure 8: All merged patterns in variant 3

Experimental Results

The proposed methods are implemented on top of VTM-3.0 and tested for AI, RA and LDB under CTC [2].

Variant 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.01% | 99% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 98% | 100% |
| Class B | 0.04% | 0.03% | 0.03% | 98% | 100% |
| Class C | 0.09% | 0.05% | 0.05% | 99% | 100% |
| Class E | 0.21% | 0.13% | 0.13% | 101% | 100% |
| Overall | 0.07% | 0.04% | 0.05% | 99% | 100% |
| Class D | 0.10% | -0.08% | -0.09% | 101% | 100% |
| Class F (optional) | 0.08% | 0.04% | 0.04% | 100% | 99% |

Table 3: BD-Rate in All Intra

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.05% | 0.02% | 100% | 100% |
| Class A2 | 0.05% | -0.01% | 0.06% | 100% | 100% |
| Class B | 0.13% | 0.02% | -0.07% | 100% | 101% |
| Class C | 0.12% | -0.14% | -0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | -0.04% | -0.03% | 100% | 100% |
| Class D | 0.11% | -0.28% | -0.32% | 100% | 101% |
| Class F (optional) | 0.07% | -0.04% | 0.07% | 100% | 100% |

Table 4: BD-Rate in Random Access

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | -0.28% | -0.06% | 99% | 100% |
| Class C | 0.19% | 0.05% | 0.06% | 99% | 101% |
| Class E | 0.20% | 0.17% | 0.04% | 99% | 98% |
| Overall | 0.18% | -0.06% | 0.01% | 99% | 100% |
| Class D | 0.14% | -0.18% | 0.17% | 99% | 101% |
| Class F (optional) | 0.01% | -0.20% | -0.10% | 100% | 100% |

Table 5: BD-Rate in Low Delay B

Variant2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.02% | 0.02% | #NUM! | 101% |
| Class A2 | 0.01% | 0.01% | 0.01% | #NUM! | 101% |
| Class B | 0.04% | 0.04% | 0.04% | #NUM! | 101% |
| Class C | 0.11% | 0.04% | 0.04% | #NUM! | 101% |
| Class E | 0.20% | 0.12% | 0.12% | #NUM! | 103% |
| Overall | 0.08% | 0.04% | 0.04% | #NUM! | 101% |
| Class D | 0.11% | -0.12% | -0.12% | #NUM! | 100% |
| Class F (optional) | 0.09% | 0.04% | 0.03% | #NUM! | 102% |

Table 6: BD-Rate in All Intra

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.04% | 0.04% | #NUM! | 100% |
| Class A2 | 0.06% | 0.05% | 0.12% | #NUM! | 100% |
| Class B | 0.14% | 0.06% | -0.05% | #NUM! | 101% |
| Class C | 0.14% | -0.09% | -0.12% | #NUM! | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.00% | -0.02% | #NUM! | 100% |
| Class D | 0.12% | -0.22% | -0.30% | #NUM! | 102% |
| Class F (optional) | 0.08% | -0.02% | 0.07% | #NUM! | 100% |

Table 7: BD-Rate in Random Access

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | -0.16% | -0.23% | #NUM! | 101% |
| Class C | 0.20% | -0.07% | -0.06% | #NUM! | 101% |
| Class E | 0.02% | -0.11% | 0.09% | #NUM! | 103% |
| Overall | 0.15% | -0.12% | -0.09% | #NUM! | 102% |
| Class D | 0.16% | -0.53% | -0.33% | #NUM! | 102% |
| Class F (optional) | 0.29% | 0.33% | 0.32% | #NUM! | 102% |

Table 8: BD-Rate in Low Delay B

Variant 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.04% | 0.04% | #NUM! | 101% |
| Class A2 | 0.01% | 0.02% | 0.01% | #NUM! | 101% |
| Class B | 0.05% | 0.03% | 0.03% | #NUM! | 101% |
| Class C | 0.08% | 0.02% | 0.02% | #NUM! | 101% |
| Class E | 0.15% | 0.05% | 0.05% | #NUM! | 102% |
| Overall | 0.07% | 0.03% | 0.03% | #NUM! | 101% |
| Class D | 0.13% | -0.13% | -0.13% | #NUM! | 101% |
| Class F (optional) | 0.07% | 0.02% | 0.01% | #NUM! | 102% |

Table 9: BD-Rate in All Intra

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.07% | 0.00% | #NUM! | 100% |
| Class A2 | 0.04% | 0.08% | 0.03% | #NUM! | 100% |
| Class B | 0.11% | 0.10% | -0.14% | #NUM! | 101% |
| Class C | 0.10% | -0.09% | -0.08% | #NUM! | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.02% | -0.06% | #NUM! | 100% |
| Class D | 0.11% | -0.35% | -0.31% | #NUM! | 102% |
| Class F (optional) | 0.05% | -0.03% | 0.04% | #NUM! | 100% |

Table 10: BD-Rate in Random Access

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | -0.19% | 0.18% | #NUM! | 101% |
| Class C | 0.20% | 0.24% | 0.13% | #NUM! | 102% |
| Class E | 0.10% | 0.24% | -0.33% | #NUM! | 102% |
| Overall | 0.17% | 0.06% | 0.04% | #NUM! | 101% |
| Class D | 0.19% | -0.37% | -0.20% | #NUM! | 101% |
| Class F (optional) | 0.08% | -0.05% | 0.19% | #NUM! | 102% |

Table 11: BD-Rate in Low Delay B

Conclusion

In this contribution, the simplification of ALF coefficients merge is proposed which can provide reduced encoder complexity as well as syntax clean-up by slight coding loss (variant 1: AI 0.07%, RA 0.10% LDB 0.18%, variant 2: AI 0.08%, RA 0.11% LDB 0.15%, variant 3: AI 0.07%, RA 0.09% LDB 0.17%). This contribution proposes to adopt this simplification into next VTM.

References

[1] JVET-L1002, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, J. Chen, Y. Ye, S. Kim (editors)

[2] JVET-L1010, “JVET common test conditions and software reference configurations for SDR video	”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring (editors)

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).