| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0368 |
| --- | --- |

| Title: | AHG8: 360Lib support for chroma sample location in PHEC blending process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng-Hsuan Shih, Ya-Hsuan Lee, Jian-Liang Lin, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {ch.shih, louise.lee, jl.lin, yc.zhang,cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution reports a modification of the chroma sample location in the blending process for PHEC to align with the fix of the chroma sample located proposed in last meeting. A patch of this modification is also attached.

Introduction

In last meeting, a modification about fixing the chroma sample location was adopted into 360Lib software [1]. It was proposed to change the chroma sample location type in the projection format conversion to align with the input video sequence. As shown in Figure 1, chroma sample type 2 was used in 360Lib-7.0 and the older versions. In 360Lib-8.0 [2], support for different chroma sample types is added, and type 0 is set as default to align with the VTM.

Figure 1. Chroma sample location types in YCbCr 4:2:0 format [1]

However, for PHEC, it has a blending process that needs to calculate the sampling location for the updated pixels which has not been modified accordingly. Therefore, in this contribution, we suggest to change the chroma sample location type in the blending process to align with the fix of the chroma sample location. A patch of the modification is attached to this proposal.

Experimental results

The proposed patch is applied to 360Lib-8.0. The experiments are conducted on 360Lib-8.0 and VTM-3.0 [3] following the JVET common test conditions (CTC) and evaluation procedures [4] under the VTM-3.0 configuration setting. Following the CTC, the frame size is set to 3840x2560 with the face size of 1280x1280. Compared to PHEC, Table I demonstrates the average BD-rate performance in terms of end-to-end WS-PSNR and End-to-end S-PSNR-NN for the proposed modification.

Table I BD-rate performance of the proposed modification for PHEC

|  |  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.00% | -0.04% | -0.03% | 0.00% | -0.04% | -0.03% |
|  | ChairLift | 0.00% | -0.03% | -0.02% | 0.00% | -0.02% | -0.02% |
|  | KiteFlite | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% |
|  | Harbor | 0.00% | -0.02% | -0.02% | 0.00% | -0.03% | -0.02% |
|  | Trolley | 0.00% | -0.01% | -0.01% | 0.00% | -0.01% | -0.01% |
|  | GasLamp | 0.00% | -0.01% | -0.02% | 0.00% | -0.01% | -0.02% |
| Class S2 | Balboa | 0.00% | -0.02% | -0.02% | 0.00% | -0.03% | -0.02% |
|  | Broadway | 0.00% | -0.03% | -0.02% | 0.00% | -0.03% | -0.03% |
|  | Landing2 | 0.00% | -0.02% | -0.02% | 0.00% | -0.02% | -0.03% |
|  | BranCastle2 | 0.00% | -0.02% | -0.02% | 0.00% | -0.02% | -0.02% |
| Class S1 |  | 0.00% | -0.02% | -0.02% | 0.00% | -0.02% | -0.02% |
| Class S2 |  | 0.00% | -0.02% | -0.02% | 0.00% | -0.02% | -0.02% |
| Overall |  | 0.00% | -0.02% | -0.02% | 0.00% | -0.02% | -0.02% |

Conclusions

In this contribution, a fix related to the chroma sample location in PHEC blending process is proposed. A patch of the modification is also provided.

References

- P. Hanhart, Y. He, and Y. Ye, “AHG8: Chroma sample location type support for 360Lib”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L10238, Oct. 2018, Macau, CN.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

- P. Hanhart, J. Boyce, K. Choi, and J.-L. Lin, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1012, Oct. 2018, Macau, CN.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).