##### Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqDepth, and the width and the height of the current coding block in luma samples cbWidth and cbHeight.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the variable availableL, specifying the availability of the block located directly to the left of the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the variable availableA specifying the availability of the coding block located directly above the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ) as inputs, and the output is assigned to availableA.

When this process is invoked for the syntax element merge_subblock_flag[ x0 ][ y0 ], the  location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 + cbHeight −  1 )  and the variable availableL, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 )  and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

When this process is invoked for the syntax element merge_subblock_flag[ x0 ][ y0 ], the location ( xNbA, yNbA ) is set equal to ( x0 + cbWidth − 1, y0 − 1) and the variable availableA is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ) as inputs, and the output is assigned to availableA.

( x0 + cbWidth − 1, y0 − 1)

The variables sizeC, sizeTh2 and sizeTh1 are derived as follows:

sizeTh2 = ( MaxBtSizeY = = 128 ) ? 1024 : ( ( MaxBtSizeY = = 64 ) ? 512 : 256 ) 	(9-19)

sizeTh1 = ( MaxBtSizeY = = 128 ) ? 128 : 64	(9-20)

sizeC = cbWidth * cbHeight			(9-21)

The assignment of ctxInc is specified as follows with condL and condA for the syntax elements alf_ctb_flag[ x0 ][ y0 ][ cIdx ], qt_split_cu_flag[ x0 ][ y0], mtt_split_cu_flag[ x0 ][ y0 ], cu_skip_flag[ x0 ][ y0 ], amvr_mode[ x0 ][ y0 ], inter_affine_flag[ x0 ][ y0 ] and merge_subblock_flag[ x0 ][ y0 ] specified in Table 9-11:

ctxInc = ( condL  &&  availableL ) + ( condA  &&  availableA ) + ctxSetIdx * 3	(9-22)

Table 9-11 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxSetIdx |
| --- | --- | --- | --- |
| alf_ctb_flag[ x0 ][ y0 ][ cIdx ] | alf_ctb_flag[ xNbL ][ yNbL ][ cIdx ] | alf_ctb_flag[ xNbA ][ yNbA ][cIdx ] | cIdx |
| qt_split_cu_flag | cqtDepth[ xNbL ][ yNbL ] > cqtDepth | cqtDepth[ xNbA ][ yNbA ] > cqtDepth | ( cqtDepth < 2) ? 0 : 1 |
| mtt_split_cu_flag treeType = = DUAL_TREE_CHROMA | cbHeight[ xNbL ][ yNbL ] < cbHeight | cbHeight[ xNbA ][ yNbA ] < cbWidth | 3 |
| mtt_split_cu_flag treeType ! = DUAL_TREE_CHROMA | cbHeight[ xNbL ][ yNbL ] < cbHeight | cbHeight[ xNbA ][ yNbA ] < cbWidth | ( sizeC > sizeTh2 ) ? 0 : ( ( sizeC > sizeTh1 ) ? 1 : 2 ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | 0 |
| amvr_mode[ x0 ][ y0 ] | amvr_mode[ xNbL ][ yNbL ] | amvr_mode[ xNbA ][ yNbA ] | 0 |
| merge_subblock_flag[ x0 ][ y0 ] | merge_subblock_flag[ xNbL ][ yNbL ] ¦ inter_affine_flag[ xNbL ][ yNbL ] | merge_subblock_flag[ xNbA ][ yNbA ] ¦ inter_affine_flag[ xNbA ][ yNbA ] | 0 |
| inter_affine_flag [ x0 ][ y0 ] | merge_subblock_flag[ xNbL ][ yNbL ] ¦ inter_affine_flag[ xNbL ][ yNbL ] | merge_subblock_flag[ xNbA ][ yNbA ] ¦ inter_affine_flag[ xNbA ][ yNbA ] | 0 |