| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0380 |
| --- | --- |

| Title: | CE2: Affine Merge flag coding (Test 2.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Christophe Gisquet Patrice Onno Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the results of CE2 test 2.2.1.a and test 2.2.1.b related to the subblock Merge mode coding. The first test 2.2.1.a avoids the subblock mode for the Merge Skip as the Combined Intra Inter Merge mode. The second test 2.2.1.b changes the block positions for the subblock flag context derivation in order to have the same positions as the positions of the affine inherited Merge candidates.

An average Luma BDR is reported compared to VTM3.0 of 0.09%, 0.03%, 0.01% for respectively RA, LD and LDP configurations for test 2.2.1.a. And an average Luma BDR is reported compared to VTM3.0 of -0.01%, 0.00%, -0.03% for respectively RA, LD and LDP configurations for test 2.2.1.b.

# Introduction

## Affine and Skip motion compensation

The affine motion compensation prediction is particularly efficient for complex motion like rotation, zoom in, zoom out and perspective motion. The Skip mode is efficient for static content or constant motion. It seems that these modes compensate different kinds of motion and are incompatible.

## Subblock Merge candidate position and affine flag

In the current VTM3.0, 5 affine inherited candidates can be derived in the subblock Merge candidate list.  The five block positions used to derive the affine parameters for the affine inherited candidates are A1, B1, B0, A0, and B2 as depicted in Figure 1 (a).

The subblock flag is CABAC coded with a context value. This context value is the sum of the subblock flag of the 2 neighbouring blocks A2 and B3 as depicted in Figure 1 (b). So this context CtxIdx is given by the following formula:

CtxIdx = IsAffine(A2) + IsAffine(B3)

| a | b |
| --- | --- |

Figure 1 (a) Subblock Merge mode candidates positions. (b) Block position for affine flag context derivation.

# Proposed modifications

## Test CE2.2.1.a: Avoid Subblock for Skip mode

The CE2.2.1.a modification consists in avoiding the subblock motion compensation for the Merge Skip mode as depicted in Table 1.

This reduces the decoding and parsing complexity of the Merge Skip mode because all conditions to decode the subblock flag are avoided and the subblock flag doesn’t need to be parsed. This doesn’t reduce the worst case but it reduces the decoding complexity because the Merge Skip mode is the most selected mode in Inter slices.

Moreover this modification reduces the encoding complexity by avoiding the check of the Merge Skip mode for subblcok Merge.

Table 1 Inter mode enabled for motion compensation type.

| INTER modes | Classical MC | Subblock MC |
| --- | --- | --- |
| MERGE SKIP | X |  |
| MERGE | X | X |
| AMVP | X | X |

## Test CE2.2.1.b: Align subblock context block positions with the affine Inherited Merge candidate positions

The CE2.2.1.b modification aligns the block positions used to derive the subblock flag context with the Inherited Merge candidate positions. The proposed block positions for the affine flag context are A1 and B1 as depicted in Figure 2 and the context index formula is:

CtxIdx = IsAffine(A1) + IsAffine(B1)

With this modification, only the affine flag value of 5 blocks positions needs to be checked before decoding or parsing a subblock flag instead of 7 with the current coding.

Figure 2 Proposed Block positions for Subblock flag Context.

# Experimental results

The proposed method is implemented and tested on top of the VTM3.0.

## Test CE2.2.1.a: Avoid Subblock for Skip mode

Table 2 gives the results of avoiding subblock for Merge Skip mode compared to VTM3.0. The proposed modification has a small impact on the coding efficiency for RA configurations with 0.9% BDR loss on Luma. For LD configuration, almost no average BDR changes are observed.

In addition, we have generated the results for Classes A1 and A2 for both LD configurations as recommended during the presentation of our initial contribution. For both classes, there is the same impact on coding efficiency as for other classes.

Table 2 Results of CE2.2.1.a compared to VTM3.0

## Test CE2.2.1.b: Align Subblock flag context block positions with the inherited affine Merge candidate positions

Table 3 gives the results of the CE2.2.1 test. There is almost no change on coding efficiency for RA and LD configurations. Yet, very small gains are observed.

Table 3 Results of CE2.2.1.b compared to VTM3.0

# Conclusion

In this contribution, the results of both CE2.2.1.a and CE2.2.1.b are reported. Both tests are related to the subblock flag coding. The first one avoids the affine motion compensation for Merge Skip mode as for the combined Intra Inter prediction. This avoids an incompatibility of 2 modes. This modification also reduces the decoding and parsing complexity by avoiding multiple checkings the neighboring blocks for the most selected mode in video coding. Moreover the encoding complexity is reduced by avoiding subblock Merge Skip evaluation.

The second test (CE2.2.1.b) aligns the positions checked for the subblock context with the inherited affine merge block positions to reduce the worst case complexity of the subblock flag checking to decode the subblock Merge flags from 7 to 5 checks without impact on the coding efficiency.

According to these small modifications and the advantages offered for complexity, we propose to adopt both CE2.2.1 modifications for the next VTM software.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).