| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0386 |
| --- | --- |

| Title: | AHG17: On slice_type (tile_group_type) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Yago Sanchez Robert Skupin | Email: | Firstname.Lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a cleanup of the slice_type concept of previous standards. The distinction between I, P and B slice/tile types is getting less sharp with technologies like CPR. It is proposed to distinguish intra- and inter-coding based on flags signalling the actually allowed prediction methods and in addiction, to add the number of active reference picture lists.

# Introduction

Already in HEVC, P slices were not used very frequently. Even in low-delay scenarios, B slices took over with more flexibility in predictions.

In the SCC extension and in VVC, current picture referencing (CPR) was introduced. CPR allows a “motion”-compensated block-based prediction within an intra-coded picture. Hence, creating inter-type prediction intra slices/tiles that have both I- and P-type properties.

# Proposed Syntax

## Variant 1: basic syntax

| slice_type | ue(v) |
| --- | --- |
| current_picture_only_reference_flag | u(1) |
| current_picture_inter_reference_flag | u(1) |
| if( current_picture_only_reference_flag == 0 ¦¦ current_picture_inter_reference_flag == 1) |  |
| num_ref_pic_lists_active_minus1 | u(1) |

current_picture_only_reference_flag equal to 1 allows only intra-prediction. Reference picture lists, if available, shall not contain any picture other than the currently decoded picture. current_picture_only_reference_flag shall be equal to 1 for all tiles of an IRAP picture.

Intent: This flag decides between classical intra- (I) and inter (P,B) prediction types.

current_picture_inter_reference_flag equal to 1 specifies that the currently decoded picture shall be included into the reference picture list(s).

Intent: CPR would be enabled using this flag.

num_ref_pic_lists_active_minus1 indicates the number of active reference picture lists. The variable NumRefPicLists is set equal to ( num_ref_pic_lists_active_minus1 + 1 ). If num_ref_pic_lists_active_minus1 is not included in the bitstream, NumRefPicLists is set equal to 0.

Intent:  num_ref_pic_lists_active_minus1 equal to zero is a classical P-tile, equal to 1 is a B tile.

## Variant 2: conditional syntax

| slice_type | ue(v) |
| --- | --- |
| current_picture_only_reference_flag | u(1) |
| if( current_picture_only_reference_flag ) |  |
| current_picture_inter_reference_flag | u(1) |
| else |  |
| num_ref_pic_lists_active_minus1 | u(1) |

While variant 1 allows all combinations of tools (e.g. CPR in P- and B, CPR in I with one or two reference picture lists, etc.), variant 2 disallows the combination of non-intra coding and CPR. It also intents to disallow using more than one reference picture list in CPR.

This syntax should be considered, if the constraints on CPR are finalized.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).