| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0399 |
| --- | --- |

| Title: | Modification of Triangular PU Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Wei-Jung Chien, Vadim Seregin, Yung-Hsuan Chao, Han Huang, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-4535 hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents 3 changes to the triangular PU mode. The changes include 1) encoding of triangular PU motion information, 2) generation of uni-prediction motion vector candidate set and 3) motion vector storage. Compared to VTM-3.0, the change 1) achieved -0.01% in terms of average luma BD-rate for both randomaccess and lowdelay. The change 1) + 2) achieved -0.06% (RA) and -0.04% (LB) in terms of average luma BD-rate. And the change 1) + 2) + 3) achieved -0.05% (RA) and -0.04% (LB) in terms of average luma BD-rate.

# Introduction

The triangular PU mode [1] was adopted in 12th JVET meeting. In this method, a CU is split into 2 triangular PU by its diagonal or anti-diagonal line. Each PU uses a uni-directional motion vector for motion compensation. At current stage the triangular PU mode is only applied to skip and merge modes and the motion vector used by this mode is represented as an index of a candidate set. Adaptive pixel blending based on motion information on both sides are performed. Transform and quantization are performed on the whole CU.

# Proposed techniques

## Motion information encoding

In VTM-3.0, the motion information of triangular PU mode (splitting direction, motion indexes of the 2 PUs) is combined into a value with a range of [0,39]. The mapping between the combined value and the motion information parameters is performed by a look-up table.

In this contribution, instead of combining the motion information into a single value, 3 values are encoded into bitstreams:

- The splitting direction flag is encoded.

- The motion index of PU0 (Idx0) is encoded in the same way as the merge index of regular merge mode.

- Since the value of Idx0 and Idx1 cannot be equal. For PU1, instead of encoding the value of Idx1, the value of Idx1'=Idx1-(Idx1<Idx0 ?0 :1) is signalled.

## Motion vector candidate set generation.

In VTM-3.0, the candidate set of motion information is generated as follows:

- A procedure similar to regular merge candidate list generation is performed to generate a candidate set (S0) containing up to 7 motion vectors.

- A final candidate set of 5 uni-directional motion vectors (S1) is further generated based on S0. The consideration order of motion vectors is as follows:

  - Uni-directional MV(s).

  - List 0 part of bi-directional MV(s).

  - List 1 part of bi-directional MV(s).

  - The average motion vector of List 0 and List 1 part of bi-directional MV(s).

  - Zero MVs.

Figure 1. neighboring MVs considered for merge/triangle mode.

In this contribution, the regular merge candidate list is used directly as S0. Compared with the procedure used in VTM-3.0, the main differences are summarized as follows:

- For regular merge candidate list, MV5 and MV7 as shown in figure 1 are tested conditionally but in VTM, MV5 and MV7 are always tested.

- The method in VTM performs full pruning for each spatial motion vectors when adding an MV to S0, that is, each spatial MV is compared with all existing MVs in S0 and will be disregarded if the same MV is found. The regular merge candidate list does partial pruning.

- HMVP and pairwise average candidates are considered for regular merge candidate list.

In this contribution, the generation candidate list of 5 uni-directional MVs (S1) is performed in a 1-pass manner. MVs in S0 are searched sequentially, if a MV is a uni-directional MV, then the MV itself is tested for S1, if it is a bi-directional MV, the List0 part and List1 parts of the MV are tested as 2 uni-directional MVs for S1. It is noted that the average MV of List0 and List1 parts of a bi-directional is not considered in this step, thus the complexity of scaling MV from one reference picture to another is avoided.

In the method in VTM, because of the multi-pass manner of the generation of S1, the first item of S1 may come from the last item of S0, the encoder needs to wait until S0 is fully generated before being able to find out the 1st item of S1. The proposed method in this contribution the generation of S0 and S1 can be performed in a pipelined way: after each MV is added to S0, it can be tested immediately for S1. At the same time, early termination of S0 generation can be enabled: when there’re enough items in S1 (S1 has achieved its maximum size or at the decoder side, the candidates for both Idx0 and Idx1 have been generated.) The generation of S0 can be terminated.

## Motion vector storage of triangular PU mode

In VTM-3.0, for triangular PU mode, uni-directional MVs of each PU are stored as motion vectors for CU areas except the areas across the splitting edge of the to triangular PUs. For these edge areas (Figure 2), bi-directional MVs are preferred and the combination of the 2 uni-directional MVs is performed. If one MV is from List0 and the other from List1, the 2 MVs can be combined into a bi-MV directly. When they are from the same reference list, the mapping of one MV to the other reference list is performed. If the mapping succeeds, a bi-MV is generated for MV storage, otherwise uni-MVs are stored for these areas.

In this contribution, the operation of MV mapping is performed only when LDC flag is true. When LDC flag is false, in the case of the 2 uni-directional MVs are from same reference list, uni-directional MV is stored for edge areas without further checking.

Mv0

Mv1

Bi

Figure 2. MV storage of triangular PU mode

# Simulation results

## Performance of the proposed motion information encoding

Simulation results as below, the anchor is VTM3.0 with default setting, the proposed motion information encoding method is tested.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 97% | 99% |
| Class A2 | -0.02% | -0.05% | 0.07% | 101% | 99% |
| Class B | -0.02% | 0.13% | 0.06% | 99% | 99% |
| Class C | -0.01% | 0.03% | 0.12% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.08% | 99% | 99% |
| Class D | -0.02% | -0.12% | -0.09% | 100% | 99% |
| Class F (optional) | -0.01% | -0.03% | -0.01% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.24% | 0.24% | 99% | 95% |
| Class C | -0.01% | 0.09% | -0.03% | 102% | 101% |
| Class E | 0.00% | 0.11% | -0.41% | 100% | 99% |
| Overall | -0.01% | -0.04% | -0.01% | 100% | 98% |
| Class D | -0.03% | 0.05% | 0.70% | 100% | 101% |
| Class F (optional) | -0.01% | 0.28% | 0.29% | 99% | 97% |

## Performance of proposed motion information encoding + candidate list generation

Simulation results as below, the anchor is VTM3.0 with default setting, the tested method includes both motion information encoding and candidate list generation of this contribution.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | -0.01% | 99% | 101% |
| Class A2 | -0.09% | -0.10% | -0.02% | 101% | 100% |
| Class B | -0.07% | 0.01% | -0.10% | 100% | 101% |
| Class C | -0.07% | -0.06% | -0.10% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.04% | -0.06% | 100% | 101% |
| Class D | -0.07% | -0.10% | -0.21% | 100% | 101% |
| Class F (optional) | -0.02% | -0.08% | -0.04% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.22% | -0.14% | 100% | 101% |
| Class C | 0.00% | 0.10% | 0.12% | 102% | 102% |
| Class E | -0.13% | -0.18% | -0.55% | 100% | 99% |
| Overall | -0.04% | -0.10% | -0.16% | 101% | 101% |
| Class D | 0.06% | 0.52% | 0.19% | 101% | 103% |
| Class F (optional) | -0.03% | 0.22% | 0.34% | 100% | 102% |

## Performance of all proposed 3 changes enabled

Simulation results as below, the anchor is VTM3.0 with default setting, the tested method includes motion information encoding, candidate list generation and MV storage of this contribution.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.00% | 98% | 100% |
| Class A2 | -0.09% | -0.06% | -0.06% | 97% | 98% |
| Class B | -0.06% | 0.01% | -0.10% | 97% | 97% |
| Class C | -0.04% | -0.07% | -0.01% | 96% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.04% | -0.05% | 97% | 97% |
| Class D | -0.07% | -0.04% | -0.14% | 97% | 93% |
| Class F (optional) | -0.01% | -0.03% | 0.00% | 98% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.22% | -0.14% | 98% | 94% |
| Class C | 0.00% | 0.10% | 0.12% | 99% | 98% |
| Class E | -0.13% | -0.18% | -0.55% | 98% | 96% |
| Overall | -0.04% | -0.10% | -0.16% | 98% | 96% |
| Class D | 0.06% | 0.52% | 0.19% | 99% | 99% |
| Class F (optional) | -0.03% | 0.22% | 0.34% | 97% | 95% |

# Conclusions

This contribution presents 3 changes to the triangular PU mode. The changes include 1) encoding of triangular PU motion information, 2) generation of uni-prediction motion vector candidate set and 3) motion vector storage. Compared to VTM-3.0, the change 1) achieved -0.01% in terms of average luma BD-rate for both randomaccess and lowdelay. The change 1) + 2) achieved -0.06% (RA) and -0.04% (LB) in terms of average luma BD-rate. And the change 1) + 2) + 3) achieved -0.05% (RA) and -0.04% (LB) in terms of average luma BD-rate.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R.-L. Liao and C. S. Lim, “CE10: Triangular prediction unit mode”, JVET-K0144, Ljubljana, July 2018.